| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0308 |
| --- | --- |

| Title: | TE B4: Crosscheck for 4.4.2 on inter-layer adaptive filter (JCTVC-L0075) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Xiaoxia Cai 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: Tel: Email: Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com 86-10-82171657 wenhao.zhang@intel.com 86-10-82171674 xiaoxia.cai@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of JCTVC-L0075, TE4.4.2 inter-layer adaptive filter, proposed by MediaTek. The software provided by the proponent was reviewed and found to match the proposal. The cross-check result confirms the result reported by proponent. It’s should be noted the coding time in the table is not accurate since non-homogenous grid is used for simulation.

# Introduction

In the JCT-L0075, a 7x7cross + 3x3 square Wiener filter based inter layer adaptive filter is applied to the up-sampled base layer pictures before inter layer intra prediction. The coefficients of the filter are trained for each color component one slice. There are flags indicating that whether the inter layer adaptive filter will applied to the component or not. 9 filter coefficients are transited to the decoder after the flag when it is on. The tool was implemented on and evaluated based on Smuc0.1.1.

# Experimental results

The software package provided by the proponent was successfully compiled and tested under the common test conditions of TE4 [1]. The anchor is Smuc0.1.1. There was no mismatch between encoder and decoder. Table1 is the crosscheck results for AI, RA and LDP configuration. More detailed results can be found in the Excel attachment.

Table 1 Performance data of TE4.2.2 inter-layer adaptive filter

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | -0.1% | -0.2% | -0.1% | -0.3% | -0.5% | -0.3% |  |  |  |
| Overall (EL+BL) | -0.1% | -0.1% | -0.1% | -0.3% | -0.5% | -0.3% |  |  |  |
| Overall (EL) | -0.2% | -0.3% | -0.2% | -1.3% | -2.0% | -1.1% |  |  |  |
| Enc Time[%] | 100.8% |  |  | 100.9% |  |  |  |  |  |
| Dec Time[%] | 96.8% |  |  | 97.4% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% |  |  |  | -4.3% | -2.6% | -2.2% |
| Class B | -0.2% | -0.1% | 0.0% | -0.3% | -0.5% | -0.1% | -1.3% | -0.8% | -0.2% |
| Overall (EL+BL) | -0.1% | -0.1% | 0.0% | -0.3% | -0.5% | -0.1% | -2.1% | -1.3% | -0.8% |
| Overall (EL) | -0.2% | -0.2% | 0.0% | -1.0% | -1.6% | -0.3% | -4.5% | -3.1% | -2.0% |
| Enc Time[%] | 100.3% |  |  | 100.5% |  |  | 101.6% |  |  |
| Dec Time[%] | 96.0% |  |  | 96.1% |  |  | 99.0% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | -6.0% | -3.1% | -2.9% |
| Class B | -0.4% | -0.6% | -0.3% | -0.8% | -1.2% | -0.6% | -2.2% | -1.4% | -0.7% |
| Overall (EL+BL) | -0.3% | -0.4% | -0.2% | -0.8% | -1.2% | -0.6% | -3.3% | -1.9% | -1.3% |
| Overall (EL) | -0.6% | -0.7% | -0.5% | -2.2% | -3.4% | -1.6% | -6.1% | -4.1% | -3.1% |
| Enc Time[%] | 100.5% |  |  | 101.0% |  |  | 101.8% |  |  |
| Dec Time[%] | 95.3% |  |  | 95.7% |  |  | 99.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# References

[1] J.Chen, “Description of Tool Experiment B4: Inter-layer filtering in SHVC,” JCTVC-K01104, Shanghai, Oct. 2012.