| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0309 |
| --- | --- |

| Title: | TE-B4: Results of test 4.2.1 on adaptive up-sampling filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Xiang Li Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8028 cjianle@qti.qualcomm.com lxiang@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the test results of Tool Experiment B4 4.2.1 on adaptive up-sampling filter for SHVC. The coefficients of 2D separable 8-tap up-sampling filter for luma component are signaled at picture level and the filtering process is also applied to inter pixel position in this test. The proposed method provides BD-rate reductions (based on BL+EL rate) of -0.1% (AI 2x), -0.6% (AI 1.5x), -0.3% (RA 2x), -0.6% (RA 1.5x), -2.0% (RA SNR), -0.6% (LP 2x), -1.0% (LP 1.5x), -3.0% (LP SNR), respectively.

# Technical description

In the current SHVC base software, the up-sampling process applies a 2D separable 8-tap filter for luma component and a 2D separable 4-tap filter for chroma component for spatial scalability case. This test studied performance of adaptive luma up-sampling filter for SHVC [1]. The adaptive filter coefficients are derived at the encoder side and signaled to the decoder for each picture. The adaptive filter has the same features with the current fixed up-sampling filter, which has symmetric and 7 bits accuracy filter coefficients, hence it does not change the filtering process or introduce additional filtering process. The filter coefficients are coded with Exp-Golomb code in the adaptive picture set syntax.

# Test results

The table below summarizes the performance of the tested method in SHVC common test conditions. Full results are provided in the attached Excel sheets.

Tab.1 BD-Rate Saving of TE-B4 4.2.1 against SHVC v0.1.1

# References

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park, “Description of Tool Experiment B4: Inter-layer filtering for SHVC”, doc. JCTVC-K1104, 11th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Shanghai, China, 10-19 Oct., 2012

# Appendix: filter coefficients derivation

The vertical filter and the horizontal filter are derived sequentially. First, the vertical filter is derived by solving

where  is the vertical auto-correlation matrix of the original EL picture and  is the vertical cross-correlation between the original EL picture and the reconstructed BL picture. In spatial scalability case, the original EL picture is horizontally down-sampled to match the width of the BL picture and used as the input to this derivation process. The derived filter  is used to vertically up-sample/filter the BL picture and the resulting picture is used to derive the horizontal filter.

The horizontal filter is derived by solving

where  is the horizontal auto-correlation matrix of the original EL picture and  is the horizontal cross-correlation between the original EL picture and the vertically up-sampled/filtered BL picture.

The BL picture is up-sampled/filtered using both the derived filter set and the fixed filter set which is also used for intra residual prediction mode. The sum of absolute transform differences (SATD) between the filtered picture and the original EL picture are calculated. When SATD for the derived filter are smaller than those for the fixed filter, the derived filter is used in intra-BL prediction for the current picture. Otherwise, the fixed filter set is used in intra-BL prediction. A flag is signaled to the decoder at picture level to indicate the choice of the filter.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).