| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0310 |
| --- | --- |

| Title: | TE C5: Crosscheck for 5.2.7 on EL AMVP mode (JCTVC-L0164) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Xiaoxia Cai 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: Tel: Email: Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com 86-10-82171657 wenhao.zhang@intel.com 86-10-82171674 xiaoxia.cai@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of JCTVC-L0164, TE5.2.7 enhancement layer AMVP motion vector candidate list extension, proposed by Huawei. The software provided by the proponent was reviewed and found to match the proposal. The cross-check result confirms the result reported by proponent. It’s should be noted the coding time in the table is not accurate since non-homogenous grid is used for simulation.

# Introduction

JCT-L0164 proposed to add a scaled base layer motion vector into the enhancement layer (EL) AMVP motion vector candidate list. The collocated base layer SPU motion information is inserted in the first place of the list. The buffer for the base layer motion information will not be compressed till the encoding of enhancement layer is done.

A recursive motion information padding method, which gets motion information from neighbor SPU, was also proposed in JCT-L0164 in case that the collocated SPU’s motion information is not valid. The zero mv will be used as the derived base layer mv candidate in case that the recursive padding process is also failed to find the mv.

# Experimental results

The platform used to test the encoder and decoder is Linux + GCC 4.2.2. The software package provided by the proponent was successfully compiled and tested under the common test conditions of TE5 [1]. The anchor is Smuc0.1.1. There was no mismatch between encoder and decoder. Table 1 is the crosscheck results for RA and LD-P configuration, which is identical to the one provided by proposal. More detailed results can be found in the Excel attachment.

Table 1 Performance data of EL AMVP mode

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.7% | -2.8% | -2.8% |  |  |  | -1.6% | -3.1% | -3.3% |
| Class B | -1.4% | -2.0% | -2.2% | -1.5% | -1.8% | -2.1% | -1.5% | -2.2% | -2.5% |
| Overall (EL+BL) | -1.5% | -2.3% | -2.4% | -1.5% | -1.8% | -2.1% | -1.6% | -2.5% | -2.7% |
| Overall (EL) | -2.7% | -4.0% | -4.2% | -3.8% | -4.3% | -5.0% | -3.0% | -4.4% | -4.9% |
| Enc Time[%] | 97.7% |  |  | 97.1% |  |  | 99.2% |  |  |
| Dec Time[%] | 98.3% |  |  | 97.4% |  |  | 99.7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.3% | -2.0% | -2.0% |  |  |  | -1.3% | -2.1% | -2.1% |
| Class B | -0.8% | -1.2% | -1.3% | -0.8% | -1.0% | -1.0% | -1.0% | -1.3% | -1.4% |
| Overall (EL+BL) | -1.0% | -1.4% | -1.5% | -0.8% | -1.0% | -1.0% | -1.1% | -1.5% | -1.6% |
| Overall (EL) | -1.6% | -2.4% | -2.6% | -1.7% | -2.1% | -2.1% | -1.8% | -2.4% | -2.6% |
| Enc Time[%] | 98.8% |  |  | 98.3% |  |  | 100.1% |  |  |
| Dec Time[%] | 99.8% |  |  | 100.0% |  |  | 102.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# References

[1] V. Seregin, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer,” JCTVC-K01105, Shanghai, Oct. 2012.