| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0314 |
| --- | --- |

| Title: | Non-TE B4: Crosscheck for combination of TE B4 4.2.1 and TE3 4.6.2.1 (JCTVC-L0211) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 Xiaoxia Cai 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: Tel: Email: Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com 86-10-82171657 wenhao.zhang@intel.com 86-10-82171674 xiaoxia.cai@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of Crosscheck for combination of TE B4 4.2.1 and TE3 4.6.2.1, proposed by Qualcomm in JCTVC-L0211. The software provided by the proponent was checked and tested. The cross-check result confirms the result reported by proponent.

# Introduction

JCT-L0211 proposed to combine the adaptive up-sampling filter (AUF) and generalized residual prediction (GRP) tools together. The GRP tool with weighting modes 0, 0.5 and1 was chosen, the fast GRP mode was enabled as well.

# Experimental results

The software package provided by the proponent was successfully compiled and tested under the common test conditions for TE4 [1], with anchor Smuc0.1.1. There was no mismatch between encoder and decoder. The encoder time increases about 20% while the decoder time is almost the same with anchor. The performance data is shown in Table 1 and detailed results can be found in the attached excel sheets.

Table 1 Performance data of Combination of TE B4 4.2.1 and TE3 4.6.2.1

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.9% | -5.0% | -5.2% |  |  |  | -6.0% | -5.7% | -6.1% |
| Class B | -2.3% | -3.6% | -3.8% | -3.3% | -5.2% | -5.6% | -3.5% | -4.9% | -5.4% |
| Overall (EL+BL) | -2.1% | -4.0% | -4.2% | -3.3% | -5.2% | -5.6% | -4.2% | -5.2% | -5.6% |
| Overall (EL) | -3.9% | -7.5% | -7.9% | -8.3% | -12.3% | -13.2% | -8.3% | -9.9% | -10.6% |
| Enc Time[%] | 121.0% |  |  | 112.7% |  |  | 116.5% |  |  |
| Dec Time[%] | 102.9% |  |  | 101.4% |  |  | 125.7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.6% | -4.6% | -4.2% |  |  |  | -8.8% | -5.7% | -5.4% |
| Class B | -3.6% | -3.0% | -2.7% | -4.9% | -4.4% | -3.7% | -5.6% | -4.3% | -3.9% |
| Overall (EL+BL) | -3.3% | -3.4% | -3.1% | -4.9% | -4.4% | -3.7% | -6.5% | -4.7% | -4.3% |
| Overall (EL) | -5.8% | -6.3% | -5.9% | -11.7% | -9.8% | -8.7% | -11.6% | -8.2% | -7.7% |
| Enc Time[%] | 126.0% |  |  | 115.6% |  |  | 121.0% |  |  |
| Dec Time[%] | 100.6% |  |  | 97.8% |  |  | 126.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# References

[1] J.Chen, et.al, “Description of Tool Experiment B4: Inter-layer filtering in SHVC”, JCTVC-K1104, Shanghai, China, Oct. 2012.