| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0317 |
| --- | --- |

| Title: | AHG8: YUV444 and RGB screen content test sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuhui Wang, Tao Lin, Kailun Zhou | Tel: Email: | 86-21-5103-0961 lintao@tongji.edu.cn |
| Source: | Tongji University, Shanghai, China |  |  |

_____________________________

# Abstract

This contribution proposes six RGB and six YUV444 screen content video sequences for AHG8 experiments of HEVC extension on screen content coding.

# Introduction

This contribution submits six RGB screen content video sequences that are made available to JCT-VC and parent bodies by Tongji University. Tongji University proposes to adopt these sequences for the study and improvement of the effectiveness of HEVC standard for screen content coding. YUV444 versions of the six sequences are also provided.

# Content description

Six video sequences in both RGB and YUV444 formats are provided. Table 1 shows the specifications of the test sequences.

Table 1 Specification of the test sequences

| Sequence Name | sc_cad_waveform sc_pcb_layout sc_ppt_doc_xls | sc_web_browsing sc_video_conferencing_doc_sharing sc_cg_twist_tunnel |
| --- | --- | --- |
| Resolution | 1920 × 1080p | 1280 × 720p |
| Frame rate | 20 fps, progressive | 30 fps, progressive |
| Bit depth | 8 | 8 |
| Color format | planar RGB and YUV 4:4:4 | planar RGB and YUV 4:4:4 |
| Duration | 10 seconds (200 frames) | 10 seconds (300 frames) |

The test sequences were captured from a computer graphics card HDMI output by a Tongji-customized video capture card in raw RGB 24-bit format.

The RGB sequences were converted to YCbCr 4:4:4 8-bit format in accordance with ITU-R Rec. BT.709 using the following formula with clipping to 0 or 255:

Y =  0.212600*R + 0.715200*G + 0.072200*B

Cb= -0.099907*R - 0.336093*G + 0.436000*B + 128

Cr=  0.615000*R - 0.558608*G - 0.056392*B + 128

All processing was performed in single precision floating point.

A brief description of the test sequences is listed in Table 2.

Table 2 Brief description of the test sequences

| No | File name | Brief description | Features |
| --- | --- | --- | --- |
| 1 | sc_cad_waveform_1920x1080_20_200_8bit_444.yuv sc_cad_waveform_1920x1080_20_8bit_200_rgb.rgb | CAD waveform | - Typical digital circuit design waveform with source code window and schematic window - One pixel thin lines with many colors - Scrolling - Zooming |
| 2 | sc_pcb_layout_1920x1080_20_200_8bit_444.yuv sc_pcb_layout_1920x1080_20_8bit_200_rgb.rgb | PCB layout windows | - Typical PCB layout window - Display of multiple layers with different colors - Component footprint display - One pixel diagonal, horizontal, vertical lines and curves with different colors - Enabling/disabling display of some layers - Moving around - Zooming in and out |
| 3 | sc_ppt_doc_xls_1920x1080_20_200_8bit_444.yuv sc_ppt_doc_xls_1920x1080_20_8bit_200_rgb.rgb | Ppt, doc, and xls windows | - Text, graph, chart - Copy and paste between ppt, doc, and xls windows - One pixel thin lines and curves - Zooming chart and graph |
| 4 | sc_web_browsing_1280x720_30_300_8bit_444.yuv sc_web_browsing_1280x720_30_8bit_300_rgb.rgb | Web browsing | - Browsing different webpages |
| 5 | sc_video_conferencing_doc_sharing_1280x720_30_300_8bit_444.yuv sc_video_conferencing_doc_sharing_1280x720_30_8bit_300_rgb.rgb | Video conferencing with doc sharing | - A video conferencing window - A document sharing window - Switching between excel sheets with texts or plots - Excel sheet scrolling and panning |
| 6 | sc_cg_twist_tunnel_1280x720_30_8bit_444.yuv sc_cg_twist_tunnel_1280x720_30_8bit_rgb.rgb | 3D computer graphics | - Fast spinning twist - Fast moving tunnel - Texture-mapped and light-shaded (using Gouraud shading) rendering - Underneath wireframe rendering |

# Conclusion

We propose to adopt these sequences for the study and improvement of the effectiveness of HEVC standard for screen content coding.

# Conditions of Use – Copyright and Release Notes

These sequences have been generated by Tongji University, and all intellectual property rights remain with Tongji University. These materials shall be used for research, development, experiment, publication, and demo of video coding technologies purposes only under the following conditions of use: 1) do not alter any picture of the test sequences except to resize it; 2) do not use only portions of any picture; 3) any publication or demo using the test sequences must mention or display the acknowledgement of “by courtesy of Tongji University“.

This copyright and permission notice shall be duplicated whenever the data are copied or distributed.

These materials cannot be distributed with charge. Tongji University makes no warranties with respect to the material and expressly disclaims any warranties regarding its fitness for any purpose. Unless the above conditions are agreed to by the recipient, no permission is granted for any use, copying, modification, and distribution of the data. By using these sequences the user agrees to the conditions of this copyright and disclaimer.

# Underlying Microsoft Product Right

Microsoft Windows and Office were used in the screen shots with permission from Microsoft. See Use of Microsoft Copyrighted Content (http://www.microsoft.com/about/legal/en/us/intellectualproperty/permissions/default.aspx):

# Availability of the Sequences

Tongji University agrees to donate the test sequences to the video coding community and to upload the test sequences to JCTVC website under the Condition of Use set forth on 4 of this document. Contact email: vlsi@tongji.edu.cn.