| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0320 |
| --- | --- |

| Title: | Cross-verification results of CE1: Test 2 - Non 32x32 point transforms in chroma for Range Extensions (L0146) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng, Yongbing Lin | Tel: Email: | zhengxiaozhen@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution document reports the cross verification results of proposal L0146. The source code was implemented based on HM Range Extension branch. We compiled, inspected, and ran the code of these cases with CTC configurations of Range Extension. The cross-check has been completed successfully and the results match those provided by proponents of JCTVC-L0146.

# Test description and crosscheck results

This document reports the cross-check results of L0146. The encoder and decoder reconstructed YUV files generated from the above settings are matched. The experimental results of the rate-distortion are illustrated in Error: Reference source not found. These results match the BD-rate results provided by proponents of JCTVC-L0146.

Table 1: Cross-check results of L0146

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.1% | 1.8% | 2.2% | 0.9% | 1.6% | 1.6% |
| YCbCr 4:4:4 | 0.9% | 3.7% | 4.6% | 0.8% | 2.4% | 2.9% |
| YCbCr 4:2:2 | 0.1% | 0.9% | 0.9% | 0.1% | 0.5% | 0.5% |
| Overall | 0.7% | 2.1% | 2.6% | 0.6% | 1.5% | 1.7% |
|  | 0.7% | 2.1% | 2.6% | 0.6% | 1.6% | 1.6% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 108% |  |  | 108% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.8% | 1.8% | 1.7% | 0.3% | 1.4% | 0.7% |
| YCbCr 4:4:4 | 0.4% | 4.8% | 3.8% | 0.4% | 2.6% | 2.4% |
| YCbCr 4:2:2 | 0.1% | 1.4% | 1.4% | 0.1% | 0.7% | 0.8% |
| Overall | 0.5% | 2.6% | 2.3% | 0.3% | 1.5% | 1.3% |
|  | 0.4% | 2.6% | 2.0% | 0.3% | 2.5% | 0.2% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 122% |  |  | 120% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.5% | 1.9% | 1.3% | 0.4% | 1.7% | 0.6% |
| YCbCr 4:4:4 | 0.3% | 4.8% | 3.9% | 0.4% | 2.9% | 2.3% |
| YCbCr 4:2:2 | 0.1% | 2.2% | 2.4% | 0.1% | 1.0% | 1.3% |
| Overall | 0.3% | 2.9% | 2.5% | 0.3% | 1.8% | 1.4% |
|  | 0.3% | 1.9% | 2.5% | 0.3% | 2.9% | 1.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 117% |  |  | 117% |  |  |