| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0321 |
| --- | --- |

| Title: | AHG7: Cross-verification of JCTVC-L0175, A Consumer 4:4:4 Coding Method Based On A 4:2:0 Codec |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A. | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of JCTVC-L0175, “AHG7: A Consumer 4:4:4 Coding Method Based On A 4:2:0 Codec.” Simulations were performed using the HM9.1 for encoding and decoding, along with format conversion source code provided by the proponents. 4:4:4 rgb video undergoes a color space conversion followed by conversion to 4:2:0, which in turn is encoded and decoded by HM9.1. The decoded 4:2:0 video is converted to 4:4:4 rgb, whose PSNR is measured with respect to the input 4:4:4 rgb video. The simulation results performed for this cross-verification match those provided by the authors of JCTVC-L0175.

# Procedure and results

A selection of 8-bit rgb sequences are converted using software provided by the proponent. This software has four options for color space filters for conversion and downsampling to 4:2:0: YCoCg, YFbFr, YFbFr2, and YCbCr. The converted videos are encoded using QP values of 17, 22, 27, and 32. The bitstreams are then decoded, and the resulting 4:2:0 sequences are converted and upsampled back to 4:4:4 rgb using the same conversion software provided by the proponents. Results are tabulated and plotted, in the manner shown for one example in Figure 1.

Figure 1. Example tabulation of results

In the spreadsheet accompanying this cross-verification document, these results also were subtracted from those provided by the proponent. If the subtraction yields zero for all elements, then the cross-verification results are identical to those tabulated by the proponent. Three sets of experiments were run: All-intra, random-access, and low-delay B. All results of the cross-verification matched those from the proponent.