| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0322_r1 |
| --- | --- |

| Title: | Comparison of Compression Performance of HEVC Draft 9 with AVC High Profile and Performance of HM9.0 with Temporal Scalability Characteristics |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Gary J. Sullivan Jizheng Xu | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution is a further study of the relative objective (i.e. PSNR-based) compression performance of HEVC Main Profile and AVC High Profile. It builds upon the prior work reported in JCTVC-G399 JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, JCTVC-K0279, updating the results by using the latest available reference software (JM-18.4 and HM-9.0). Experimental results (without class F) reportedly show that 1) for the Main Profile all-intra configuration, HM-9.0 can save about 21.9% in bit rate; 2) for the Main Profile random-access configuration, HM-9.0 can save about 34.3% in bit rate; and 3) for the Main Profile low-delay configuration, HM-9.0 can save about 36.7% in bit rate when compared with JM-18.4.

HM-9.0 in random-access configuration with enabled temporal scalability is also tested. It is reported that when enabling temporal scalability with 4 different temporal layers in random-access configuration, only 0.3% performance loss is observed on average.

# Comparison between HM-9.0 and JM-18.4

We use HM-9.0 and JM-18.4 in the tests. The JM software was configured with the same coding structure and referencing structure as HM-9.0. Both the JM anchor and HM test encoding used 8-bit encoding and decoding. The detailed JM configuration is the same as the one used in our previous contribution [6].

The average YUV PSNR (shown in the right-most column of the tables below) was calculated as

PSNRavg=6×PSNRY+PSNRU+PSNRV8

It is noted that these results are based on PSNR measurements, which are an imperfect substitute for subjective quality assessment, and that the subjective performance of HEVC may actually be generally somewhat better than its objective (PSNR) performance may indicate.

The summary results of performance comparison between AVC High Profile and HEVC Main Profile are shown in the Tables 1, 2, and 3. Details can be found in the attached spreadsheets. We do not provide the encoding time and decoding time ratio results, since the codebases are different and with different degrees of optimization; and neither of them is really speed-optimized.

The HEVC Main Profile is specified in [7], and we use the configuration files provided in [8] to generate the HEVC Main Profile bitstream.

Table 1 - Results for All Intra (AVC High Profile vs. HEVC Draft 9 Main Profile)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | −23.6% | −21.1% | −19.9% | −23.2% |
| Class B | −22.7% | −22.1% | −21.7% | −22.6% |
| Class C | −19.7% | −20.8% | −21.0% | −20.0% |
| Class D | −16.4% | −16.8% | −17.4% | −16.7% |
| Class E | −28.8% | −27.0% | −26.8% | −28.5% |
| Class F | −28.6% | −25.2% | −26.1% | −28.4% |
| Average | −23.0% | −22.0% | −21.9% | −23.0% |
| Average without F | −21.9% | −21.3% | −21.1% | −21.9% |

Table 2 - Results for Random Access (AVC High Profile vs. HEVC Draft 9 Main Profile)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | −36.6% | −37.6% | −39.4% | −36.9% |
| Class B | −39.8% | −37.5% | −37.0% | −39.5% |
| Class C | −30.4% | −32.0% | −32.1% | −30.7% |
| Class D | −28.0% | −31.3% | −33.1% | −28.9% |
| Class E |  |  |  |  |
| Class F | −31.3% | −31.1% | −32.6% | −31.6% |
| Average | −33.5% | −34.1% | −34.9% | −33.8% |
| Average without F | −34.1% | −34.8% | −35.5% | −34.3% |

Table 3 - Results for Low Delay (AVC High Profile vs. HEVC Draft 9 Main Profile)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A |  |  |  |  |
| Class B | −42.1% | −34.4% | −35.4% | −41.2% |
| Class C | −32.7% | −32.2% | −32.9% | −32.8% |
| Class D | −29.9% | −31.3% | −32.8% | −30.3% |
| Class E | −44.1% | −39.3% | −39.5% | −43.3% |
| Class F | −34.0% | −35.1% | −38.1% | −34.9% |
| Average | −36.5% | −34.2% | −35.5% | −36.4% |
| Average without F | −37.1% | −34.0% | −34.9% | −36.7% |

# HM-9.0 with temporal scalability in random-access configuration

We enabled temporal scalability in random-access configurations in HM-9.0 by modifying the configuration file a little. Only the GOP structure part is modified as below:

Four different temporal layers are used (temporal_id from 0 to 3). The summarized experimental results are shown in Table 4. The detailed experimental results and the full configuration files are also attached.

Table 4 – Results of enabling temporal scalability in random-access configuration (BD-Rate)

|  | RA-Main |  |  | RA-HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.0% | 0.1% | −0.1% | 1.0% | 0.2% | −0.1% |
| Class B | 0.3% | −0.1% | −0.1% | 0.4% | 0.2% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% |
| Class D | −0.1% | −0.1% | −0.1% | −0.1% | 0.2% | −0.2% |
| Overall | 0.3% | 0.0% | −0.1% | 0.3% | 0.2% | 0.0% |
| Class F | 0.3% | 0.2% | 0.3% | 0.2% | 0.2% | 0.2% |
| Enc. T. | 98% |  |  | 98% |  |  |
| Dec. T. | 101% |  |  | 101% |  |  |

From Table 4 we can know that enabling temporal scalability only degrades 0.3% RD performance on average for random-access coding structure. The 1.0% loss on Class A is mainly caused by two 10-bit sequences.

For low-delay coding structures, if temporal scalability is enabled, some of the pictures may not be able to use the nearest reconstructed picture as reference picture. And it will bring performance loss. We also perform some experiments on enabling temporal scalability in low-delay coding structures. Three different temporal layers are supported by using the following GOP structure.

Table 5 provides the summary reuslts and the detailed results can be found in the spreadsheet attached.

Table 5 – Results of enabling temporal scalability in low-delay configuration (BD-Rate)

|  | LB-Main |  |  | LB-HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class B | 9.4% | 5.0% | 3.7% | 9.9% | 7.1% | 5.6% |
| Class C | 10.5% | 9.2% | 9.0% | 10.7% | 10.0% | 9.4% |
| Class D | 7.5% | 5.3% | 3.8% | 7.7% | 7.1% | 5.4% |
| Class E | 8.0% | 5.1% | 3.5% | 8.9% | 8.8% | 8.1% |
| Overall | 8.9% | 6.1% | 5.0% | 9.4% | 8.2% | 7.0% |
| Class F | 19.1% | 19.8% | 20.8% | 19.1% | 19.5% | 19.5% |
| Enc. T. | 98% |  |  | 98% |  |  |
| Dec. T. | 100% |  |  | 100% |  |  |
|  | LP-Main |  |  | LP-HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class B | 7.8% | 3.5% | 2.1% | 7.9% | 4.8% | 4.1% |
| Class C | 9.1% | 7.6% | 7.2% | 9.1% | 8.1% | 7.6% |
| Class D | 7.0% | 5.4% | 3.0% | 6.8% | 6.2% | 5.4% |
| Class E | 6.9% | 5.3% | 3.5% | 7.8% | 7.9% | 7.7% |
| Overall | 7.7% | 5.3% | 3.9% | 7.9% | 6.6% | 5.9% |
| Class F | 17.8% | 18.6% | 19.0% | 17.7% | 18.8% | 18.7% |
| Enc. T. | 100% |  |  | 100% |  |  |
| Dec. T. | 101% |  |  | 100% |  |  |

# References

- JCTVC-G399, “Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile,” Geneva, CH, Nov. 2011.

- JCTVC-H0360, “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile,” San José, CA, USA, Feb. 2012.

- JCTVC-I0409, “Comparison of Compression Performance of HEVC Draft 6 with AVC High Profile,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-J0236, “Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile,” Stockholm, SE, 11–20 July 2012.

- JCTVC-J0242, “Encoding with fixed Lagrange multipliers,” Stockholm, SE, 11–20 July 2012.

- JCTVC-K0279, “Comparison of Compression Performance of HEVC Draft 8 with AVC High Profile and Performance of HM8.0 with Different Delay Characteristics,” Shanghai, CN, 10–19 Oct. 2012.

- JCTVC-K1003, “High Efficiency Video Coding (HEVC) text specification draft 9 (SoDIS),” Shanghai, CN, 10–19 Oct. 2012.

- JCTVC-K1100, “Common HM test conditions and software reference configurations,” Shanghai, CN, 10–19 Oct. 2012.