| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0323r1 |
| --- | --- |

| Title: | Specification of active reference indices and decoded picture buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhijie Yang, Brian Heng, Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Email: | zhyang@broadcom.com bheng@broadcom.com wwan@broadcom.com |
| Source: | Broadcom |  |  |

_____________________________

# Abstract

The decoded picture buffer (DPB) is used to store decoded pictures for reference and output. In H.264/AVC, the current decoded picture is stored in a separate buffer from the DPB and the maximum number of reference frames is equal to the DPB size. In the current working draft text of HEVC, the current decoded picture is directly stored into the DPB and the maximum number of reference frames is equal to the DPB size minus one. This difference causes some values of the number of the reference pictures and active reference picture list that were valid in H.264/AVC to be no longer valid or consistent in HEVC. This contribution proposes reducing the maximum number of active reference indices from 16 to 15, increasing the minimum size of the decoded picture buffer from 0 to 1 and reducing the maximum number of all types of reference pictures by 1.

# Introduction

The decoded picture buffer (DPB) is used to store decoded pictures for reference and output. In H.264/AVC, the current decoded picture is first stored in a separate buffer from the DPB and is inserted into the DPB after decoding is done. In the current working draft of HEVC [1], the current decoded picture is directly stored into the DPB. Hence, the maximum number of reference frames in H.264/AVC is equal to the DPB size, while the maximum number of reference pictures in HEVC is equal to the DPB size minus one.

## Maximum number of active reference indices

The active reference index number is the number of the reference indices used in the current slice. In H.264/AVC, the maximum number of active reference indices is equal to the maximum number of reference pictures, i.e. 16 for frame pictures and 32 for field pictures.

In HEVC, although the maximum DPB size is still 16, the maximum number of reference pictures is 15 since one picture buffer is used for the current decoded picture. On the other hand, in the current working draft text for HEVC, the maximum number of active reference indices is still 16 as shown in the following examples from Sections 7.4.2.3 and 7.4.8.1 with the relevant value highlighted in yellow:

7.4.2.3 Picture parameter set RBSP semantics

……

num_ref_idx_l0_default_active_minus1 specifies the inferred value of num_ref_idx_l0_active_minus1 for P and B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_l0_default_active_minus1 shall be in the range of 0 to 15, inclusive.

num_ref_idx_l1_default_active_minus1 specifies the inferred value of num_ref_idx_l1_active_minus1 for B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_l1_default_active_minus1 shall be in the range of 0 to 15, inclusive.

7.4.8.1 General slice segment header semantics

……

num_ref_idx_l0_active_minus1 specifies the maximum reference index for reference picture list 0 that may be used to decode the slice. num_ref_idx_l0_active_minus1 shall be in the range of 0 to 15, inclusive. …

num_ref_idx_l1_active_minus1 specifies the maximum reference index for reference picture list 1 that shall be used to decode the slice. num_ref_idx_l1_active_minus1 shall be in the range of 0 to 15, inclusive. …

Although the reference picture list could be filled up using duplicate reference pictures as described in the reference picture lists construction process, it is suggested that the specification would be clearer and more consistent using the same convention as in H.264/AVC.  More specifically, setting the maximum number of the active reference picture indices equal to the maximum number of the reference pictures.

## Decoded picture buffer size and reference picture number

Since the decoded picture buffer includes the current decoded picture, it shall always have at least one picture buffer. On the other hand, in the current working draft text for HEVC, the decoded picture buffer size max_dec_picture_buffering is allowed to have the value zero as shown in the following examples from Sections 7.4.2.1, 7.4.2.2 , 7.4.8.1 and 7.4.8.2 with the relevant value highlighted in yellow:

7.4.2.1 Video parameter set RBSP semantics

……

vps_max_dec_pic_buffering[ i ] specifies the required size of the decoded picture buffer in units of picture storage buffers when HighestTid is equal to i. The value of vps_max_dec_pic_buffering[ i ] shall be in the range of 0 to MaxDpbSize (as specified in subclause A.4), inclusive.

7.4.2.2 Sequence parameter set RBSP semantics

…..

sps_max_dec_pic_buffering[ i ] specifies the maximum required size of the decoded picture buffer in units of picture storage buffers when HighestTid is equal to i. The value of sps_max_dec_pic_buffering[ i ] shall be in the range of 0 to MaxDpbSize (as specified in subclause A.4), inclusive.

Although the maximum number of reference pictures shall be equal to the decoded picture buffer size minus one, in the current working draft text for HEVC, the value of max_dec_picture_buffering is directly used as the maximum number of reference pictures in several instances.

7.4.8.1 General slice segment header semantics

……

num_long_term_sps specifies the number of candidate long-term reference pictures specified in the active sequence parameter set that are included in the long-term reference picture set of the current picture. The value of num_long_term_sps shall be in the range of 0 to Min( num_long_term_ref_pics_sps, sps_max_dec_pic_buffering[ sps_max_sub_layers_minus1 ] −NumNegativePics[ StRpsIdx ] −  NumPositivePics[ StRpsIdx ] ), inclusive. …

num_long_term_pics specifies the number of long-term reference pictures specified in the slice header, which are included in the long-term reference picture set of the current picture. The value of num_long_term_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_sub_layers_minus1 ] − NumNegativePics[ StRpsIdx ] − NumPositivePics[ StRpsIdx ] − num_long_term_sps, inclusive. …

7.4.8.2 Short-term reference picture set semantics

……

num_negative_pics specifies the number of the following delta_poc_s0_minus1[ i ] and used_by_curr_pic_s0_flag[ i ] syntax elements. The value of num_negative_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_sub_layers_minus1 ], inclusive.

num_positive_pics specifies the number of the following delta_poc_s1_minus1[ i ] and used_by_curr_pic_s1_flag1[ i ] syntax elements. The value of num_positive_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_sub_layers_minus1 ] − num_negative_pics, inclusive.

# Proposal

After presentation of this contribution, it was suggested by the group that the proposed changes in this document were acceptable with the only change being that increasing the minimum size of the decoded picture buffer from 0 to 1 can be accomplished in an equivalent fashion by redefining the appropriate variables using the “_minus1” convention rather than modifying the minimum value from 0 to 1 as initially proposed.  To make it easier for the editors to integrate these changes, the proposed text changes are included in the attachment to this document (JCTVC-L1003_v2_with_L0323edits.doc).

- References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 9,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1003_d13, December 2012.

# Patent rights declaration(s)

Broadcom may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).