| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0324 |
| --- | --- |

| Title: | Generic Slice NAL Unit Types |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Brian Heng Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Peisong Chen Broadcom Corporation 16340 West Bernardo Dr. San Diego, CA 92127 USA | Email: Email: Email: | wwan@broadcom.com bheng@broadcom.com peisongc@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

The current working draft text of HEVC has ten different NAL unit types to specify a non-RAP slice NAL unit and six additional NAL unit types have been reserved for similar yet-to-be-defined slice types.  It is noted that H.264/AVC simply specified IDR slice and non-IDR slice units and this was sufficient for many applications because the signalled properties of these new NAL unit types were not important to these applications and/or decoders are easily capable of determining much of this information on their own.  It is also suggested that especially in an open environment, it is likely that some encoders may not understand and specify these new NAL unit types correctly.  Therefore. this contribution proposes adding generic slice NAL unit types to the HEVC standard.  Note it is not requesting removal of the new NAL unit types but simply to provide a simpler option for encoders (and more generally applications) that have no need, requirement or desire to spend resources determining whether a non-RAP slice NAL unit type fits into one of the new NAL unit types.

# Background

The HEVC WD D9 text Error: Reference source not found currently includes 10 different NAL unit types to specify a non-RAP slice NAL unit:

TRAIL_N, 	TRAIL_R

TSA_N, 	TSA_R

STSA_N, 	STSA_R

RADL_N, 	RADL_R

RASL_N, 	RASL_R

Additionally 6 NAL unit types have been reserved for similar yet-to-be-defined slice types:

RSV_VCL_N10, RSV_VCL_N12, RSV_VCL_N14
	RSV_VCL_R11, RSV_VCL_R13, RSV_VCL_R15

This requires systems to indicate and/or process leading, trailing, RADL, RASL, TSA, or STSA picture types (and possible more yet-to-be-defined) even if these properties are not needed.

H.264/AVC simply specified two NAL unit types: IDR slice and non-IDR slice NAL units.

There may be several reasons why an encoder may prefer a simpler scheme such as was specified in H.264/AVC:

- Properties of these new NAL unit types not important/useful for the application of interest

- Decoders (easily) capable of determining information of these new NAL unit types on its own without explicit signaling

- Encoders may not want to spend resources to determine which of the new NAL unit types a specific non-RAP slice NAL unit should be categorized as

- Encoders may not understand or be sure that they are using the correct NAL unit type

# Proposed Solution

It is proposed to define generic slice NAL unit types to allow encoders to choose whether they want to calculate and explicitly signal the properties of the new NAL unit types in HEVC.  More specifically, the following two NAL unit types can be defined:

SLICE_N:	Coded slice segment of a non-reference picture

SLICE_R:	Coded slice segment of a reference picture.

These generic slice types provide no indication about whether a picture is a leading, trailing, RADL, RASL, TSA, or STSA picture in case this information is not known or not needed. See Appendix for draft text changes associated with this proposed solution.

# Draft Text Changes

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0, 1 | SLICE_N, SLICE_R | Coded slice segment of a non-RAP picture slice_segment_layer_rbsp( ) | VCL |
| 2, 3 | TRAIL_N, TRAIL_R | Coded slice segment of a non-TSA, non-STSA trailing picture slice_segment_layer_rbsp( ) | VCL |
| 4, 5 | TSA_N, TSA_R | Coded slice segment of a TSA picture slice_segment_layer_rbsp( ) | VCL |
| 6, 7 | STSA_N, STSA_R | Coded slice segment of an STSA picture slice_layer_rbsp( ) | VCL |
| 8, 9 | RADL_N, RADL_R | Coded slice segment of a RADL picture slice_layer_rbsp( ) | VCL |
| 10, 11 | RASL_N, RASL_R, | Coded slice segment of a RASL picture slice_layer_rbsp( ) | VCL |
| 12, 14, 16 | RSV_VCL_N10 RSV_VCL_N12 RSV_VCL_N14 | Reserved // reserved non-RAP non-reference VCL NAL unit types | VCL |
| 13, 15, 17 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved // reserved non-RAP reference VCL NAL unit types | VCL |
| 18, 19, 20 | BLA_W_LP BLA_W_DLP BLA_N_LP | Coded slice segment of a BLA picture slice_segment_layer_rbsp( ) [Ed. (YK): BLA_W_DLP -> BLA_W_RADL?] | VCL |
| 21, 22 | IDR_W_DLP IDR_N_LP | Coded slice segment of an IDR picture slice_segment_layer_rbsp( ) [Ed. (YK: IDR_W_DLP -> IDR_W_RADL?] | VCL |
| 23 | CRA_NUT | Coded slice segment of a CRA picture slice_segment_layer_rbsp( ) | VCL |
| 24, 25 | RSV_RAP_VCL22.. RSV_RAP_VCL23 | Reserved // reserved RAP VCL NAL unit types [Ed.(YK): Some RAP related restrictions need to be specified for these 2 NUTs] | VCL |
| 26..31 | RSV_VCL24.. RSV_VCL31 | Reserved // reserved non-RAP VCL NAL unit types [Ed.(YK): Some non-RAP related restrictions may need to be specified for these NUTs] | VCL |
| 32 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 33 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 34 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 35 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 36 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 37 | EOB_NUT | End of bitsteam end_of_bitstream_rbsp( ) | non-VCL |
| 38 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 39, 40 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 41..47 | RSV_NVCL41.. RSV_NVCL47 | Reserved | non-VCL |
| 48..63 | UNSPEC48.. UNSPEC63 | Unspecified | non-VCL |

- NOTE 1 – A CRA picture may have associated RASL or RADL pictures present in the bitstream.

NOTE 2 – A BLA picture having nal_unit_type equal to BLA_W_LP may have associated RASL or RADL pictures present in the bitstream. A BLA picture having nal_unit_type equal to BLA_W_DLP does not have associated RASL pictures present in the bitstream, but may have associated RADL pictures in the bitstream. A BLA picture having nal_unit_type equal to BLA_N_LP does not have associated leading pictures present in the bitstream.

NOTE 3 – An IDR picture having nal_unit_type equal to IDR_N_LP does not have associated leading pictures present in the bitstream. An IDR picture having nal_unit_type equal to IDR_W_DLP does not have associated RASL pictures present in the bitstream, but may have associated RADL pictures in the bitstream.

NOTE 4 – When the value of nal_unit_type is equal to SLICE_N, TRAIL_N, TSA_N or STSA_N, the decoded picture is not included in any of RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr of any picture with the same value of TemporalId. A coded picture with nal_unit_type equal to SLICE_N, TRAIL_N, TSA_N or STSA_N may be discarded without affecting the decodability of other pictures with the same value of TemporalId.

- NOTE 7 – When a picture has nal_unit_type equal to SLICE_N or SLICE_R, no indication is given regarding whether the picture is a leading, trailing, RADL, RASL, TSA, or STSA picture.

The variable IdrPicFlag is specified as

IdrPicFlag = ( nal_unit_type  = =  IDR_W_DLP  | |  nal_unit_type  = =  IDR_N_LP )	(7-1)

The variable RapPicFlag is specified as

RapPicFlag = ( nal_unit_type  >=  18  &&  nal_unit_type  <=  25 )	(7-2)

All coded slice segment NAL units of an access unit shall have the same value of nal_unit_type.

Each picture, other than the first picture in the bitstream, is considered to be associated with the previous RAP picture in decoding order.

When a picture is a leading picture, it shall be a RADL or RASL picture.

When a picture is a trailing picture, it shall not be a RADL or RASL picture.

When a picture is a leading picture, it shall precede, in decoding order, all trailing pictures that are associated with the same RAP picture.

No RASL pictures shall be present in the bitstream that are associated with a BLA picture having nal_unit_type equal to BLA_W_DLP or BLA_N_LP.

No RASL pictures shall be present in the bitstream that are associated with an IDR picture.

No RADL pictures shall be present in the bitstream that are associated with a BLA picture having nal_unit_type equal to BLA_N_LP or that are associated with an IDR picture having nal_unit_type equal to IDR_N_LP.

- NOTE 5 – It is possible to perform random access at the position of a RAP access unit by discarding all access units before the RAP access unit (and to correctly decode the RAP picture and all the subsequent non-RASL pictures in decoding order), provided each parameter set is available (either in the bitstream or by external means not specified in this Specification) when it needs to be activated.

Any picture that has PicOutputFlag equal to 1 that precedes a RAP picture in decoding order shall precede the RAP picture in output order and shall precede any RADL picture associated with the RAP picture in output order.

Any RASL picture associated with a CRA or BLA picture shall precede any RADL picture associated with the CRA or BLA picture in output order.

Any RASL picture associated with a CRA picture shall follow, in output order, any RAP picture that precedes the CRA picture in decoding order. [Ed. (TK) Why do we need this restriction? (GJS): Just so that each RAP-to-RAP chunk is a roughly-sensible segment of the video content, I suppose. We have other such "unnecessary" constraints, such as the constraint that RASL's need to be leading pictures and the constraints on relative decoding order of RASL, RADL and trailing pictures.]

When sps_temporal_id_nesting_flag is equal to 1 and TemporalId is greater than 0, the nal_unit_type shall be equal to TSA_R, TSA_N, RADL_R, RADL_N, RASL_R, or RASL_N.

8.3.2	Decoding process for reference picture set

- There shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr for which one or more of the following are true.

  - The entry is equal to "no reference picture".

  - The entry is a picture that has nal_unit_type equal to SLICE_N, TRAIL_N, TSA_N, STSA_N, RADL_N, RASL_N, RSV_VCL_N10, RSV_VCL_N12, or RSV_VCL_N14 and TemporalId equal to that of the current picture.

  - The entry is a picture that has TemporalId greater than that of the current picture.

D.2.18	SOP description SEI message semantics

sop_desc_nal_ref_flag[ i ] equal to 0 indicates the i-th picture in decoding order within the SOP has a nal_unit_type equal to SLICE_N, TRAIL_N, TSA_N, STSA_N, RADL_N, RASL_N, RSV_VCL_N10, RSV_VCL_N12, or RSV_VCL_N14. [Ed. TK. This was changed since nal_ref_flag has been removed. Perhaps sop_desc_nal_ref_flag[ i ] can be removed if no longer useful.]

The variable nalRefFlag[ j ] is specified as follows for the j-th picture in decoding order starting from j equal to 0 for the first picture of the SOP containing the picture associated with the SEI message.

–	If the j-th picture had nal_unit_type equal to SLICE_N, TRAIL_N, TSA_N, STSA_N, RADL_N, RASL_N, RSV_VCL_N10, RSV_VCL_N12, or RSV_VCL_N14, nalRefFlag[ j ] is set to 0.

–	Otherwise, nalRefFlag[ j ] is set to 1.

D.2.22	Temporal level zero index SEI message semantics

The variable picForReference is specified as follows:

–	If TemporalId is equal to sps_max_sub_layers_minus1 and nal_unit_type is equal to SLICE_N, TRAIL_N, TSA_N, STSA_N, RADL_N, RASL_N, RSV_VCL_N10, RSV_VCL_N12 or RSV_VCL_N14, picForReference is equal to 0.

–	Otherwise, picForReference is equal to 1.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 9,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1003_d13, December 2012.

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).