| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0325r2 |
| --- | --- |

| Title: | Bounding redundant SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Waqar Ahmad Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Email: Email: | wwan@broadcom.com waqarahmad@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This document proposes a restriction on the number of redundant SEI messages. During the discussion of document K0120 at the 12th JCT-VC meeting, it was noted that redundant copies of SEI messages are currently allowed with no constraint on the number of these copies. It is recognized that a small number of redundant copies may be useful to improve error resilience characteristics of a bitstream but suggested that allowing an unbounded number of redundant copies does not seem to have any practical functionality and increases the burden on decoder implementations.  Therefore, this document proposes bounding redundant SEI messages as a requirement of bitstream conformance.

# Background

The current working draft [1] allows for redundant copies of SEI messages with no constraint on the number of these copies. Such redundant repetition, may be useful for certain applications, but can put an unnecessary performance burden on decoder implementations. More specifically, some practical decoder implementation allocate less than 1% of their cycles for processing of SEI messages. Allowing for unbounded repeats of SEI messages would result in implementation needing to unnecessarily allocate more resources for no or questionable functional benefit. Bounding the maximum repetition rate to a small practical limit eliminates this problem without compromising on the functionality of repeating SEI messages.

# Proposed Solution

It is proposed that redundant repeats of SEI message within an SEI payload which are currently unbounded be bounded. For example a practical limit of 6 repeats should be enough to satisfy redundancy requirements, e.g. for error resiliency. Note that the proposal here is mainly focused on having a bound and the suggested value of 6 is simply a value that the authors deem to be a reasonable number that the decoders can rely on to keep their processing budget to less than 1% for SEI message processing.

# Proposed text changes

After presentation of this contribution, it was suggested by the group that a statement limiting the repetition of each SEI message be included in the semantics of each SEI message instead of a general statement that applies to all SEI messages to handle potential confusion over scope and allow for unique cases.  It was also suggested to increase the limit from 6 to 8.  The proposed text changes are included in the attachment to this document (JCTVC-L1003_v1_with_L0325edits.doc).

Note that the proposed text changes apply the constraint of a maximum of eight messages with the same SEI payload data as follows:

- The constraint is specified per decoding unit for the decoding unit information SEI message.

- The constraint is not specified for the filler payload, user data registered and user data unregistered SEI messages as there was concern expressed by the group at limiting the number of these SEI messages.

- The constraint is bounded by the number of slice segments instead of 8 for the region refresh information SEI message as suggested by the group.

- The constraint is specified per access unit for other SEI messages.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 9,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1003_d13, December 2012.

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).