| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0330 |
| --- | --- |

| Title: | Test of Transform Selection for Inter-Layer Texture Prediction on SMuC 0.1.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Marta Karczewicz Jianle Chen 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

# A transform selection scheme was presented in JCTVC Shanghai meeting (JCTVC-K0035 and K0321). Multiple transforms are allowed for coding the luma component of inter-layer texture prediction residues. This contribution reports the test result of this scheme on SMuC0.1.1. With 3 candidate transforms, experimental results reportedly show luma BD-rate reductions of -1.67% and -1.45% for AI-2X, AI-1.5X respectively.

# Introduction

Inter-layer texture prediction is an important tool for scalable video coding where base layer reconstructed pixels are used to predict enhancement layer pixels. In light of that residue characteristics of the inter-layer texture prediction can be different from those of the single layer residues, a transform selection scheme was proposed in [1] and [2]. In addition to the two transforms in the HEVC main profile (i.e., DCT Type-2 and DST Type-7), additional DCT and DST transforms are applied to the enhancement layer to achieve better residue energy compaction. This contribution reports the test results of this scheme on SMuC0.1.1.

# Proposed Method

Three candidate transforms can be used for encoding the luma component of inter-layer texture prediction residues. These three transforms are derived based on the DST Type-3, DCT Type-2 and DST Type-3 transforms. The transform selection method is not applied to chroma components, for which only the existing HEVC transform (DCT Type-2) is used.

For each CU encoded with inter-layer texture prediction mode, the encoder selects a transform and signals the transform_selection_index to the decoder. The signaling is skipped if the CU does not contain non-zero coefficients. The syntax element transform_selection_index is coded using a truncated unary code with CABAC. The contexts for each bin and each CU size are separated, so there are 8 (2 bins x 4 CU sizes) contexts in total for this syntax. The mapping between transform_selection_index and transform type is shown in Table 1.

Table 1 Candidate Transforms

| transform_selection_index | Binarization | Transform |
| --- | --- | --- |
| 0 | 0 | DST Type-3 |
| 1 | 10 | DCT Type-2 |
| 2 | 11 | DCT Type-3 |

2.1 DCT Type-3 and DST Type-3

The transform coefficients of DCT Type-3 and DST Type-3 can be simply derived from the coefficients of DCT Type-2.

Let A, B and C be the N×N transform matrices of DCT Type-2, DCT Type-3 and DST Type-3, respectively. For the element at the i-th row and j-th column,

,

,   for even i,

,  for odd i.

It can be seen DCT Type-3 is the transpose of DCT Type-2, and thus the forward transform of DCT Type-3 can be implemented using the pipeline of the inverse transform of DCT Type-2, and vice versa. For DST Type-3, the input coefficients need extra row flipping and sign inverting for odd columns before reusing the transform pipeline.

2.2 Fast Transform Selection

To reduce the complexity, the encoder first tests each candidate transform with 1 level RQT. The one with the lowest RD cost will be selected and a full RQT will be performed with this transform.

# Experimental results

The proposed method is implemented on SMuC 0.1.1. Anchor is generated using the same software with IntraBL4x4 transform bug-fix (both forward and backward transforms are DCT type-2).  Test conditions specified in [3] are used. BD-rate is calculated using both enhancement layer and base layer rates. Running time is measured on a homogeneous cluster. Besides testing the 3 transforms case, a 2 transforms version (DCT Type-2 and DCT Type-3) is also tested.

2.1 Results with 3 candidate transforms

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.86% | 1.62% | 1.65% |  |  |  |
| Class B | -1.60% | 1.47% | 1.40% | -1.45% | 2.38% | 2.39% |
| Overall (EL+BL) | -1.67% | 1.51% | 1.47% | -1.45% | 2.38% | 2.39% |
| Overall (EL) | -3.12% | 3.41% | 3.30% | -4.58% | 8.08% | 8.17% |
| Enc Time[%] | 111.94% |  |  | 107.77% |  |  |
| Dec Time[%] | 100.30% |  |  | 97.26% |  |  |

2.2 Results with 2 candidate transforms

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.25% | 1.10% | 1.15% |  |  |  |
| Class B | -1.06% | 0.96% | 0.93% | -0.97% | 1.53% | 1.56% |
| Overall (EL+BL) | -1.11% | 1.00% | 0.99% | -0.97% | 1.53% | 1.56% |
| Overall (EL) | -2.08% | 2.29% | 2.24% | -3.09% | 5.35% | 5.44% |
| Enc Time[%] | 108.25% |  |  | 102.22% |  |  |
| Dec Time[%] | 98.73% |  |  | 98.89% |  |  |

# Conclusions

This contribution reports the result of transform selection for inter-layer texture prediction on SMuC 0.1.1. In addition to DCT Type-2 in HEVC reference software, two extra transforms (DCT Type-3 and DST Type-3) are introduced. The implementation of the extra transforms can reuse the transform module in the reference software, thus having minor impact on implementation cost.

# References

- J. Chen, et al. “Description of scalable video coding technology proposal by Qualcomm (configuration 1)”, JCTVC-K0035, Shanghai, China, Oct. 2012.

- L. Guo, M. Karczewicz, J. Sole and J. Chen, “Transform Selection for Inter-Layer Texture Prediction in Scalable Video Coding”, JCTVC-K0321, Shanghai, China, Oct.2012.

- L. Guo, Y. He, D. -K. Kwon, J. Zan, H. Lakshman, and J.-W. Kang, “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC”, JCTVC-K1102, Shanghai, China, Oct. 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).