| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0333 |
| --- | --- |

| Title: | CE1: Test 3 - Square transform blocks for 4:2:2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This proposal describes the usage of square transform blocks for the chroma components in the HM Range Extension 4:2:2 model. The reference benchmark for CE1 (JCTVC-K1121) uses rectangular transform blocks. Square transforms average BD-rate losses are from 0.1% to 0.7% for luma and from 2.0% to 5.4% for chroma components depending on the configuration and the QP range.

# Technical description

HEVC Range Extensions reference software uses rectangular transforms for the 4:2:2 chroma components. Core Experiment 1 [1] tests the usage of two square transforms instead of one rectangular transform. Test 3 compares the performance of square transforms for all chroma block sizes against rectangular transforms, which is the CE anchor.

## Technical details

In CE1, a software was released with the following features:

- Two square transforms instead of a rectangular one for chroma components

  - For each chroma component, signaling of two cbf and invoking twice residual coding

  - Transform skip enabled: one flag per 4x4 transform

  - No need for the 32-point transform in the chroma path

- Removed the QP increase by 3 associated to the rectangular transforms

- Removed the scaling lists associated to the rectangular transforms

- Kept the intra prediction modifications (angular and planar modes) for rectangular block prediction

## Issues

Received comments from the CE participants on the released software are the following:

- Although the bitstream contains separate transform skip flags for each 4x4 block resulting from splitting the 4x8 block, the decoder is not able to handle the case where the two flags have differing values. In the implementation, this is not an issue because the encoder does not test this case.

- The reconstruction process for intra-predicted blocks produces a Nx2N block, so the residual from the upper NxN block is not added to reference samples for the lower NxN block.

The following actions have been taken:

- A software solving the first point above is attached to this proposal: the decoder is able to handle two different transform skip flags in a 4x8 block. Note that the generated bitstreams are the same.

- Reconstruction of the upper square in the loop of the prediction of the lower square might be more aligned with the HEVC design for intra coding. This case has been implemented and proposed in related contribution in this meeting.

# Simulation results

The method is implemented on top of the HM9.0.1_RExt software. Simulations are carried under the common test conditions for range extensions [2] and for the RDOQ off case. The performance is compared to the anchor in terms of BD-rate savings. Table 1 and 2 show the performance for common conditions and RDOQ off, respectively.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 1.0% | 2.6% | 7.4% | 0.9% | 1.4% | 4.8% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.8% | 4.3% | 5.3% | 0.6% | 2.4% | 3.3% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.7% | 3.9% | 6.5% | 0.7% | 2.4% | 3.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.9% | 2.7% | 3.9% | 0.6% | 1.7% | 1.7% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.6% | 2.1% | 2.1% | 0.4% | 1.8% | 0.4% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.3% | 4.6% | 4.1% | 0.2% | 1.1% | 0.6% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.9% | 5.1% | 5.6% | 0.7% | 3.1% | 2.9% |
| Overall | 0.7% | 3.6% | 5.0% | 0.6% | 2.0% | 2.4% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 91% |  |  | 96% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.7% | 2.1% | 6.1% | 0.4% | 0.6% | 4.1% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.3% | 5.2% | 4.5% | 0.4% | 2.7% | 4.5% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.5% | 6.5% | 7.6% | 0.2% | 3.2% | 6.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.5% | 2.2% | 3.1% | 0.4% | 1.6% | 1.7% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.1% | 4.3% | 5.4% | 0.2% | 2.0% | 1.9% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.2% | 4.6% | 4.6% | 0.0% | 3.1% | 2.5% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.5% | 5.2% | 6.2% | 0.3% | 3.8% | 4.1% |
| Overall | 0.4% | 4.3% | 5.4% | 0.3% | 2.4% | 3.6% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.3% | 1.8% | 5.5% | 0.0% | 0.5% | 3.8% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.0% | 4.1% | 3.8% | 0.0% | 2.3% | 3.9% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.2% | 6.6% | 8.2% | -0.2% | 4.1% | 7.3% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.3% | 2.1% | 2.9% | 0.3% | 1.7% | 1.8% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.1% | 3.4% | 4.2% | 0.3% | 1.5% | 1.2% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.1% | 2.8% | 2.0% | 0.1% | 2.3% | 1.3% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.2% | 5.1% | 6.2% | 0.0% | 3.7% | 3.9% |
| Overall | 0.2% | 3.7% | 4.7% | 0.1% | 2.3% | 3.3% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Table 1. BD-rates for common conditions

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.9% | 2.5% | 7.4% | 0.6% | 1.5% | 5.2% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.6% | 3.3% | 5.8% | 0.3% | 2.2% | 3.9% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.6% | 3.5% | 7.1% | 0.4% | 1.9% | 3.9% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.5% | 2.7% | 4.2% | 0.2% | 1.5% | 2.3% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.4% | 2.0% | 3.2% | 0.1% | 1.3% | 1.8% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.0% | 4.6% | 4.4% | -0.2% | 1.5% | 1.4% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.7% | 4.8% | 5.5% | 0.4% | 2.8% | 3.0% |
| Overall | 0.5% | 3.3% | 5.4% | 0.3% | 1.8% | 3.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 94% |  |  | 96% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.8% | 2.3% | 6.6% | 0.5% | 0.6% | 4.7% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.2% | 5.6% | 4.7% | 0.3% | 2.9% | 4.6% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.5% | 6.6% | 8.1% | 0.2% | 3.2% | 6.4% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.4% | 2.7% | 3.8% | 0.3% | 1.7% | 2.5% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.2% | 3.6% | 4.4% | 0.3% | 1.5% | 2.4% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.1% | 6.0% | 5.8% | 0.0% | 3.7% | 3.3% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.5% | 5.7% | 6.8% | 0.3% | 4.0% | 4.8% |
| Overall | 0.4% | 4.6% | 5.7% | 0.3% | 2.5% | 4.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.5% | 1.7% | 5.2% | 0.4% | 0.5% | 3.9% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | 4.5% | 3.7% | 0.3% | 2.0% | 4.3% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.1% | 6.9% | 8.1% | -0.1% | 3.7% | 6.4% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.3% | 2.6% | 3.4% | 0.4% | 2.0% | 2.4% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.2% | 3.1% | 3.7% | 0.3% | 1.3% | 2.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.2% | 5.3% | 4.1% | 0.1% | 3.4% | 2.5% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.3% | 5.1% | 6.0% | 0.2% | 3.9% | 4.0% |
| Overall | 0.2% | 4.2% | 4.9% | 0.2% | 2.4% | 3.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  |

Table 2. BD-rates for RDOQ off case

# References

- J. Sole, “HEVC Range Extensions Core Experiment 1: Square and non-square transforms for 4:2:2 chroma format coding”, Joint Collaborative Team on Video Coding, JCTVC-K1121, October 2012.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions” Joint Collaborative Team on Video Coding, JCTVC-K1006, Shanghai, October 2012.

# Patent rights declaration

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).