| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0334 |
| --- | --- |

| Title: | Color Gamut Scalable Video Coding: New Results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Louis Kerofsky, Andrew Segall, Seung-Hwan Kim, and Kiran Misra 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-360-817-87644 lkerofsky@sharplabs.com asegall@sharplabs.com kimse@sharplabs.com kmisra@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Abstract

This contribution reports new results in the area of color gamut scalable video coding.  In the last meeting, JCTVC-K0241 proposed a color gamut scalable system employing a gain-offset model to map the color gamut of the baselayer to the color gamut of the enhancement layer.  Conversion from BT.709 to BT.2020 was the main emphasis, which was asserted to correspond to an application scenario with an HD baselayer and UHDTV enhancement layer.  Here, new results are reported for the method.  These new results include the use of the SMuC software, experiments combining color gamut and spatial scalability, and simulations with additional, wide color gamut sequences.  Results show an average enhancement layer bit-rate savings of 84% and 74% for AI and RA color space scalability scenarios, and an average enhancement layer bit-rate savings of 36% and 26% for AI and RA combined spatial and color space scalability simulations.

# Introduction

The upcoming deployment of UHDTV devices and content will use a different color gamut than legacy devices.  Specifically, HD uses the BT.709 recommendation [1], while UHDTV will use the BT.2020 recommendation [2].  A key difference between these systems is that the color gamut of UHDTV is significantly larger than HD.  It is asserted that will provide a more “life like” viewing experience, which is consistent with other UHDTV characteristics, such as high resolution.

With the scalable extension design beginning within JCTVC, it is important to completely address the requirements of future applications that require scalability.  As captured in previous documents, a primary motivation for the spatial extension work in JCTVC is to support UHDTV spatial resolutions with HD resolution base layers.  Moreover, the project requirements document capture that an enhancement layer and a base layer in the developing system should support different color gamuts.  Here, we propose a system to support color gamut scalability to achieve these goals.

The diagram of a general scalable encoder including a prediction tool for color and bit-depth differences between the base and enhancement layers is shown in Figure 1.  The key features illustrated are that the inputs for the base and enhancement layers are represented in different color gamuts, and that the enhancement layer encoder is able to use reconstructed frame from the decoder during encoding the enhancement layer.

Figure 1 Color Space Scalable Coder

We note that our system completely supports a change in bit-depth between the enhancement and base layer color gamuts as well, which is motivated by the definition of BT.2020 as either a 10-bit or 12-bit color gamut, as compared to the common use of 8-bit BT.709 in consumer applications.  Moreover, we want to clarify that “color gamut scalability” differs from the idea of “color format scalability” previously addressed by the video coding community i.e. [2].  This previous work is concerned with the change in the chroma subsampling format, or resolution rather than the color gamut.

# Relation between BT.709 and BT.2020

Both BT.709 and BT.2020 define representations of color pixels in luma and chroma components. Each recommendation defines conversion to and from the CIE-XYZ linear color space.  This common intermediate can be used to define the conversion of BT.709 luma and chroma values to corresponding BT.2020 luma and chroma values.  See for an illustration of the computation.  Details can be found in the text of the relevant recommendations.

.

Figure 2 Conversion from BT.709 to BT.2020

We illustrate the difference by presenting the BT.709 and BT.2020 versions of a frame below.  When displayed on appropriately calibrated monitors, these images would be identical.  Note the desaturation of color seen in the BT.2020 image.  This desaturation is due to the more saturated primaries of BT.2020 in comparison to BT.709.

Figure 3 BT.709 image

Figure 4 BT.2020 image

## BT.709 to BT.2020 transformation

Studying the transformation from BT.709 to BT.2020 lends some insight to construction of a predictor between color spaces.  The full conversion can be defined by a series of equations with specific values of the matrices defined in the relevant specifications. The transform is analyzed in detail in appendix A.

The composition of five basic functions gives the nonlinear conversion between BT.709 and BT.2020.  This can be expressed as:

(1)

At each point, the full transform can be approximated by a matrix gain and vector offset model derived from the Jacobian:

(2)

The Jacobian is straight forward to calculate from the component functions. The result takes the form:

(3)

Note that several elements of this matrix product are constant but that the second and forth terms depend on the position where they are evaluated.  We illustrate the range of the nine elements of the Gain matrix at various (R,G,B)  points of BT.709 in Figure 5.  In this figure,  the rgb pixel values range over the cube [0,1]x[0,1]x[0,1] but are represented on 1 dimension for easy of visualization. Many of the off diagonal elements are near zero.  The diagonal elements of the gain matrix are plotted in Figure .  The diagonal elements show variation between color channels as well as variation with the RGB pixel value.

Figure 5 All 9 Gain elements vs. pixel

Figure 6 Diagonal Gain elements vs. pixel

The average gain matrix over samples of BT.709 is given by

1.1853    0.0118    0.0715

-0.0189    0.9515   -0.0504

0.1453    0.0134    0.6698

This average treats all points in BT.709 equally.  We emphasize that this is likely suboptimal – but it does allow us to see a few properties.  These properties are consistent with the scatter plots of BT.709 and BT.2020 data examined previously.

- This matrix is nearly diagonal and can be further approximated by a diagonal matrix giving a gain and offset per color channel c.  This gives a complexity reduction in the color space prediction process by allowing components to be independently processed.

- The gains for each color component differ ranging from 1.185 to 0.6698 for this average. The need to support different gains per color component is important to improved prediction accuracy.

- The gains vary with position suggesting the value of adaptively selecting parameters based on image content.

Note the quantization of YCbCr values to digital representation in 8-bit BT.709 and 10-bit BT.2020 introduces additional gain and offset factors from this analysis presented above. The difference in bit-depth contributes a factor of 4 to the gain and the offset of Cb and Cr elements by 128 and 512 alters the offset values.  These changes are compatible with the gain offset prediction model and are taking into account when predicting 10-bit BT.2020 digital samples directly from 8-bit BT.709 samples. From this analysis a gain offset model allowing different gain and offset values per color channel and allowing the parameters to be varied based on image content is suggested.  This model can act directly on the quantized digital representations of Y, Cb and Cr components.

This analysis is insightful but scatter plots of the relation between color channels of BT.709 and BT.2020 for sample frames indicate the value of the general gain offset model.  The three plots below show the relation between BT.709 and BT.2020 values for individual Y, Cb, and Cr color planes of a sample image.  This is similar to the results in our prior contribution illustrated the relation between BT.709 and BT.2020 content [5].  Each plot shows a scatter plot of BT.2020 data vs. BT.709 data along with a line showing the relation assuming a bit-depth difference only and a gain offset model fit to the data.

(a)

(b)

(c)

Figure 7 Relation of BT.709 and BT.2020

The bit-depth based predictor suffers particularly on the U and V channels where the general Gain and Offset predictor fits much better.

# Predictor for CGS

The relation between BT.709 and BT.2020 suggests a gain offset model is an effective prediction of BT.2020 pixels from corresponding BT.709 pixels.  We can expect this prediction to have limitations due not only to the linear approximation introduced about for pixels in the BT.709 gamut but also when the representation of the pixel in BT.2020 does not lie in the BT.709 gamut.

A general gain offset model is used for the prediction allowing flexibility in the case where significant fraction of pixel do not lie in the BT.709 gamut or other color space combinations are used.

The color prediction process is described by a gain and offset model.  The model operates on individual color planes.  To facilitate integer calculation, a parameter describes the number of fraction bits used in the calculation in the parameter numFractionBits.  For each channel a gain[c] and offset[c] are specified.  The prediction is defined by

Pred[c][x][y] =  (gain[c]*In[x][y] + (1<<(numFractionBits-1))>> numFractionBits + offset[c]

# Syntax for CGS

Signalling of the prediction parameters is performed by adding additional syntax elements to PPS when pps_extension_flag is set.  The syntax element, use_color_prediction, is used to indicate the use of color prediction on the current picture as shown in Table 1.

Table 1 signaling of the prediction parameters

pps_extension_flag

if( pps_extension_flag )

use_color_prediction

if(use_color_prediction)

color_pred_data ( )

rbsp_trailing_bits( )

If the use_color_prediction flag is equal to ‘1’, the color_pred_data function is called to signal color prediction parameters. As shown in Table 2, color prediction parameters include numFractionBits, color_predictor_gain[] array, and color_predictor_offset[] array and those are signalled or inferred according to ‘color_prediction_model’.

Table 2 syntax elements for color_pred_data

color_prediction_model

if(color_prediction_model==2) {

numFractionBits

for( I = 0; I < 3; i++ ){

color_predictor_gain [ I ]

color_predictor_offset [ I ]

}

}

Table 3 signaling the ‘color_prediction_model’ parameter

| color_prediction_model | Name |
| --- | --- |
| 0 | Bit Increment |
| 1 | Fixed Gain Offset |
| 2 | Gain Offset Picture Adaptive |

Depending on the color_prediction_model parameters are either inferred or signalled.

Bit Increment: The number of fraction bits and values of the offest array are zero.  The values of the gain array are all equal and calculated from the difference in bit depth between the base and enhancement layers gain[c] = 1<<( bitdepthEL – bitdepthBL)

Fixed Gain Offset: The model parameters are defined by the specification for instance for given color space difference.  The number of fraction bits, gain and offest arrays are defined.  The values used for the GO-NA model for all test conditions are listed in the results section.

Gain Offset Picture Adaptive:  The model parameters are signaled in the bitstream.  The signalling includes numberFractionBits, gain[] and offset[] for each color component.

# Selection of Adaptive Gain Offset

The adaptive functionality was used to design prediction parameters for each sequence based on the first picture.  The algorithm used the decoded, possibly upsampled, base layer picture and the input enhancement layer picture.  Gain and offset values were designed for each color component to minimize the prediction MSE between the predictor constructed from the base layer image and the input enhancement layer image.  A conventional MSE minimization method was used as in our prior work [5].  The MSE minimization is of comparatively low complexity and done once per sequence.  This design criterion is suboptimal in many cases but shows the potential of adapting the parameters to some sequence content.  The syntax supports signalling more optimal design parameters.

# Color Predictor and Upscaling combination.

The current implementation applies the color predictor following any upsampling used to support resolution scalability.  With this order, the bit-depth of the interpolation filter input remains at 8-bit and the bit-depth increase corresponding to BT.2020 is applied following interpolation.  This is not a fundamental design restriction but was chosen to use the existing SMUC interpolation on 8-bit content.

The current implementation applies the color prediction process following the full up sampling calculation and limitation to 8-bit samples.  The upsampler was not modified from the SMuC software (version 0.1.1) in these tests.

# Experiment conditions

The experimental conditions for evaluating the scalable codec are described in this section.  These include: coding conditions, quantizer values, parameter design, and additional wide color gamut content.

## Coding conditions:

The coding conditions tested include Random Access and All Intra configurations with both equal resolution for base and enhancement layers and 2x upscaling.  These are denoted as: RA-CGS, RA-2X, AI-CGS, AI-2X.

## Quantizer values:

The proposed color gamut scalable codec is compared to simulcast using HM8.1.  Previously we studied the relation between BT.709 and BT.2020 and determined that corresponding color channels are correlated.  The BT.709 to BT.2020 color space conversion is nonlinear in general but a linear approximation of the relation within color channels gives different gains per color channel.  For similar visual fidelity of BT.2020 and BT.709 image representations, the quantizer bin widths should be related by the corresponding gain. This is relevant to balance the RDOQ performance among color channels of the BT.2020 layer.

BinWidth[c]2020 = Gain[c]*BinWidth[c]709

This translates to a relation between QP parameters of

QP[c]2020 = QP[c]709  + 6*log2(Gain[c])

The current HM software accounts for the bit-depth difference between 8-bit BT.709 and 10-bit BT.2020 by using an offset of 12 in the QP value independent of channel.  The Luma channel is roughly scaled by 4 which corresponds to the bit-depth change from 8-bit BT.709 to 10-bit BT.2020. So no change in the QP value for base layer and enhancement layer is needed.  The chroma components have gains less than 4.  As a consequence, the relative quantizer bin-width should differ between the color components of BT.709 and BT.2020.  This is achieved using the Chroma QP offsets of HM.  We use QP chroma offsets of -2 and -6 respectively for the Cb and Cr color channels.

For spatial scalable results, we test with a difference in QP between the base and enhancement layer.  The chroma QP offsets of -2 and -6 are used to account for the scaling of Cb and Cr channels.

When it comes to QP settings for scalable coding, 4 base layer QP points (22,26,30,34) are tested, and the difference QP between the base layer  QP and enhancement QP are set to ‘0’ and ‘2’ for SNR scalable test and spatial scalable test, respectively.

## Parameter design:

Three parameter design methods are tested: Bit Increment (BI), Gain Offset Sequence Adaptive (GO-SA), and Gain Offset NonAdaptive (GO-NA).

The BI design uses the bit-depth difference between 8-bit BT.709 and 10-bit BT.2020 resulting in a predictor which has gain four and offset zero for all content and QP values.

The GO-SA design uses the first frame of the sequence to design the prediction parameters. The base layer frame is reconstructed and up sampled as appropriate to match the resolution of the enhancement layer frame.  The GO-SA prediction parameters are designed to minimize the MSE between the input EL frame and the reference constructed from the BL.  This is done once for the first frame of the sequence under each different coding condition.

The GO-NA design uses a single (fixed) parameter set for all sequences, QP levels and coding conditions.  The results of the GO-SA design were examined and an average set of parameters selected for the set of coding experiments.  Naturally these are biased to the set of sequences and conditions used for these experiments but provide one example to understand the benefit of tuning the predictor based on image content.

## Wide Gamut source content.

We evaluated the codec on five additional wide color gamut sequences SWG-1 to SWG-5. These were originally captured in the xvYCC wide color gamut space and then mapped to BT.2020 and BT.709 respectively.  An indication of the color gamut of the BT.2020 sequence SWG-1 is shown below.  Note the number of pixels outside of the BT.709 gamut.

Figure 8 Color Gamut of SWG-1

# Results

Comparison of the three color gamut prediction methods with HM8.1 simulcast is given in the attached spreadsheet. Summaries of the BD rate performance between HM8.1 simulcast and each prediction method are shown below.

Table 4 comparison of BD rate performance between HM8.1 simulcast vs. BI

|  |  | AI HEVC 2x |  |  | AI HEVC CGS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A | -25.10% | -29.60% | -26.90% | -45.40% | -45.00% | -41.70% |
|  | Class B | -17.10% | -21.30% | -17.50% | -43.30% | -42.50% | -38.10% |
|  | SWG | -12.10% | -16.00% | -13.50% | -24.70% | -23.60% | -22.40% |
|  | AVG | -18.10% | -22.30% | -19.30% | -37.80% | -37.03% | -34.07% |
| EL | Class A | -43.50% | -47.20% | -44.80% | -91.00% | -90.80% | -90.30% |
| Only | Class B | -29.00% | -32.60% | -29.00% | -86.90% | -86.40% | -85.30% |
|  | SWG | -22.40% | -25.70% | -23.50% | -49.80% | -48.60% | -47.80% |
|  | AVG | -31.63% | -35.17% | -32.43% | -75.90% | -75.27% | -74.47% |
|  |  | RA HEVC 2x |  |  | RA HEVC CGS |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A | -16.60% | -10.40% | 1.30% | -40.80% | -35.30% | -8.50% |
|  | Class B | -11.60% | -11.30% | -5.10% | -38.10% | -32.70% | -11.80% |
|  | SWG | -7.80% | -9.10% | -4.60% | -20.40% | -17.40% | -14.90% |
|  | AVG | -12.00% | -10.27% | -2.80% | -33.10% | -28.47% | -11.73% |
| EL | Class A | -31.20% | -26.20% | -15.90% | -81.50% | -80.00% | -72.30% |
| Only | Class B | -20.60% | -20.10% | -14.50% | -76.30% | -73.90% | -66.40% |
|  | SWG | -13.30% | -14.40% | -10.00% | -37.10% | -34.40% | -32.40% |
|  | AVG | -21.70% | -20.23% | -13.47% | -64.97% | -62.77% | -57.03% |

Table 5 comparison of BD rate performance between HM8.1 simulcast vs. GO-SA

|  |  | AI HEVC 2x |  |  | AI HEVC CGS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A | -26.90% | -31.40% | -31.60% | -47.60% | -48.20% | -51.00% |
|  | Class B | -19.80% | -24.00% | -22.10% | -46.20% | -46.40% | -47.80% |
|  | SWG | -16.10% | -20.70% | -19.40% | -30.90% | -29.60% | -29.90% |
|  | AVG | -20.93% | -25.37% | -24.37% | -41.57% | -41.40% | -42.90% |
| EL | Class A | -46.20% | -49.70% | -49.70% | -95.70% | -95.70% | -95.90% |
| Only | Class B | -33.00% | -36.70% | -34.70% | -93.10% | -93.00% | -93.10% |
|  | SWG | -29.70% | -33.20% | -32.20% | -62.20% | -60.80% | -61.00% |
|  | AVG | -36.30% | -39.87% | -38.87% | -83.67% | -83.17% | -83.33% |
|  |  | RA_HEVC_2x |  |  | RA HEVC CGS |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A | -18.30% | -10.50% | -8.70% | -44.60% | -38.70% | -43.30% |
|  | Class B | -14.00% | -12.40% | -9.20% | -42.40% | -36.10% | -38.80% |
|  | SWG | -10.80% | -10.90% | -5.80% | -26.20% | -18.70% | -17.20% |
|  | AVG | -14.37% | -11.27% | -7.90% | -37.73% | -31.17% | -33.10% |
| EL | Class A | -34.00% | -27.80% | -26.10% | -90.30% | -89.50% | -90.20% |
| Only | Class B | -24.60% | -23.00% | -20.00% | -86.30% | -84.60% | -85.20% |
|  | SWG | -17.90% | -18.00% | -13.00% | -46.50% | -40.70% | -39.70% |
|  | AVG | -25.50% | -22.93% | -19.70% | -74.37% | -71.60% | -71.70% |

Table 6 comparison of BD rate performance between HM8.1 simulcast vs. GO-NA

|  |  | AI_HEVC_2x |  |  | AI_HEVC_CGS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | Class A | -24.20% | -29.40% | -29.00% | -47.10% | -46.80% | -49.80% |
|  | Class B | -16.60% | -21.70% | -19.00% | -45.40% | -44.90% | -46.50% |
|  | SWG | -15.60% | -20.20% | -18.80% | -29.80% | -29.20% | -29.30% |
|  | AVG | -18.80% | -23.77% | -22.27% | -40.77% | -40.30% | -41.87% |
| EL | Class A | -42.80% | -47.10% | -46.60% | -94.10% | -94.00% | -94.40% |
| Only | Class B | -28.60% | -33.10% | -30.50% | -91.10% | -90.80% | -91.00% |
|  | SWG | -28.60% | -32.40% | -31.20% | -60.30% | -59.50% | -59.40% |
|  | AVG | -33.33% | -37.53% | -36.10% | -81.83% | -81.43% | -81.60% |
|  |  | RA_HEVC_2x |  |  | RA_HEVC_CGS |  |  |
|  |  | Y | U | V | Y | U | V |
|  | Class A | -14.70% | -9.90% | -11.00% | -42.90% | -37.10% | -41.60% |
|  | Class B | -10.80% | -11.30% | -7.70% | -39.20% | -35.20% | -39.00% |
|  | SWG | -10.40% | -10.90% | -5.40% | -24.50% | -17.90% | -16.60% |
|  | AVG | -11.97% | -10.70% | -8.03% | -35.53% | -30.07% | -32.40% |
| EL | Class A | -29.10% | -25.30% | -25.90% | -85.60% | -84.50% | -85.40% |
| Only | Class B | -20.00% | -20.20% | -16.90% | -81.00% | -79.10% | -80.20% |
|  | SWG | -17.30% | -17.70% | -12.50% | -44.50% | -39.30% | -38.30% |
|  | AVG | -22.13% | -21.07% | -18.43% | -70.37% | -67.63% | -67.97% |

It is reported that the Bit Increment (BI) prediction model prediction method provides the average Y-BD bitrate changes over simulcast in enhancement layer (EL only) coding are observed to be AI_HEVC_2x: -31.63%, AI_HEVC_CGS: -75.90%, RA_HEVC_2x: -21.70%,  and RA_HEVC_CGS: -64.97%.

As shown in Table 5 the proposed GO-SA prediction method provides the average Y-BD bitrate changes over simulcast in EL are observed to be AI_HEVC_2x: -36.30%, AI_HEVC_CGS: -83.67%, RA_HEVC_2x: -25.50%,  and RA_HEVC_CGS: -74.37%.

It is also reported that the proposed GO-NA prediction method provides the average Y-BD bitrate changes over simulcast in EL are observed to be AI_HEVC_2x: -33.33%, AI_HEVC_CGS: -81.83%, RA_HEVC_2x: -22.13%,  and RA_HEVC_CGS: -70.37%.

The Bit Increment prediction model works well on the Y and Cb channels but suffers on Cr channel in some cases.  This agrees with the lower gain on the Cr channel observed in prior BT.709 and BT.2020 data analysis.  The fixed and sequence adaptive Gain Offset models give consistent performance better than the BI design. The performance varies by sequence.

In general the GO-SA and GO-NA have been shown to provide improved coding gain compared to the bit-depth difference based BI.  As an example, the RD curve for each prediction method has been compared in Figure 9 and Figure 10.

Figure 9 comparison of RD curves for “Traffic” (HM8.2 simulcast, BI, GO-SA, GO-NA)

Figure 10 comparison of RD curves for “PeopleOnStreet” (HM8.2 simulcast, BI, GO-SA, GO-NA)

A comparison of the EL RD performance between GO-NA and GO-SA shows the benefits of allowing adaptive prediction parameter based on each sequence. GO-SA provides about 3% of BD rate savings but it requires more computational complexity to calculate the prediction parameter. For the GO-NA case, the prediction parameter is fixed and doesn’t require any additional complexity to calculate the prediction parameter in the encoder side.  In addition, comparison of EL BD rate performance under ‘AVC base layer’ has been shown in Table 7. From the result, it is confirmed that the proposed color gamut prediction scheme constantly provides significant coding performance in the AVC base layer coding. More detailed results regarding AVC base layer coding can be found in the attached spreadsheet.

Table 7 comparison of EL BD rate performance over simulcast (AVC base layer)

|  |  | RA HEVC 2x |  |  | RA HEVC CGS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
|  | BI | -26.0% | -20.7% | -12.4% | -73.3% | -71.7% | -63.7% |
| Class A | GO-NA | -23.7% | -19.5% | -18.8% | -78.0% | -76.4% | -77.1% |
|  | GO-SA | -29.0% | -21.7% | -23.9% | -82.0% | -80.6% | -81.6% |
|  | BI | -15.9% | -16.1% | -10.7% | -65.0% | -62.4% | -55.7% |
| Class B | GO-NA | -15.0% | -15.9% | -11.5% | -70.2% | -67.5% | -68.5% |
|  | GO-SA | -19.2% | -18.3% | -14.6% | -74.0% | -71.1% | -72.2% |

# Conclusions

A scalable system for support different color gamuts and bit-depths in different layers of a bit-stream is proposed.  The experiments focused on the case of a 8-bit BT.709 base layer, and 10-bit BT.2020  enhancement layer but this is not a restriction.  This addresses the issue of efficiently supporting a UHDTV and HD device in the same bit-stream.  Low complexity Gain Offset prediction models were examined and show benefit compared to prediction based solely on bit-depth differences.

We propose to include the color prediction tool including the ability to adaptively signal the predictor in the scalable HEVC design.

# References

- ITU-R Recommendation BT.709 “Parameter values for the HDTV standards for production and international programme exchange” Dec. 2010.

- ITU-R Recommendation BT.2020  “Parameter values for UHDTV systems for production and international programme exchange” April 2012.

- H. Schwarz, D. Marpe, and T. Wiegand, “Overview of the Scalable Video Coding extension of the H.264/AVC standard,” IEEE Trans. Circuits Syst. Video Technol., vol. 17, no. 9, pp. 1103–1120, Sept. 2007.

- B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8,” JCTVC-J1003_d7, Stockholm, July 2012.

- L. Kerofsky, A. Segall, S. Kim, K. Misra, “Color Gamut Scalable Video Coding”,JCTVC-K-0241, Shanghai, October 2012.

# Appendix A calculation of Jacobian of BT.709 to BT.2020 transformation

The definition of luma and chroma components is of similar form in Bt.709 and BT.2020 differing in specific matrix elements.

The linear RGB values in BT.709 and BT.2020 are related by a matrix T.  This matrix can be derived by pass through the intermediate XYZ space.

Figure A Relation between linear RGB709 and RGB2020

The nonlinearity used with 8-bit BT.709 and 10-bit BT.2020 is the same defined through a scalar function f described below.

Where the function f(x) is defined below

Figure B Nonlinearities

The composition of five basic functions gives the nonlinear conversion between BT.709 and BT.2020.  This can be expressed as:

Where the individual functions are described below:

Near a point Y0Cb0Cr0 in BT.709, this transformation can be approximated by using the Jacobian of the transformation as:

This is special case of a 3x3 gain matrix and an offset vector.  The gain matrix depends on position in the BT.709 space.

Where

To study the variation of this transform we compute the Jacobian of the entire transform from the individual components.

The Jacobian of the transformation is the product of the Jacobians of individual components where each component is evaluated at the appropriate point.  The individual Jacobians are straightforward to compute.  Since some functions F1, F3, and F5 are linear their Jacobians are constant given by their linear operator.  The functions F2 and F4 have diagonal Jacobians which are not constant but vary depending on the location in RGB where they are evaluted.  The result is:

Where the derivatives of the functions f and f-1 are given by

The full Jacobian can be computed by matrix multiplication of the components.

# Patent rights declaration

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).