| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0335 |
| --- | --- |

| Title: | TE-A1: Results of test 4.3 on fixed up-sampling filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Rapaka Krishna Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8028 cjianle@qti.qualcomm.com krapaka@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the test results of Tool Experiment A1 4.3 on fixed up-sampling filter for SHVC. The filter coefficients for chroma component are designed with 6 bits accuracy which is aligned to HEVC motion compensation interpolation filter design. Compared to the simulcast anchor, BD-rate reductions (based on BL+EL rate) of the tested method are -22.8% (AI 2x), -32.6% (AI 1.5x), -16.4% (RA 2x), -26.1% (RA 1.5x), -12.3% (LP 2x), -21.9% (LP 1.5x), respectively.

# Technical description

This test studied performance of the fixed up-sampling filter for SHVC [1]. The solution applies 2D separable 8-tap filter for luma component and 2D separable 4-tap filter for chroma component. The tested filter coefficients are shown in in the following tables. It should be noted that the chroma filter coefficients is designed with 6 bits accuracy which is aligned to HEVC motion compensation interpolation filter.

Tab.1 Luma up-sampling filters.

| Phase shift | Luma up-sampling filter |
| --- | --- |
| 0 | { 0, 0, 0, 64, 0, 0, 0, 0}/64 |
| 1/3 | {-1, 4, -11, 52, 26, -8, 3, -1}/64 |
| 1/2 | {-1, 4, -11, 40, 40, -11, 4, -1}/64 |
| 2/3 | {-1, 3, -8, 26, 52, -11, 4, -1}/64 |

Tab.2 Chroma up-sampling filters.

| Phase shift | Chroma up-sampling filter |
| --- | --- |
| 0 | { 0, 64, 0, 0}/64 |
| 1/4 | {-4, 54, 16, -2}/64 |
| 1/3 | {-6, 52, 20, -2}/64 |
| 3/8 | {-6, 46, 28, -4}/64 |
| 1/2 | {-4, 36, 36, -4}/64 |
| 7/12 | {-4, 30, 42, -4}/64 |
| 2/3 | {-2, 20, 52, -6}/64 |
| 11/12 | { 0, 4, 62, -2}/64 |

# Test results

The tables below summarize performance of the tested method for spatial scalability cases. Full results are provided in the attached Excel sheets.

Tab.3 BD-Rate Saving of TE-A1 4.3 against simulcast anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.0% | -27.2% | -28.0% |  |  |  |
| Class B | -21.1% | -21.2% | -21.0% | -32.6% | -33.0% | -33.0% |
| Overall (EL+BL) | -22.8% | -22.9% | -23.0% | -32.6% | -33.0% | -33.0% |
| Overall (EL) | -34.8% | -35.2% | -35.3% | -58.0% | -58.1% | -58.2% |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -18.5% | -7.8% | -9.3% |  |  |  |
| Class B | -15.5% | -9.2% | -7.9% | -26.1% | -20.3% | -18.7% |
| Overall (EL+BL) | -16.4% | -8.8% | -8.3% | -26.1% | -20.3% | -18.7% |
| Overall (EL) | -25.2% | -13.6% | -12.9% | -47.2% | -37.9% | -35.2% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -13.6% | -1.9% | -3.0% |  |  |  |
| Class B | -11.7% | -6.9% | -5.1% | -21.9% | -17.1% | -15.2% |
| Overall (EL+BL) | -12.3% | -5.5% | -4.5% | -21.9% | -17.1% | -15.2% |
| Overall (EL) | -19.6% | -8.6% | -7.2% | -40.6% | -32.3% | -29.0% |

# References

- A. Segall, J. Chen, J. Dong, E. Alshina, “Description of Tool Experiment A1: Upsampling filter for SHVC”, document JCTVC-K1101, 11th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Shanghai, China, 10-19 Oct., 2012

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).