| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0336 |
| --- | --- |

| Title: | Non-TE5: on motion mapping in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Vadim Seregin Liwei Guo Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8028 cjianle@qti.qualcomm.com vseregin@qti.qualcomm.com liweig@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

A motion mapping scheme is tested in TE5.4.1 to investigate the gain of inter-layer motion prediction for the potential high level syntax only SHVC framework. This contribution presents an alternative motion mapping scheme that is asserted to be simpler than the tested method in TE5.

# Introduction

In the potential high level syntax only SHVC framework, inter-layer motion parameters prediction can be enabled by set the inter-layer reference picture as the collocated picture for TMVP derivation. In case of spatial scalability case, motion mapping process between two layers is necessary to be performed to avoid any low level modification in TMVP derivation process. The concept of motion mapping is being tested as test 5.4.1 in TE5 [1]. In TE5.4.1 method, motion parameter of each 4x4 block is derived by using the motion information from up to four neighbor blocks.

# Tested method

A simplified motion mapping method is tested in this contribution. In this method, the motion parameters (MV and ref_idx) and prediction mode for each 16x16 block of up-sampled inter-layer reference picture is copied from its collocated base layer block. The collocated block is located based on the center position of the current block. The 16x16 block size is chosen to be compliant with HEVC motion compression concept, since the inter-layer picture will be placed into the reference list. The picture level information which is used in TMVP derivation process is also copied from the base layer picture to avoid low level modification.

# Experimental results

Simulation result of the proposed method are obtained by using the same test condition which is specified for TE5.4.1 [1] and summarized in the following table. The result shows that the tested method has almost same performance compared to the TE5.4.1 method.

Tab.1 Results of proposed algorithm compared to not using inter-layer motion prediction

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.7% | -2.4% | -2.6% |  |  |  | -1.6% | -3.9% | -4.5% |
| Class B | -1.2% | -2.7% | -3.1% | -1.7% | -3.0% | -3.4% | -2.1% | -4.0% | -4.5% |
| Overall (EL+BL) | -1.1% | -2.6% | -3.0% | -1.7% | -3.0% | -3.4% | -2.0% | -4.0% | -4.5% |
| Overall (EL) | -2.1% | -4.7% | -5.3% | -4.4% | -7.1% | -8.2% | -3.8% | -7.2% | -8.1% |
| Enc Time[%] | 91.0% |  |  | 86.7% |  |  | 89.6% |  |  |
| Dec Time[%] | 88.3% |  |  | 84.8% |  |  | 89.5% |  |  |
| Enc Mem[%] | 107.3% |  |  | 106.4% |  |  | 105.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.6% | -0.6% |  |  |  | -1.0% | -1.6% | -1.7% |
| Class B | -0.2% | -0.5% | -0.5% | -0.4% | -0.6% | -0.7% | -0.7% | -1.1% | -1.2% |
| Overall (EL+BL) | -0.2% | -0.6% | -0.5% | -0.4% | -0.6% | -0.7% | -0.8% | -1.2% | -1.4% |
| Overall (EL) | -0.4% | -1.0% | -1.0% | -0.9% | -1.2% | -1.6% | -1.3% | -2.0% | -2.2% |
| Enc Time[%] | 86.9% |  |  | 82.1% |  |  | 87.0% |  |  |
| Dec Time[%] | 87.6% |  |  | 83.0% |  |  | 87.7% |  |  |
| Enc Mem[%] | 108.8% |  |  | 107.7% |  |  | 106.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Reference

[1] V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, and H. Laksman, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Shanghai, China, Oct. 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).