| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0337 |
| --- | --- |

| Title: | SHVC: Cross-verification of restriction on inter-layer texture prediction at high temporal hierarchy (JCTVC-L0278) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

JCTVC-L0278 studies the inter-layer pixel prediction at high temporal hierarchy in order to show the necessity of the followings.

-	Inter-layer pixel and syntax prediction be able to be set independently.

-	Inter-layer pixel prediction be set in terms of temporal hierarchy.

This contribution reports the cross-verification results of the proposal.

The BD-rate results tested by the cross-checker perfectly match those provided by the proponent.

# Introduction

JCTVC-L0278 [1] studies the inter-layer pixel prediction at high temporal hierarchy in order to show the necessity of the followings.

-	Inter-layer pixel and syntax prediction be able to be set independently.

-	Inter-layer pixel prediction be set in terms of temporal hierarchy.

This contribution reports the cross-verification results of the proposal.

# Experiments

## Tests and software

Following configurations are to be tested.

- Anchor: 
SMuC 0.1.1 (revision 7 of the common software with SHVC support) is used.
Intra BL mode is always enabled in all the pictures.

- Tested: JCTVC-L0278 Test B: Intra BL disabled at highest temporal hierarchy
Four configurations Test A, Test B, Test C and Test D are tested in JCTVC-L0278 [1]. This contribution only reports the cross-verification results of Test B.
In Test B, Intra BL is disabled at the highest temporal hierarchy on SMuC 0.1.1.
In the software:

  - Intra BL mode is enabled in each picture POC of which is even number (0, 2, 4, ...) and Intra BL mode is disabled in each picture POC of which is odd number (1, 3, 5, ...) under Random Access condition.

  - High-level syntax is not changed.

## Test conditions

The common test conditions for SHVC are used [2][3].

RA 2x/1.5x configuration are tested.

MD5 hash of each decoding picture is computed.

## Experimental environment

The experimental environment is presented in Table 1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2 presents the simulation results for Test B.

Table 2 Simulation results for Test B

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.4% | 0.6% | 1.1% |  |  |  |
| Class B | 0.8% | 0.2% | 0.0% | 2.1% | 1.3% | 1.0% |
| Overall (EL+BL) | 1.0% | 0.3% | 0.3% | 2.1% | 1.3% | 1.0% |
| Overall (EL) | 2.3% | 0.8% | 0.8% | 7.7% | 4.6% | 3.8% |
| Enc Time[%] | 96.9% |  |  | 96.3% |  |  |
| Dec Time[%] | 98.5% |  |  | 97.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusions

The BD-rate results　of JCTVC-L0278 Test B tested by the cross-checker perfectly match those provided by the proponents.

It is recommended that the issues be studied further under AHG.

# References

[1] K. Sato, “SHVC: On Inter-layer Prediction”, JCTVC-L0278, Geneva, January 2013

[2] “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC”, JCTVC-K1102, October 2012

[3] “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Shanghai, October 2012

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).