| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0339 |
| --- | --- |

| Title: | TE-B4: Cross-verification of test 4.3.2 on Inter-layer SAO (JCTVC-L0085) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document of JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jianle Chen 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8028 cjianle@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document is a cross-check report on TE-B4 test 4.1.1 from interDigital on selectable upsampling filter (JCTVC-L0056). It is verified that the encoding/decoding results completely match those reported by the proponent, and the description also matches the implementation in the software.

# Technical description

In JCTVC-L0056 [1], up-sampling filter can be switched among a set of filters which are hard-coded in the system. In addition to the existing 8-tap filter, 2D non-separable up-sampling filters are employed by the tested methods. The filter selection is indicated by an index signalled in slice head. The main features of this test are summarized below.

- Switch between 2D non-separable 7x7/6x7/7x6/6x6 (depending on phase) up-sampling filter and the existing 8-tap filter

- 4 sets of additional non-separable 2D filters

- Filter coefficient has 8 bits accuracy

# Test results

Three experiments are were in this test. They are

- Proposed method with asymmetric filter coefficients tested with SMuC 0.1.1

- Proposed method with symmetric filter coefficients tested with SMuC 0.1.1

- Proposed method with asymmetric filter coefficients tested with SMuC 0.1.1 + refIdx framework

It is verified that BD-rate saving of all three tests is completely match those reported by the proponent.

The tables below summarizes the performance of the three test. Full results are provided in the attached Excel sheets.

Tab.1 BD-Rate Saving of TE-B4 4.1.1 with asymmetric coefficients against SHVC v0.1.1

Tab.2 BD-Rate Saving of TE-B4 4.1.1 with symmetric coefficients against SHVC v0.1.1

Tab.3 BD-Rate Saving of TE-B4 4.1.1 with asymmetric coefficients against SHVC v0.1.1 with refIdx framework

# References

- J. Dong, Y. He, Y. Ye, “TE4: Results of test 4.1.1 on selectable upsampling filter (SUF)”, doc. JCTVC- L0056, 12th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, 14–23 Jan. 2013