| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0341 |
| --- | --- |

| Title: | TE2-3.1.9: Crosscheck of inter-layer intra prediction signaling in JCTVC-L0110 proposed by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the inter-layer intra prediction signalling proposed by Samsung in JCTVC-L0110. In this contribution, an intra_BL_skip_flag is signalled after skip_flag. When this flag is equal to 1, the inter-layer intra prediction is used for current CU and the residual coding is skipped. An intra_BL_flag is signalled after the intra_BL_skip_flag to indicate whether the inter-layer intra prediction is used nor not. If the intra_BL_flag is equal to 1, the residual coding is not skipped.  The residual coding function for inter-layer intra prediction reuses the inter-coded block residual coding function. The verification task was done successfully, and the results match exactly those provided by Samsung.

# Verification

Samsung provided the SMuC source code of their proposed codec described in JCTVC-L0110 [1]. One test is evaluated. The verification is carried out by setting the following parameters in the provided code:

#define INTRA_BL                         1      ///< inter-layer texture prediction
#if INTRA_BL
#define TE2_INTRA_BL                     1
#if TE2_INTRA_BL
#define TE2_INTRA_BL_SKIP                1
#endif
#endif

Six configurations (AI-2x, AI-1.5x, RA-2x, RA-1.5x, LDP-2x, LDP-1.5x) were run for the proposed inter-layer intra prediction signalling with the provided code.

# Results

The BD-rates from our experimental results match exactly those provided by Samsung. The anchor is HM-8.1 simulcast. The detailed results are listed in JCTVC-L0341.xls.

# Conclusion

The results presented by Samsung in JCTVC-L0110 are confirmed.

# References

- T. Lee, E. Alshina, “TE2 Subtest 1: Inter-layer texture prediction signalling (Test 3.1.9),” Document of Joint Collaborative Team on Video Coding, JCTVC-L0110, January, 2013.