| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0344 |
| --- | --- |

| Title: | TE5-5.2.6: Crosscheck of inter-layer motion vector prediction in JCTVC-L0101 proposed by Canon |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the inter-layer motion vector prediction proposed by Canon in JCTVC-L0101. In JCTVC-L0101, the temporal collocated candidate is put in the 1st position in the merging candidate list. The motion vector of the bottom-right (H-position) neighbouring block of the corresponding block in base layer is put in the last position in the merging candidate list. The base motion vector is followed by 4 offset candidates derived according to this base motion vector candidate. The generation of these candidates is adding one offset (offset value: 4 or -4) alternatively to the horizontal or vertical component of the motion vector from list 0 and its inverse value to components of motion vector from list 1. The duplicate check is applied. The total merging candidate number is changed to 6. In addition, the combined results with TE5-5.3.3 are also evaluated. The verification task was done successfully ,and the results match exactly those provided by Canon.

# Verification

Canon provided the SMuC source code of their proposed codec described in JCTVC-L0101 (TE5-5.2.6) [1] and in JCTVC-L0102 (TE5-5.3.3) [2]. Two tests are evaluated. The verifications are carried out by setting the following parameters in the provided code:

Test 1: TE5-5.2.6

#define CANON_MEMORY_FIXES             1
#define CANON_BUGFIX_DBF_LEAK          1
#define CANON_BASE_MODE                 0
#if CANON_BASE_MODE
#define CANON_BASE_MODE_DCT_FIX       0
#endif
#define CANON_MERGE                      1

Test 2: TE5-5.2.6 with TE5-5.3.3

#define CANON_MEMORY_FIXES             1
#define CANON_BUGFIX_DBF_LEAK          1
#define CANON_BASE_MODE                 1
#if CANON_BASE_MODE
#define CANON_BASE_MODE_DCT_FIX       0
#endif
#define CANON_MERGE                      1

Six configurations (RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR) were run for the proposed inter-layer motion vector prediction with the provided code.

# Results

The BD-rates from our experimental results match exactly those provided by Canon. The anchor is SMuC-0.1.1. The detailed results are listed in JCTVC-L0344_TE5-5.2.6.xls, and JCTVC-L0344_TE5-5.2.6_TE5-5.3.3.xls.

# Conclusion

The results presented by Canon in JCTVC-L0101 are confirmed.

# References

- G. Laroche, T. Poirier, P. Onno, C. Gisquet, E. François, “TE5: Results of Test 5.2.6 on the inter-layer motion vector prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0101, January 2013.

- C. Gisquet, T. Poirier, J. Taquet, F. Le Léannec, E. François, G. Laroche, P. Onno, “TE5: Results of Test 5.3.3 on the inter-layer prediction mode (base mode),” Document of Joint Collaborative Team on Video Coding, JCTVC-L0102, Januray, 2013.