| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0345 |
| --- | --- |

| Title: | Crosscheck of constrained intra prediction at enhancement layer in JCTVC-L0134 proposed by LG |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the constrained intra prediction modification proposed by LG. In SMuC-0.1.1 enhancement layer coding, when constrained_intra_pred_flag is set to 1, only the intra prediction coded blocks can be used for the intra prediction of neighbouring blocks. The inter-layer intra prediction coded blocks cannot be used. In the proposed method, the inter-layer intra prediction coded blocks can be used for the intra prediction of neighbouring blocks in the enhancement layer if the corresponding block in BL is in I_slice. The verification task was done successfully and the results matched exactly those provided by LG.

# Verification

LG provided the SMuC source code of their proposed codec described in JCTVC-L0134 [1]. One SMuC-0.1.1 anchor with setting constrained_intra_pred_flag to 1 and one test with proposed method are evaluated. The verifications are carried out by setting the following parameters in the provided code:

Anchor: CIP_on

#define CONSTRAINED_INTRA_ON             1
#define CU_BASED_CONSTRAINED_INTRA      0

Test: CIP_on with the proposed method

#define CONSTRAINED_INTRA_ON             1
#define CU_BASED_CONSTRAINED_INTRA      1

Eight configurations (AI-2x, AI-1.5x, RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR) were run for the proposed inter-layer motion vector prediction with the provided code.

# Results

The BD-rates from our experimental results match exactly those provided by LG. The anchor is CIP_on, the test is CIP_on with the proposed method. The detailed results are listed in JCTVC-L0345.xls.

# Conclusion

The results presented by LG in JCTVC-L0134 are confirmed.

# References

- C. Kim, J. Kim, B. Jeon, “Non-TE 2: Constrained intra prediction at enhancement layer,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0134, Jan. 2013.