| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0345 |
| --- | --- |

| Title: | Crosscheck of constrained intra prediction at enhancement layer in JCTVC-L0134 proposed by LG |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the constrained intra prediction modification proposed by LG in JCTVC-L0134. In SMuC-0.1.1 enhancement layer coding, when constrained_intra_pred_flag is set to 1, only the intra prediction coded blocks can be used for the intra prediction of neighbouring blocks. The inter-layer intra prediction coded blocks cannot be used. In the proposed method, the inter-layer intra prediction coded blocks can be used for the intra prediction of neighbouring blocks in the enhancement layer. The verification task was done successfully, and the results match exactly those provided by LG.

# Verification

LG provided the SMuC source code of their proposed codec described in JCTVC-L0134 [1]. One SMuC-0.1.1 anchor with setting constrained_intra_pred_flag to 1 and one test with proposed method are evaluated. The verifications are carried out by setting the following parameters in the provided code:

Anchor: CIP_on

#define CONSTRAINED_INTRA_ON             1
#define CU_BASED_CONSTRAINED_INTRA      0

Test: CIP_on with the proposed method

#define CONSTRAINED_INTRA_ON             1
#define CU_BASED_CONSTRAINED_INTRA      1

Eight configurations (AI-2x, AI-1.5x, RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x, and LDP-SNR) were run for the proposed inter-layer motion vector prediction with the provided code.

# Results

The BD-rates from our experimental results match exactly those provided by LG. The anchor is CIP_on, the test is CIP_on with the proposed method. The detailed results are listed in JCTVC-L0345.xls.

# Conclusion

The results presented by LG in JCTVC-L0134 are confirmed.

# References

- C. Kim, J. Kim, B. Jeon, “Non-TE 2: Constrained intra prediction at enhancement layer,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0134, January, 2013.