| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0347 |
| --- | --- |

| Title: | TEB4: Cross-check of inter-layer de-blocking filter performance (test 4.5.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_a.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This is verification of TEB4 from LG. The performance of Inter layer filtering of boundary between EL and BL is verified in this report. In average 0.1% (Luma) BD-rate gain vs SMuC 0.1.1 is confirmed.

# Introduction

Verified tool was the part of CfP response from MediaTek and LG and included into TEB4 test set.

Only one next to boundary sample (q0) is filtered and only one sample from neighbour block (p0) participates in the smoothing

The decision about boundary smoothing is similar to de-blocking filter decision, except special setting for ß_offset_div2 parameter. Actually decision for boundary smoothing in verified test requires more calculations than filtering itself.

Verified tool operates similarly to DC-boundary Intra smoothing in HEVC specification. Application of DC-boundary Intra smoothing was restricted (during Shanghai meeting) and applied for small-size blocks only. Restriction of DC-boundary Intra smoothing was motivated (JCTVC-K0186) by difficulties it introduces in strong in-loop de-blocking filter application decision. It was claimed that restriction of DC-boundary Intra smoothing has positive effect for visual quality. Compatibility with in-loop de-blocking filter on EL should be carefully studied for verified tool as well.

# Test results for verified algorithm.

S/w provided by proponent was tested according to TEB4 description: All Intra (x2 and x1.5), Random Access (x2, x1.5 and SNR) and Low-Delay P (x2 , x1.5 and SNR) tests were performed. Simulation results vs SMuC 0.1.1 are summarised in Table 1 below.

Table 1. Summation test results Inter layer filtering of boundary between EL and BL predictions

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.1% | -0.1% | -0.1% |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (EL+BL) | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (EL) | -0.2% | -0.2% | -0.2% | 0.0% | 0.1% | 0.2% |  |  |  |
| Enc Time[%] | 100.9% |  |  | 101.0% |  |  |  |  |  |
| Dec Time[%] | 100.8% |  |  | 100.6% |  |  |  |  |  |
| Enc Mem[%] | 99.8% |  |  | 100.0% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | 0.0% |  |  |  | -0.2% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Overall (EL+BL) | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Overall (EL) | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100.7% |  |  | 100.5% |  |  | 100.5% |  |  |
| Dec Time[%] | 100.7% |  |  | 101.4% |  |  | 102.3% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% |  |  |  | -0.1% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% |
| Overall (EL+BL) | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% |
| Overall (EL) | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 0.3% | -0.2% | 0.1% | 0.1% |
| Enc Time[%] | 100.6% |  |  | 100.5% |  |  | 100.6% |  |  |
| Dec Time[%] | 101.6% |  |  | 101.9% |  |  | 102.2% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -0.1% |  |  |  |  |  | -0.1% |  |  |

Performance and run time reported in first release of this cross-check was not adequate. Up-dated version is accurate. Average Encoding/Decoding time increment is 1% in our test.

# Conclusion

The performance of Inter layer boundary filtering is verified in this report. In average 0.1% Luma BD-rate gain vs SMuC 0.1.1 is confirmed. No performance degradation was found for Chroma.