| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0351_r3 |
| --- | --- |

| Title: | Non-CE1: Square transform blocks for 4:2:2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA Andrea Gabriellini, Marta Mrak 56 Wood Lane, W12 7SB London, UK | Tel: Email: Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com +44-303040-9641 marta.mrak@bbc.co.uk |
| Source: | Qualcomm, BBC |  |  |

_____________________________

# Abstract

This proposal describes the usage of square transform blocks for the chroma components in the HM Range Extension 4:2:2 model. Following the HEVC design, the upper square transform is reconstructed prior to the intra prediction of the lower square transform. The proposed method averages BD-rate losses from 0.0% to 0.8% for luma and from 1.9% to 5.6% for chroma components depending on the configuration and the QP range. A second test is also performed where the angular intra prediction for chroma components is performed with the same angle adjustement as in HM Range Extension 4:2:2; in this second test it is reported that average BD-rate losses of between 0.0% and 0.2% for the luma component and average BD-rate losses between 1.5% and 5.0% for the chroma components.

# Technical description

HEVC Range Extensions Core Experiment 1 [1] tests the usage of two square transforms instead of one rectangular transform for the 4:2:2 chroma components. Test 3 compares the performance of square transforms for all chroma block sizes against the anchor that is using rectangular transforms.

This proposal is constructed on the top of JCTVC-L0333 [2]. The basic difference is that the upper square transform block is reconstructed prior to the intra prediction of the lower transform in a Nx2N chroma block. This follows the HEVC intra prediction structure. Also, it allows the usage of closer samples for the prediction of the lower square block. Other characteristics of the method are as follows:

- Two square transforms instead of a rectangular one for chroma components

  - Signaling of two cbf and invoking residual coding twice

  - Transform skip enabled: one flag per 4x4 transform

  - No need for the 32-point transform in the chroma path

- Removed the QP increase by 3 associated to the rectangular transforms

- Removed the scaling lists associated to the rectangular transforms

- Removed the intra prediction modification for planar modes specific for rectangular blocks.

- Removed the intra prediction modification for angular modes specific for rectangular blocks

A second test evaluated the effect of the angle adjustment performed in HM Range Extension 4:2:2 for angular intra prediction for chroma components. Since the use of square blocks does not allow the full range of angle adjustments available for rectangular chroma blocks a limit was introduced to the maximum absolute value of intraPredAngle.When the absolute value of intraPredAngle exceeds 32 it is clipped to 32 (or -32, depending on the sign of the original value). When such clipping is required, invAngle is also modified and set to 256.

# Simulation results

The method is implemented on top of the HM9.0.1_RExt software. Simulations are carried under the common test conditions for range extensions [3]. The performance is compared to the anchor in terms of BD-rate savings. Table 1 shows the performance of proposed modifications.

Table 1 shows the rsults obtained with the first test, where average BD-rate losses of 0.0% to 0.8% are recorded for the luma component and average BD-rate losses of 1.9% to 5.6% for the chroma components.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 1.0% | 2.2% | 7.3% | 0.7% | 1.0% | 4.6% |
| Kimono1_1920x1080_24_10bit_422.yuv | 1.0% | 4.6% | 5.7% | 0.8% | 2.5% | 3.4% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.8% | 3.9% | 6.7% | 0.7% | 2.4% | 3.0% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 1.2% | 2.9% | 3.9% | 0.7% | 1.8% | 1.6% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.7% | 1.9% | 2.1% | 0.4% | 1.7% | 0.4% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.3% | 4.8% | 4.3% | 0.2% | 1.1% | 0.6% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.8% | 4.7% | 5.1% | 0.5% | 2.9% | 2.6% |
| Overall | 0.8% | 3.6% | 5.0% | 0.6% | 1.9% | 2.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 92% |  |  | 98% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.7% | 2.3% | 6.4% | 0.3% | 0.6% | 4.2% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.4% | 5.5% | 4.7% | 0.4% | 2.8% | 4.5% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.6% | 6.8% | 8.1% | 0.2% | 3.3% | 6.3% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.5% | 2.3% | 3.2% | 0.3% | 1.6% | 1.7% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.1% | 4.1% | 5.9% | 0.2% | 1.8% | 2.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.2% | 4.7% | 4.7% | 0.0% | 3.0% | 2.5% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.6% | 5.5% | 6.4% | 0.3% | 3.9% | 4.2% |
| Overall | 0.4% | 4.5% | 5.6% | 0.3% | 2.4% | 3.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.2% | 1.9% | 5.6% | 0.0% | 0.6% | 3.9% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | 4.7% | 4.0% | 0.1% | 2.4% | 4.0% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.1% | 6.8% | 8.6% | -0.3% | 4.1% | 7.8% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.1% | 2.2% | 2.9% | 0.1% | 1.7% | 1.6% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.1% | 3.2% | 4.5% | 0.2% | 1.3% | 1.2% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.1% | 3.8% | 3.0% | 0.0% | 2.4% | 1.6% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.4% | 5.5% | 6.3% | 0.1% | 3.8% | 4.0% |
| Overall | 0.2% | 4.0% | 5.0% | 0.0% | 2.3% | 3.4% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 1. BD-rates for common conditions for Test 1

Table 2 shows the results for test 2, where the angular intra prediction for chroma components has been modified as described above. The reported average BD-rate lossed are smaller in this case, from 0.0% to 0.2% for the luma component and from 1.5% to 5.0% for the chroma components. The run times are substantially the same for both tests and the reference (HM Range Extension 9.0.1).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.4% | 1.7% | 6.6% | 0.2% | 0.6% | 4.1% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.2% | 3.7% | 4.7% | 0.2% | 2.1% | 2.8% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.3% | 3.4% | 6.1% | 0.3% | 2.1% | 2.6% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.1% | 1.6% | 2.6% | -0.2% | 0.9% | 0.7% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.2% | 1.5% | 1.7% | 0.0% | 1.4% | 0.1% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.1% | 4.3% | 3.8% | 0.0% | 0.9% | 0.4% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.3% | 4.2% | 4.7% | 0.2% | 2.6% | 2.3% |
| Overall | 0.2% | 2.9% | 4.3% | 0.1% | 1.5% | 1.9% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.5% | 1.8% | 5.6% | 0.2% | 0.3% | 3.8% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.1% | 4.8% | 4.1% | 0.2% | 2.6% | 4.2% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.3% | 6.2% | 7.5% | 0.0% | 3.1% | 5.9% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.1% | 1.6% | 2.3% | -0.1% | 1.1% | 1.1% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.0% | 3.8% | 5.8% | 0.0% | 1.6% | 1.9% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.1% | 4.1% | 4.2% | 0.0% | 2.9% | 2.3% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.2% | 4.7% | 5.8% | 0.1% | 3.5% | 3.8% |
| Overall | 0.2% | 3.8% | 5.0% | 0.0% | 2.1% | 3.3% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.1% | 1.8% | 5.6% | -0.1% | 0.5% | 3.9% |
| Kimono1_1920x1080_24_10bit_422.yuv | -0.1% | 4.2% | 3.8% | 0.0% | 2.3% | 3.8% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.0% | 6.3% | 8.1% | -0.3% | 3.9% | 7.3% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.0% | 1.8% | 2.6% | 0.0% | 1.5% | 1.4% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | -0.1% | 3.0% | 4.3% | 0.0% | 1.2% | 1.0% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.1% | 2.8% | 2.2% | 0.0% | 2.3% | 1.6% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.2% | 4.9% | 5.8% | 0.0% | 3.6% | 3.7% |
| Overall | 0.0% | 3.5% | 4.6% | -0.1% | 2.2% | 3.2% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |

Table 1. BD-rates for common conditions for Test 2

# Conclusions

A method using square transforms for 4:2:2 chroma components is presented. On one side, the proposed method shows some performance penalty. On the other, it removes the non-square transforms (and corresponding quantization and scaling processes), the introduction of a 32-point transform in the chroma channel and the modification of the intra prediction process for 4:2:2.

# Draft text changes

JCTVC-L0148 [4] provides WD changes required to support square chroma blocks and also covers intra prediction on square blocks. Additional clipping required to enable intra prediction on square blocks is described in the following highlighted amendment to WD changes from JCTVC-L0148 [4]:

8.4.4.2.6	Specification of Intra_Angular (2..34) prediction mode

...

The variables intraPredAngleAdj and invAngleAdj are derived as follows:

–	When ChromaArrayType is equal to 2 and cIdx is not equal to 0, the following applies:

- if intraPredMode is greater than 17, intraPredAngleAdj is set equal to (intraPredAngle>>1) and invAngle is set equal to (intraPredAngle<<1),

- Otherwise (intraPredMode is less than 18), intraPredAngleAdj is set equal to (intraPredAngle<<1) and invAngle is set equal to (intraPredAngle>>1).

If Abs(intraPredAngle) is greater than 32,  intraPredAngle is set equal to (Sign(intraPredAngle)*32) and invAngle is set equal to 256.

–	Otherwise (ChromaArrayType is not equal to 2 or cIdx is equal to 0), intraPredAngleAdj is set equal to intraPredAngle and invAngleAdj is set equal to invAngle.

The variable nNeighNegIdx is set equal to nNeighY-TUHeight.

# References

- J. Sole, “HEVC Range Extensions Core Experiment 1: Square and non-square transforms for 4:2:2 chroma format coding”, Joint Collaborative Team on Video Coding, JCTVC-K1121, October 2012.

- J. Sole and M. Karczewicz, “CE1: Test 3 - Square transform blocks for 4:2:2”, Joint Collaborative Team on Video Coding, JCTVC-L0333, January 2013.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions” Joint Collaborative Team on Video Coding, JCTVC-K1006, Shanghai, October 2012.

- C. Rosewarne, V. Kolesnikov and M. Maeda, “CE1: Test 3 - Square transforms for Range Extensions”, Joint Collaborative Team on Video Coding, JCTVC-L0148, January 2013.

# Patent rights declaration

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).