| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0353 |
| --- | --- |

| Title: | TE2: Crosscheck for test 3.2.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1479 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document provides the cross-check results for TE2 test 3.2.3 on inter-layer texture prediction signaling with reference index framework. Source codes and test results provided by the proponents have been verified. The cross-check results exactly match to the results provided by the proponents.

# Introduction

JCTVC-L0251 [1] uses reference index framework for inter-layer texture prediction signaling. It is implemented on top of SMuC-0.1.1 with RED_IDX_FRAMEWORK enabled. It proposes to infer motion vector to be zero when referencing the up-sampled base-layer reference picture.

# Experimental Results

The proposed method is compared with HM8.1 Simulcast for crosscheck. It was confirmed that the cross-check results exactly match to the results provided by the proponents.

Table 1. BD-rate results by JCTVC-L0251 compared to HM-8.1 Simulcast

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -27.1% | -26.5% | -27.3% |  |  |  |  |  |  |
| Class B | -21.3% | -20.4% | -20.2% | -32.9% | -33.0% | -33.2% |  |  |  |
| Overall (EL+BL) | -23.0% | -22.1% | -22.2% | -32.9% | -33.0% | -33.2% |  |  |  |
| Overall (EL) | -35.1% | -34.0% | -34.3% | -58.8% | -58.5% | -58.9% |  |  |  |
| Enc Time[%] | 234.3% |  |  | 179.5% |  |  |  |  |  |
| Dec Time[%] | 100.0% |  |  | 94.7% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -17.6% | -5.4% | -6.8% |  |  |  | -20.5% | -9.2% | -8.1% |
| Class B | -14.4% | -5.8% | -3.9% | -25.6% | -18.9% | -17.0% | -17.9% | -6.2% | -1.7% |
| Overall (EL+BL) | -15.3% | -5.7% | -4.8% | -25.6% | -18.9% | -17.0% | -18.6% | -7.0% | -3.5% |
| Overall (EL) | -23.7% | -9.0% | -7.8% | -46.6% | -36.0% | -32.8% | -29.5% | -12.5% | -7.4% |
| Enc Time[%] | 111.5% |  |  | 107.6% |  |  | 113.0% |  |  |
| Dec Time[%] | 138.8% |  |  | 129.4% |  |  | 105.2% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.7% | -1.5% | -2.7% |  |  |  | -14.0% | -6.7% | -5.7% |
| Class B | -12.0% | -5.7% | -3.9% | -22.4% | -16.9% | -15.1% | -13.0% | -5.6% | -1.6% |
| Overall (EL+BL) | -12.5% | -4.5% | -3.5% | -22.4% | -16.9% | -15.1% | -13.3% | -5.9% | -2.8% |
| Overall (EL) | -19.9% | -7.1% | -5.8% | -41.7% | -32.3% | -29.0% | -21.4% | -11.2% | -6.8% |
| Enc Time[%] | 109.6% |  |  | 105.2% |  |  | 108.2% |  |  |
| Dec Time[%] | 155.1% |  |  | 147.5% |  |  | 111.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Reference

[1] K. Misra, J. Zhao and A. Segall, “TE2-3.2.3 Infer Zero motion vector when referencing co-located base layer picture,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0251, Jan. 2013.