| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0354 |
| --- | --- |

| Title: | TE3: Crosscheck for test 4.3.1 on adaptive predictor compensation by using reconstructed BL texture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1479 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document provides the cross-check results for TE3 test 4.3.1 on adaptive predictor compensation by using reconstructed BL texture. Source codes and test results provided by the proponents have been verified.

# Introduction

JCTVC-L0072 [1] proposes adaptive predictor compensation (APC) that uses reconstructed base-layer (BL) samples as enhancement-layer (EL) sample predictors. For Inter 2Nx2N CU, when there is at least one coded coefficient, the APC is applied by an explicit APC flag to generate EL sample predictors by averaging the EL original sample predictors and the BL reconstructed samples of the corresponding block.

# Experimental Results

The proposed method is compared with SMuC_0.1.1 for crosscheck. It was confirmed that the cross-check results exactly match to the results provided by the proponents.

Table 1. BD-rate results by JCTVC-L0072 compared to SMuC_0.1.1

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.3% | -1.7% | -1.9% |  |  |  | -0.4% | -2.0% | -2.3% |
| Class B | -0.4% | -1.7% | -1.9% | -0.6% | -2.3% | -2.7% | -1.0% | -3.8% | -4.5% |
| Overall (EL+BL) | -0.4% | -1.7% | -1.9% | -0.6% | -2.3% | -2.7% | -0.8% | -3.3% | -3.9% |
| Overall (EL) | -0.6% | -2.8% | -3.2% | -1.2% | -4.6% | -5.3% | -1.3% | -5.2% | -6.1% |
| Enc Time[%] | 101.6% |  |  | 95.5% |  |  | 103.4% |  |  |
| Dec Time[%] | 95.6% |  |  | 88.0% |  |  | 94.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.1% | -2.9% | -3.1% |  |  |  | -1.4% | -3.9% | -4.3% |
| Class B | -1.3% | -2.5% | -2.9% | -1.5% | -4.1% | -4.7% | -2.7% | -6.9% | -8.1% |
| Overall (EL+BL) | -1.2% | -2.6% | -3.0% | -1.5% | -4.1% | -4.7% | -2.4% | -6.0% | -7.0% |
| Overall (EL) | -1.9% | -4.2% | -4.7% | -2.7% | -7.9% | -8.6% | -3.3% | -8.8% | -10.2% |
| Enc Time[%] | 107.2% |  |  | 99.6% |  |  | 102.1% |  |  |
| Dec Time[%] | 97.5% |  |  | 95.4% |  |  | 94.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Reference

[1] T.-D. Chuang, Y.-W. Huang, P. Lai, S. Liu and S. Lei, “TE3-4.3.1: Adaptive predictor compensation by using reconstructed BL texture,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0072, Jan. 2013.