| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0356 |
| --- | --- |

| Title: | Crosscheck of JCTVC-L0139 on Non-TE5: Motion assignment for Intra/Intra BL at enhancement layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1479 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document provides cross-check results for motion assignment for spatial neighboring Intra and IntraBL PUs in the merge process at enhancement layer. Source codes and test results provided by the proponents have been verified.

# Introduction

JCTVC-L0139 [1] proposes the modified merge process for enhancement layer as follows:

1) In case of spatial a neighboring PU is Intra or Intra BL, the corresponding PU at base layer is considered as a merge candidate if it is neither Intra nor Intra BL.

2) Uncompressed base-layer MVs are used

3) The possible number of candidates at the base layer obtained in this way is limited to 2.

# Experimental Results

The proposed method was compared with SMuC-0.1.1 software with and without inter-layer motion prediction, respectively. It was confirmed that the cross-check results exactly match to the results provided by the proponents. Please note that encoding/decoding time in Table 2 may not be accurate.

Table 1. BD-rate results by JCTVC-L0139 compared to SMuC-0.1.1 without inter-layer motion prediction

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.7% | -5.5% | -5.3% |  |  |  | -2.8% | -6.5% | -6.9% |
| Class B | -2.1% | -3.7% | -3.8% | -2.4% | -3.8% | -4.2% | -2.4% | -4.1% | -4.4% |
| Overall (EL+BL) | -2.3% | -4.2% | -4.2% | -2.4% | -3.8% | -4.2% | -2.5% | -4.8% | -5.1% |
| Overall (EL) | -4.4% | -7.7% | -7.8% | -6.4% | -9.6% | -10.3% | -5.1% | -9.2% | -9.7% |
| Enc Time[%] | 97.7% |  |  | 92.8% |  |  | 96.5% |  |  |
| Dec Time[%] | 103.7% |  |  | 97.5% |  |  | 96.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.6% | -3.4% | -3.3% |  |  |  | -2.5% | -4.3% | -4.3% |
| Class B | -1.2% | -2.3% | -2.3% | -1.3% | -2.1% | -2.4% | -1.9% | -2.7% | -2.9% |
| Overall (EL+BL) | -1.3% | -2.6% | -2.6% | -1.3% | -2.1% | -2.4% | -2.1% | -3.1% | -3.3% |
| Overall (EL) | -2.4% | -4.5% | -4.4% | -2.9% | -4.6% | -4.9% | -3.6% | -5.4% | -5.6% |
| Enc Time[%] | 95.1% |  |  | 89.2% |  |  | 94.1% |  |  |
| Dec Time[%] | 96.3% |  |  | 92.4% |  |  | 94.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2. BD-rate results by JCTVC-L0139 compared to SMuC-0.1.1 with inter-layer motion prediction using uncompressed base-layer MVs

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% |  |  |  | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Overall (EL+BL) | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (EL) | 0.0% | -0.1% | -0.1% | -0.2% | -0.2% | -0.3% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 94.1% |  |  | 93.9% |  |  | 93.4% |  |  |
| Dec Time[%] | 107.5% |  |  | 105.6% |  |  | 96.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% |  |  |  | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (EL+BL) | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (EL) | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 95.0% |  |  | 93.5% |  |  | 94.7% |  |  |
| Dec Time[%] | 107.8% |  |  | 103.1% |  |  | 99.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Reference

[1] C. Kim and B. Jeon, “Non-TE5: Motion assignment for Intra/Intra BL at enhancement layer,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0139, Jan. 2013.