| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0357 |
| --- | --- |

| Title: | Crosscheck of JCTVC-L0195 on Non-TE4: Switchable de-ringing filter for inter-layer prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1479 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document provides cross-check results for switchable inter-layer de-ringing filter for inter-layer texture prediction by Samsung in JCTVC-L0195. Source codes and test results provided by the proponents have been verified and the results exactly match to the results provided by the proponents.

# Introduction

It is claimed in JCTVC-L0195 [1] that ringing artifacts are commonly present around edge regions in up-sampled reference-layer pictures, which degrades the performance of inter-layer textures prediction. For this reason, 3x3 de-ringing filters are proposed, which is kind of bilateral filters. The use of de-ringing filter can be switched on/off for each IntraBL block using a flag.

# Experimental Results

The cross-check is done under the TE4 test conditions [2] and compared with the SMuC-0.1.1 software. It is confirmed that the cross-check results exactly match to ones provided by the proponents. Table 1 shows the BD-rate results by the proposed method when compared to SMuC-0.1.1.

Table 1. BD-rate results by JCTVC-L0195 compared to SMuC-0.1.1

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -1.7% | -2.1% | -2.3% |  |  |  |  |  |  |
| Class B | -0.6% | -0.6% | -0.6% | -0.4% | -0.4% | -0.4% |  |  |  |
| Overall (EL+BL) | -0.9% | -1.0% | -1.1% | -0.4% | -0.4% | -0.4% |  |  |  |
| Overall (EL) | -1.9% | -2.1% | -2.3% | -1.0% | -1.2% | -1.1% |  |  |  |
| Enc Time[%] | 116.0% |  |  | 113.7% |  |  |  |  |  |
| Dec Time[%] | 112.4% |  |  | 105.9% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.7% | -0.5% | -0.8% |  |  |  | -0.6% | -0.2% | -0.1% |
| Class B | -0.6% | -0.1% | 0.0% | -0.3% | 0.2% | 0.2% | -0.5% | 0.3% | 0.4% |
| Overall (EL+BL) | -0.9% | -0.2% | -0.3% | -0.3% | 0.2% | 0.2% | -0.6% | 0.1% | 0.3% |
| Overall (EL) | -1.8% | -0.6% | -0.7% | -0.7% | 0.2% | 0.3% | -1.0% | 0.1% | 0.3% |
| Enc Time[%] | 101.9% |  |  | 96.3% |  |  | 100.4% |  |  |
| Dec Time[%] | 101.4% |  |  | 93.6% |  |  | 101.3% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.7% | -0.8% | -1.0% |  |  |  | -0.8% | -0.5% | -0.2% |
| Class B | -1.0% | -0.6% | -0.4% | -0.8% | -0.1% | 0.0% | -1.1% | -0.1% | 0.2% |
| Overall (EL+BL) | -1.2% | -0.7% | -0.6% | -0.8% | -0.1% | 0.0% | -1.0% | -0.2% | 0.0% |
| Overall (EL) | -2.3% | -1.4% | -1.2% | -2.2% | -0.5% | -0.2% | -1.9% | -0.7% | -0.2% |
| Enc Time[%] | 104.9% |  |  | 100.4% |  |  | 101.8% |  |  |
| Dec Time[%] | 105.5% |  |  | 104.5% |  |  | 109.3% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Reference

[1] Z. Ma and F. Fernandes, “Non-TE4: Switchable de-ringing filter for inter-layer prediction,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0195, Jan. 2013.

[2] J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong and J. Part, “Description of Tool Experiment B4: Inter-layer filtering in SHVC,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-K1104, Oct. 2012.