| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0363-v5 |
| --- | --- |

| Title: | Miscellaneous cleanup remarks for HEVC version 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Microsoft Corporation 1 Microsoft Way Redmond, WA 98052 USA Sachin Deshpande 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 425 703 5308 garysull@microsoft.com 1-360-817-8486 sdeshpande@sharplabs.com |
| Source: | Microsoft, Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document contains a list of miscellaneous cleanup remarks for HEVC version 1.

Version 2 of this document further includes proposed text for support of signalling and use of bit rates for DU based CPB operations.

In version 3 of this document, an additional item was included to suggest a modification of the profile/tier/level syntax structure to restructure a loop that includes flags that prevent byte alignment, such byte alignment is preserved and the number of bytes used for sub-layer presence flags is fixed.

# List of remarks

A list of miscellaneous cleanup remarks is hereby provided as follows:

- Profile/tier/level syntax should be reviewed, including the following aspects:

  - We note that there is no clear ability to define a higher tier for a level, that the headroom for higher level numbers is limited, that the range of profile indicator values seems limited (32 maximum, 3 used), that the nesting relationship between the drafted Main 10 and Main profiles and possibly the nesting relationship between the drafted Main or Main 10 and Main Still Picture profiles could perhaps alternatively be expressed as a constraint flag indication rather than as a different value of general_profile_idc.

  - We suggest to consider requiring, rather than recommending, that Main 10 profile compatibility be expressed when Main profile compatibility is expressed.

  - It has previously been suggested to enable some way to identify a bitstream as having the currently-specified syntax and decoding process without necessarily conforming to all constraints of a currently-specified profile/tier/level combination (e.g. for experimental research bitstream exchanges or some form of future extensibility). One possibility would be to use general_profile_idc equal to 0 for this. Another would be to define an "unconstraint" flag within the context of an existing general_profile_idc value. This should be discussed.

  - It may be desirable to increase the length of general_reserved_zero_16bits (which are anticipated to become "constraint set" flags in future editions, and which could also be hypothetically used for other future-extensibility purposes) to 48 bits. (See the prior meeting notes for JCTVC-K0119.)

  - (Editorial) The semantics of general_profile_idc should be modified to say something like this: "When the coded video sequence conforms to multiple profiles, general_profile_idc should indicate the profile that provides the preferred decoded result or the preferred bitstream identification, as determined by the encoder (in a manner not specified in this Specification)."

- HRD aspects:

  - It should be discussed whether or not modulo wrapping of the CPB removal delay is actually intended (or desirable) to be specified.

  - Regarding sub-picture-level HRD operation, we note that contribution JCTVC-L0044 requests consideration of decoupling the DU and AU HRD operation. This may be advisable, especially since the AU HRD operation is more well proven. We also suggest consideration of a further decoupling – such that the bit rates for DU and AU operation may not need to be the same – since ultra-low-delay decoding may use very-high-speed local data links. We also suggest that in the event of underflow with low_delay_flag equal to 1, it does not seem reasonable to require the actual CPB removal time for the final DU to match the actual CPB removal time for the AU, since the clock increment for slippage of DU times is different than the clock increment for slippage of AU times.

  - (Editorial) We should discuss the possibility of renaming the low_delay_flag (e.g. as "underflow_allowance_flag"), as its name may be confusing (e.g. with respect to DU-level HRD operation). However, we should be aware that a similar flag is similarly named in prior standards.

- SEI messages:

  - The various SEI messages should be reviewed in light of the fact that we now have the ability to define SEI messages of both prefix and suffix types. In particular, we suggest that the post-filter hint, user data registered, and user data unregistered (and possibly progressive refinement segment end) SEI message types should be allowed to be either prefix or suffix SEI messages.

  - The scope of the scene information SEI message seems to extend beyond the scope of the coded video sequence in which it appears. This seems ill-advised.

  - It should be discussed whether the SEI message type ID should conceptually have a different "numbering space" for prefix and suffix messages. Currently, the message type definition does not depend on whether the message is a prefix or a suffix.

- On the contouring artefact issue of the previous meeting: In prior consideration of JCTVC-K0139, it was suggested that it might be worthwhile to apply the modification to 16x16 TUs in cases where max TU size is 16. This topic was suggested for further study at the time, and should be further discussed.

- The text relating to slice_temporal_mvp_enable_flag does not seem entirely clear. This flag was recently discussed in JCTVC-K0251, JCTVC-K0341, and item 1.3 of JCTVC-K0120 of the previous JCT-VC meeting and some changes were included in the resulting JCTVC-K1003 text. Some attempt to deal with it has been included in JCTVC-L0030. This should be reviewed.

- On CABAC: Our understanding, based on a verbal remark, is that the initialization of the CABAC context probability estimate for cu_skip_flag may be a bit strange – specifically, that the probability estimate for skipping is lower when both the top and left neighbours are skipped than when only one of them is skipped.

- POC vs. Reference index: Throughout the HEVC specification, picture order count (POC) is used to determine whether two picture references are considered equal (as opposed to using indices into a reference picture list). One exception is in the motion vector derivation process. It is proposed to consistently use POC information in the motion vector derivation process. Changes to the specification are minor and have a normative effect only in cases where a same reference picture appears multiple times in a reference picture list.

- In-loop filtering: When in-loop filtering is enabled across tile boundaries but not across slice boundaries, some tile boundaries may not be slice boundaries, so some tile boundaries would seem to be specified to be filtered while others are not. This may require a decoder to track slice IDs across tiles to figure out whether or not to filter the tile boundary. It should be discussed whether this is really intended or advisable.

- We suggest general study of the places in JCTVC-K1003 (and JCTVC-L0030) at which the string "[Ed" appears.

- We suggest the following change of a loop in the profile_tier_level( ) syntax structure, to modify the following (based on the JCTVC-L0030-v2 text with removal of the profilePresentFlag (as agreed), which needs "orthogonal" adjustment to reflect other actions taken), such that byte alignment is preserved and the sub-layer presence flag data has a fixed length when present.

| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| --- | --- |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| if( profilePresentFlag && sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_reserved_zero_16bits[ i ] | u(16) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |

to the following (in which reserved_zero_2bits is reserved for future use and required to be 0):

| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| --- | --- |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| } |  |
| for( j = i; ( i != 0 ) && j < 8; j++ ) |  |
| reserved_zero_2bits | u(2) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| if( profilePresentFlag && sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_reserved_zero_16bits[ i ] | u(16) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |

The above-suggested version assumes that having a fixed length for the presence flag bit fields is desirable. If only achieving byte alignment is deemed sufficient, this can be simplified to the following:

| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| --- | --- |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| } |  |
| for( j = i % 4; ( i != 0 ) && j < 4; j++ ) |  |
| reserved_zero_2bits | u(2) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| if( profilePresentFlag && sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_reserved_zero_16bits[ i ] | u(16) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |

# Syntax and semantics for signalling and use of bit rates in DU based CPB operations

Following highlighted syntax and semantics are proposed in Video Usability Information (Annex E) section.

## Sub-layer HRD parameters syntax

| sub_layer_hrd_parameters( subLayerId ) { | Descriptor |
| --- | --- |
| for( i = 0; i <= CpbCnt; i++ ) { |  |
| bit_rate_value_minus1[ i ] | ue(v) |
| cpb_size_value_minus1[ i ] | ue(v) |
| if( sub_pic_cpb_params_present_flag ) { |  |
| cpb_size_du_value_minus1[ i ] | ue(v) |
| bit_rate_du_value_minus1[ i ] | ue(v) |
| } |  |
| cbr_flag[ i ] | u(1) |
| } |  |
| } |  |

## Sub-layer HRD parameters semantics

The variable CpbCnt is set equal to cpb_cnt_minus1[ subLayerId ].

bit_rate_value_minus1[ i ] (together with bit_rate_scale) specifies the maximum input bit rate for the i-th CPB when the CPB operates at the access unit level. bit_rate_value_minus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. For any i > 0, bit_rate_value_minus1[ i ] shall be greater than bit_rate_value_minus1[ i − 1 ]. When SubPicCpbFlag is equal to 0 the bit rate in bits per second is given by

BitRate[ i ] = ( bit_rate_value_minus1[ i ] + 1 ) * 2(6 + bit_rate_scale)	(E-1)

When SubPicCpbFlag is equal to 0 and the bit_rate_value_minus1[ i ] syntax element is not present, the value of BitRate[ i ] is inferred to be equal to cpbBrVclFactor * MaxBR for VCL HRD parameters and to be equal to cpbBrNalFactor * MaxBR for NAL HRD parameters, where MaxBR, cpbBrVclFactor and cpbBrNalFactor are specified in subclause A.4.

cpb_size_value_minus1[ i ] is used together with cpb_size_scale to specify the i-th CPB size when the CPB operates at the access unit level. cpb_size_value_minus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. For any i greater than 0, cpb_size_value_minus1[ i ] shall be less than or equal to cpb_size_value_minus1[ i − 1 ].

When SubPicCpbFlag is equal to 0, the CPB size in bits is given by

CpbSize[ i ] = ( cpb_size_value_minus1[ i ] + 1 ) * 2(4 + cpb_size_scale)	(E-2)

When SubPicCpbFlag is equal to 0 and the cpb_size_value_minus1[ i ] syntax element is not present, the value of CpbSize[ i ] is inferred to be equal to cpbBrVclFactor * MaxCPB for VCL HRD parameters and to be equal to cpbBrNalFactor * MaxCPB for NAL HRD parameters, where MaxCPB, cpbBrVclFactor and cpbBrNalFactor are specified in subclause Error: Reference source not found.

cpb_size_du_value_minus1[ i ] is used together with cpb_size_du_scale to specify the i-th CPB size when the CPB operates at sub-picture level. cpb_size_du_value_minus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. For any i greater than 0, cpb_size_du_value_minus1[ i ] shall be less than or equal to cpb_size_du_value_minus1[ i − 1 ].

When SubPicCpbFlag is equal to 1, the CPB size in bits is given by

CpbSize[ i ] = ( cpb_size_du_value_minus1[ i ] + 1 ) * 2(4 + cpb_size_du_scale)	(E-3)

When SubPicCpbFlag is equal to 1 and the cpb_size_du_value_minus1[ i ] syntax element is not present, the value of CpbSize[ i ] is inferred to be equal to cpbBrVclFactor * MaxCPB for VCL HRD parameters and to be equal to cpbBrNalFactor * MaxCPB for NAL HRD parameters, where MaxCPB, cpbBrVclFactor and cpbBrNalFactor are specified in subclause Error: Reference source not found.

bit_rate_du_value_minus1[ i ] (together with bit_rate_scale) specifies the maximum input bit rate for the i-th CPB when the CPB operates at the sub-picture level. bit_rate_du_value_minus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. For any i > 0, bit_rate_du_value_minus1[ i ] shall be greater than bit_rate_du_value_minus1[ i − 1 ]. When SubPicCpbFlag is equal to 1 the bit rate in bits per second is given by

BitRate[ i ] = ( bit_rate_du_value_minus1[ i ] + 1 ) * 2(6 + bit_rate_scale)	(E-55)

When SubPicCpbFlag is equal to 1 and the bit_rate_du_value_minus1[ i ] syntax element is not present, the value of BitRate[ i ] is inferred to be equal to cpbBrVclFactor * MaxBR for VCL HRD parameters and to be equal to cpbBrNalFactor * MaxBR for NAL HRD parameters, where MaxBR, cpbBrVclFactor and cpbBrNalFactor are specified in subclause A.4.

cbr_flag[ i ] equal to 0 specifies that to decode this bitstream by the HRD using the i-th CPB specification, the hypothetical stream delivery scheduler (HSS) operates in an intermittent bit rate mode. cbr_flag[ i ] equal to 1 specifies that the HSS operates in a constant bit rate (CBR) mode. When the cbr_flag[ i ] syntax element is not present, the value of cbr_flag is inferred to be equal to 0.

# Patent rights declaration(s)

Microsoft Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).