| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14-23 Jan., 2013 | Document: JCTVC-L0366 |
| --- | --- |

| Title: | Cross-check report of Test of Transform Selection for Inter-Layer Texture Prediction on SMuC 0.1.1 (JCTVC-L0330) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenzhong Chen, Shan Liu MediaTek USA 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | +1-408-5261899 { zhenzhong.chen, shan.liu}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document is a cross-check report for the contribution from Qualcomm on Transform Selection for Inter-Layer Texture Prediction (JCTVC-L0330) [1]. It is verified that the encoding and decoding results completely match the results reported by the proponent in JCTVC-L0330. The technical description also matches the implementation in the software.

- Technical description

In addition to the two transforms, i.e., DCT Type-2 and DST Type-7, in the HEVC main profile, the proponent proposed to apply additional transforms on the enhancement layer for luma component and signal at CU level by transform_selection_index. The signaling is skipped if the CU does not contain non-zero coefficients. The transform selection is based on RD optimization with 1 level RQT.

DCT Type-2, DCT Type-3, and DST Type-3 are proposed as the transform candidates for the luma component of enhancement layer in JCTVC-L0330. This means that 2 additional transforms, i.e., DCT Type 3 and DST Type 3, are introduced in the software. This will be verified as Case 1 in the cross-check report.

In addition, a case of two transform candidates, i.e., DCT Type-2 and DCT Type-3, is tested in JCTVC-L0330. In this case, 1 additional transform, i.e., DCT Type-3, is introduced in the software. This will be verified as Case 2 in the cross-check report.

# Simulation results

The proposed method in JCTVC-L0330 is verified on SMuC 0.1.1 with IntraBL4x4 transform bug fix. Anchor is generated using the same software. Test conditions specified in [2] are used. BD-rate is calculated using both enhancement layer and base layer rates. Besides verifying the results for the 3 transform candidates as Case 1, the results for 2 transform candidates are also verified as Case 2.

The table below summarizes the performance of the tested methods. Full results are provided in the attached Excel sheets. It is verified that BD-rate saving completely matches the results reported by the proponent and the enc/dec time is also similar to those reported by the proponent.

Table 1.  BD rate comparison results of L0330 against SMuC 0.1.1 with bug fix for Case 1 (3 transform candidates).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.86% | 1.62% | 1.65% |  |  |  |
| Class B | -1.60% | 1.47% | 1.40% | -1.45% | 2.38% | 2.39% |
| Overall (EL+BL) | -1.67% | 1.51% | 1.47% | -1.45% | 2.38% | 2.39% |
| Overall (EL) | -3.12% | 3.41% | 3.30% | -4.58% | 8.08% | 8.17% |
| Enc Time[%] | 109.51% |  |  | 108.28% |  |  |
| Dec Time[%] | 100.16% |  |  | 100.29% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 1.  BD rate comparison results of L0330 against SMuC 0.1.1 with bug fix for Case 2 (2 transform candidates).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.25% | 1.10% | 1.15% |  |  |  |
| Class B | -1.06% | 0.96% | 0.93% | -0.97% | 1.53% | 1.56% |
| Overall (EL+BL) | -1.11% | 1.00% | 0.99% | -0.97% | 1.53% | 1.56% |
| Overall (EL) | -2.08% | 2.29% | 2.24% | -3.09% | 5.35% | 5.44% |
| Enc Time[%] | 104.19% |  |  | 103.53% |  |  |
| Dec Time[%] | 100.28% |  |  | 99.69% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- L. Guo, M. Karczewicz, and J. Chen, “Test of Transform Selection for Inter-Layer Texture Prediction on SMuC 0.1.1,” JCT-VC Document, JCTVC-L0330, Geneva, Switzerland, Jan. 2013.

- L. Guo, Y. He, D. -K. Kwon, J. Zan, H. Lakshman, and J.-W. Kang, “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC”, JCTVC-K1102, Shanghai, China, Oct. 2012.