| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0372v2 |
| --- | --- |

| Title: | Clean-up of hrd_parameters() high-level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dzung Hoang 3960 Freedom Circle, Suite 100 Santa Clara, CA 95054 U.S.A. | Tel: Email: | +1 408 350 5052 dzung.hoang (at) zenverge.com |
| Source: | Zenverge, Inc. |  |  |

_____________________________

# Abstract

This document presents several proposals to clean up the hrd_parameters() high-level syntax.

# Problem Statement

In the WD (JCTVC-L0030_v2) hrd_parameters() syntax contains the following.

| hrd_parameters( commonInfPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| ... |  |
| for( i = 0; i <= maxNumSubLayersMinus1; i++ ) { |  |
| fixed_pic_rate_general_flag[ i ] | u(1) |
| if( !fixed_pic_rate_general_flag[ i ] ) |  |
| fixed_pic_rate_within_cvs_flag[ i ] | u(1) |
| if( fixed_pic_rate_within_cvs_flag[ i ] ) |  |
| elemental_duration_in_tc_minus1[ i ] | ue(v) |
| low_delay_hrd_flag[ i ] | u(1) |
| cpb_cnt_minus1[ i ] | ue(v) |
| if( nal_hrd_parameters_present_flag ) |  |
| sub_layer_hrd_parameters( i ) |  |
| if( vcl_hrd_parameters_present_flag ) |  |
| sub_layer_hrd_parameters( i ) |  |
| } |  |
| } |  |

The following syntax elements are conditionally present: fixed_pic_rate_within_cvs_flag[i] and elemental_duration_in_tc_minus1[i]. In the Section E.2.2 of the WD, the semantics of fixed_pic_rate_general_flag[i] states that “When fixed_pic_rate_general_flag[ i ] is equal to 1, the value of fixed_pic_rate_within_cvs_flag[ i ] is inferred to be equal to 1.”

In contrast, the following syntax elements are always present: low_delay_hrd_flag[i] and cpb_cnt_minus1[i]. However, low_delay_hrd_flag[i] is restricted to be 0 when fixed_pic_rate_within_cvs_flag[i] is equal to 1. Similarly cpb_cnt_minus1[i] is restricted to be 0 when low_delay_hrd_flag[i] is equal to 1.

# Proposed Changes to Syntax and Semantics

For consistency, it is proposed that low_delay_hrd_flag[i] and cpb_cnt_minus1[i] be conditionally present in the syntax. Below is the proposed changes marked in yellow.

| hrd_parameters( commonInfPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| ... |  |
| for( i = 0; i <= maxNumSubLayersMinus1; i++ ) { |  |
| fixed_pic_rate_general_flag[ i ] | u(1) |
| if( !fixed_pic_rate_general_flag[ i ] ) |  |
| fixed_pic_rate_within_cvs_flag[ i ] | u(1) |
| if( fixed_pic_rate_within_cvs_flag[ i ] ) |  |
| elemental_duration_in_tc_minus1[ i ] | ue(v) |
| if( !fixed_pic_rate_within_cvs_flag[ i ] ) |  |
| low_delay_hrd_flag[ i ] | u(1) |
| if( !low_delay_hrd_flag[ i ] ) |  |
| cpb_cnt_minus1[ i ] | ue(v) |
| if( nal_hrd_parameters_present_flag ) |  |
| sub_layer_hrd_parameters( i ) |  |
| if( vcl_hrd_parameters_present_flag ) |  |
| sub_layer_hrd_parameters( i ) |  |
| } |  |
| } |  |

Changes to E.2.2 are proposed below.

low_delay_hrd_flag[ i ] specifies the HRD operational mode, when HighestTid is equal to i, as specified in Annex Error: Reference source not found. When fixed_pic_rate_within_cvs_flag[ i ] is equal to 1, low_delay_hrd_flag[ i ] shallis inferred to be equal to 0.

NOTE 1 – When low_delay_hrd_flag[ i ] is equal to 1, "big pictures" that violate the nominal CPB removal times due to the number of bits used by an access unit are permitted. It is expected, but not required, that such "big pictures" occur only occasionally.

cpb_cnt_minus1[ i ] plus 1 specifies the number of alternative CPB specifications in the bitstream of the coded video sequence when HighestTid is equal to i. The value of cpb_cnt_minus1[ i ] shall be in the range of 0 to 31, inclusive. When low_delay_hrd_flag[ i ] is equal to 1, cpb_cnt_minus1[ i ] shall be equal to 0. When cpb_cnt_minus1[ i ] is not present, it is inferred to be equal to 0.

# Patent rights declaration(s)

Zenverge, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).