| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0378r3 |
| --- | --- |

| Title: | AHG7: Performance evaluation of HEVC on YUV4:2:2 interlace video sources |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuo Sugimoto Shun-ichi Sekiguchi Tokumichi Murakami 5-1-1, Ofuna, Kamakura, Kanagawa, JAPAN | Tel: Email: | +81 467 41 2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports simulation results of the common software of HEVC range extension development on interlaced video sources. Coding performance comparison of HEVC and AVC on 4:2:0/4:2:2 interlaced video sources are conducted. It was reported that the performance improvement of HEVC compared with AVC on 4:2:2 interlaced sources is 56.2%, while the improvement on 4:2:0 interlaced sources is 32.2%. It is expected by the authors that some problems exist on 4:2:2 interlace coding using JM18.4.

# Introduction

In the first version of HEVC, video coding of interlaced sources is supported using SEI massage. The coding performance improvement compared with AVC on interlaced video sources was reported in the last meeting in JCTVC-K0331 [1]. This contribution reports the coding performance improvement on 4:2:2 interlaced sources.

# Simulations

## Simulation conditions

To evaluate the coding performance of HEVC 4:2:2 interlaced coding, 4:2:0 and 4:2:2 video sources of the same materials are coded using HM9.0 and JM18.4.

Test conditions of the simulations are as follows:

| Video format | Software | settings |
| --- | --- | --- |
| 4:2:0 interlace | HM9.0 | RA-HE10 field coding |
|  | JM18.4 | Same as [1] |
| 4:2:2 interlace | HM-range-extensions branch | RA-HE10 field coding |
|  | JM18.4 | 4:2:2 profile |

We used test materials in the second version of ITE/ARIB (The Institute of Image Information and Television Engineers / Association of Radio Industries and Businesses) test materials. Summary of image characteristics provided in JCTVC-J0152[2] are as follows.

| Series | Materials A | Materials B |  | Materials C |
| --- | --- | --- | --- | --- |
| Video standard | Part 2 of Rec. ITU-R BT.709 |  |  |  |
| Number of pixels | 1920×1080 |  |  |  |
| Bit depth | 10-bit |  |  |  |
| Duration | 15 sec |  |  |  |
| Signal format | 4:4:4/59.94i 4:4:4/50i | 4:2:2/59.94p | 4:4:4/24p | 4:2:2/59.94i 4:2:2/50i |
| Colour mode | RGB | YCBCR | RGB | YCBCR |
| Scanning | Interlace | Progressive | Progressive | Interlace |

We used 4:2:2 sequences of Materials C for interlaced sources. 4:2:0 sequences are obtained by down-converting from 4:2:2 sequences. 5 test sequences are used for our simulations.

## Simulation results

Coding performance improvements are evaluated using BD-rate.

Table 1 Comparison of HEVC vs AVC on Interlace / 4:2:0

|  | Y | U | V |
| --- | --- | --- | --- |
| Colorful world B | -49.6% | -56.2% | -50.9% |
| Ginkgo trees | -15.1% | -28.0% | -34.2% |
| Fountain(dolly) | -30.3% | -56.4% | -46.4% |
| Basketball | -23.8% | -28.7% | -43.6% |
| Horse racing(dirt) | -42.4% | -54.4% | -53.5% |
| Overall | -32.2% | -44.7% | -45.7% |

Table 2 Comparison of HEVC vs AVC on Interlace / 4:2:2

|  | Y | U | V |
| --- | --- | --- | --- |
| Colorful world B | -88.1% | -97.4% | -96.1% |
| Ginkgo trees | -40.6% | -80.5% | -77.5% |
| Fountain(dolly) | -30.0% | -0.0% | -50.6% |
| Basketball | -60.7% | -90.7% | -86.6% |
| Horse racing(dirt) | -61.5% | -94.0% | -89.1% |
| Overall | -56.2% | -72.5% | -80.0% |

According to these simulation results, it was found out that coding performance difference between 4:2:2 and 4:2:0 interlaced sources is 24%.

# Conclusion

We propose to investigate further on 4:2:2 interlaced sources as one of the AHG activities.

# References

- K. Sugimoto, A. Minezawa, S. Sekiguchi and T. Murakami, "HEVC performance evaluation on interlaced video sequences", JCTVC-K0331, Meeting: Shanghai, CN, 10–19 Oct. 2012.

- T.Suzuki, P. Silcock, K. Sharman, N. Saunders, J. Gamei, and A. Tabatabai, “Proposals on HEVC professional extensions”, JCTVC-J0152, Meeting: Stockholm, SE, 11–20 July 2012.