| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0379 |
| --- | --- |

| Title: | HEVC real-time hardware encoder for HDTV signal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Akira Minezawa Hiroharu Sakate Nobuaki Motoyama Shun-ichi Sekiguchi Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN Yasuko Sugito Kazuhisa Iguchi Atsuro Ichigaya Shin-ichi Sakaida NHK(Japan Broadcasting Corporation) 1-10-11, Kinuta, Setagaya, Tokyo, JAPAN | Tel: Email: | +81-467-41-2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation, NHK |  |  |

_____________________________

# Abstract

This contribution is to provide information on a development of hardware encoder implementation based on HEVC coding tools. The developed encoder is a FPGA-based prototype system that supports real-time encoding of 1080@60P with Main 10 profile level functionality, and it has been confirmed that it can achieve significant coding gain relative to existing professional-class AVC encoder product.

# Summary

This contribution is to provide information on our development of hardware encoder implementation based on HEVC coding tools. The developed encoder is a FPGA-based prototype system that supports real-time encoding of up to 1080@60P format with Main 10 profile level functionality. We have confirmed that it can achieve significant coding gain relative to existing professional-class AVC encoder product. The encoding algorithm design employs our original prediction parameter decision technique combined with CU tree shape decision, which can achieve efficient prediction performance while achieving real-time processing. The coding tool specification is originally based on HM-4, while the current design is almost aligned with the latest draft.

# Specification

Table 1 shows specification of supported features/configurations of the developed encoder. The encoder is implemented as a single custom board (233 mm x 229 mm), whose snapshot is shown in Figure 1.

Table 1  Specification of the developed hardware encoder

| Source format | 1920×1080@60i/60p, YUV4:2:0, 10-bit |
| --- | --- |
| GOP structure | Hierarchical B structure |
| CTB size | 64x64 |
| Max CU depth | 3 |
| Reference picture management | 2 reference pictures (1 for list0 and list1 each) as short-term reference pictures |
| Max TU size | 32x32 |
| Max TU depth | 0 (i.e., RQT is disabled) |

Figure 1  Photo of the developed hardware encoder

# Preliminary evaluation on coding performance

The developed hardware encoder is currently under performance tuning in order to evaluate its actual coding performance. In this contribution, preliminary reference information on coding performance of the developed encoder is shown by using algorithm simulator that produces encoding result equivalent to that of the hardware prototype. Figure 2 and 3 represent the objective performance relative to professional-class AVC encoder product.

In this evaluation, we used 1080@60P test sequences chosen from second version of ITE/ARIB (The Institute of Image Information and Television Engineers / Association of Radio Industries and Businesses) test materials [1]. PSNR of anchor data were measured using decoded video signal output from decoder that receives coded bitstream of the AVC encoder product. The video signal format is YUV4:2:2 10-bit that is compliant with HD-SDI standard. It is noted that the AVC encoder down-converts this source to YUV4:2:0 8-bit and encodes the 8-bit video with High Profile configuration. Since our prototype encoder encodes the input video as YUV4:2:0 10-bit signal, we calculated BD value, which is shown in each figure’s caption, with luma 10-bit video signal for obtaining this reference data.

Figure 2  R-D performance for “Fountain_dolly” (BD-bitrate: -46.1%)

Figure 3  R-D performance for “Horse_racing_dirt” (BD-bitrate: -43.7%)

Figure 4 depicts result of subjective assessments of the quality of both HEVC and AVC using DSIS method [2], which evaluates the above two test sequences. The X-axis represents the bitrates. The values on the X-axis indicate the bitrates of HEVC(the developed encoder) / bitrates of AVC(product). It is noted that the bitrate for HEVC is set to half value of that of AVC. The Y-axis represents subjective quality. The error bars represent 95% confidence interval.

Figure 4  Subjective quality comparison

# Conclusion

This contribution has provided information on our hardware encoder implementation that supports HEVC coding tools those are comparable to Main 10 profile specification. It is believed that this development demonstrates that market availability of real-time 1080@60P encoder supporting HEVC Main/Main 10 profile, which can achieve significant coding gain relative to existing state-of-the-art AVC encoder product, should be feasible within a short time frame after the completion of its standardization.

# Appreciation

We would like to express our appreciation with a remark that this development is partially supported by one of "the project for Resilient Information and Communication Networks (The third supplementary budget of the general account, FY2011)" by the Ministry of Internal Affairs and Communications, Japan.

References

[1] T.Suzuki, P. Silcock, K. Sharman, N. Saunders, J. Gamei, and A. Tabatabai, “Proposals on HEVC professional extensions”, JCTVC-J0152, Meeting: Stockholm, SE, 11–20 July 2012.

[2] Rec.ITU-R BT.500-13, “Methodology for the subjective assessment of the quality of television pictures”, January 2012.