| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0381 |
| --- | --- |

| Title: | Parallelization indication in HEVC extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sühring, R. Skupin, V. George, T. Schierl Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Email: | karsten.suehring@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to add a high level syntax element to the VPS that indicates for each layer, if high level parallelization tools allow a higher level of parallelization in the enhancement layer compared to the corresponding base layer and by which factor the parallelisation opportunity differs.

# Introduction

For single-layer HEVC [1], parallelization opportunities are indicated by min_spatial_segmentation_idc [2] in the VUI syntax. In a scalable bitstream [3], this indication would have to be collected from the SPSs/VUIs of all involved layers to provide a decoder with the necessary information with respect to parallelization.

This document proposes to add another easily accessible decoder hint to high level syntax that indicates the minimum parallelization factor between base and enhancement layers that is signalled in a compiled form, e.g. in the VPS extension.

When tiles are used, a signalled factor n means that the image area covered by a base layer tile is divided in n equally sized enhancement layer tiles. When WPP is used, a factor n means that for each base layer substream, n enhancement layer substreams can be used.

If spatial scalability is used and the parallelization factor matches the factor of images sizes between base and enhancement layer, the decoder can allocate more parallel resources (like CPU cores) as indicated by the signalled factor.

It might also help an opportunistic decoder (or gateway) to select the proper layers that it might be able to decode using multiple cores where the specified level would exceed the capabilities for a single core.

# Proposed syntax and semantics

Syntax changes relative to [4] are marked in yellow.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1 ; i++ ) { |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| min_spatial_segmentation_ratio[ i ] | ue(v) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

min_spatial_segmentation_ratio, when not equal to 0,specifies the minimum factor between min_spatial_segmentation_idc of the base layer and min_spatial_segmentation_idc of layer i in multiples of 8 minus 8.

With blMinSpatialSegmentation being equal to min_spatial_segmentation_idc of the base layer and elMinSpatialSegmentation being equal to min_spatial_segmentation_idc of the enhancement layer, the following restriction shall apply:

8 * elMinSpatialSegmentation  blMinSpatialSegmentation > min_spatial_segmentation_ratio[ i ] + 8

min_spatial_segmentation_ratio equal to 0 specifies no constraint on the value of min_spatial_segmentation_idc of base and enhancement layers.

# References

- B. Bross, “High Efficiency Video Coding (HEVC) text specification draft 9“, JCTVC-K1003, Shanghai, CN, Oct. 2012

- J. Samuelsson, et al., “Decoder parallelism indication”, JCTVC-K0236, Shanghai, CN, Oct. 2012

- “Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC)“, ISO/IEC JTC1/SC29/WG11 N12957, Stockholm, Sweden, July 2012

- J. Boyce, Y.-K. Wang, "NAL unit header and parameter set designs for HEVC extensions", JCTVC-K1007, Shanghai, CN, Oct. 2012.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).