| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0384 |
| --- | --- |

| Title: | Non-TE3: Cross-check results of JCTVC-L0215 on simplification of intra residual planar prediction in TE3-4.2.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Krishna Rapaka 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +858 658 2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of a cross-check of JCTVC-L0215 on simplification of intra residual planar prediction in TE3-4.2.2. The simulation results reportedly matched those provided by the proponents.

# Introduction

The Residual Planar Intra Prediction was proposed in SMuC-0.1.1 TE3-4.2.2. In this method in TE3-4.2.2 the planar prediction of the difference signal is obtained by the following steps:

- Obtain the planar prediction Px,y  from the reconstructed difference signal

- Set the bottom-right portion of the difference prediction for the planar mode to zero, i.e., set      Px,y  = 0  if (x + y) >= N+1.

In this contribution it is reported that the residual planar intra prediction is obtained by setting the bottom left pixel value and the top right pixel value to zero in the calculation of the planar prediction, i.e.,

- Reset R0,N+1 = 0 and RN+1,0 = 0

- Obtain the planar prediction Px,y  from the reconstructed difference signal.

# Experimental results

We received the source code implemented over TE3-4.2.2 SMUC 0.1.1 from the proponents, and did a code study to verify that the proposed method is implemented as described. We used the common conditions generated from AHG10 in our experiments and ran simulations for the AI cases of (AI-2x & AI 1.5x).

For the below configuration the results match with the results provided by the proponents

#define INTRA_BL                           1

#define SVC_RESI_PRED_INTRA               1

#define SVC_RESI_PRED_INTRA_NO_MDIS      1

#define SVC_RESI_PRED_INTRA_DIFF_PLANAR       0

#define DIFF_PLANAR_SIMP_B                     1

The Test results are as follows:

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | -0.3% | -0.5% |  |  |  |
| Class B | -1.3% | -0.3% | -0.5% | -0.8% | 0.2% | 0.0% |
| Overall (EL+BL) | -1.1% | -0.3% | -0.5% | -0.8% | 0.2% | 0.0% |
| Overall (EL) | -1.9% | -0.5% | -0.9% | -2.1% | 0.5% | 0.0% |
| Enc Time[%] | 161.0% |  |  | 151.8% |  |  |
| Dec Time[%] | 109.4% |  |  | 107.8% |  |  |
| Enc Mem[%] | 103.3% |  |  | 102.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

For the below configuration the results match with the results provided by the proponents

#define INTRA_BL                           1

#define SVC_RESI_PRED_INTRA               1

#define SVC_RESI_PRED_INTRA_NO_MDIS      1

#define SVC_RESI_PRED_INTRA_DIFF_PLANAR       0

#define DIFF_PLANAR_SIMP_B                     0

The Test results are as follows:

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -0.3% | -0.5% |  |  |  |
| Class B | -1.1% | -0.4% | -0.6% | -0.7% | 0.1% | -0.1% |
| Overall (EL+BL) | -1.0% | -0.4% | -0.6% | -0.7% | 0.1% | -0.1% |
| Overall (EL) | -1.7% | -0.6% | -1.0% | -2.0% | 0.4% | -0.1% |
| Enc Time[%] | 161.2% |  |  | 151.9% |  |  |
| Dec Time[%] | 109.0% |  |  | 107.4% |  |  |
| Enc Mem[%] | 103.3% |  |  | 102.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusion

In this contribution, we have presented the results of our cross-check of JCTVC-L0215. The implemented algorithm agrees with the proponent’s description, and the simulation results also match that provided by the proponents.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).