| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0386_r1 |
| --- | --- |

| Title: | AHG6: On HEVC block artifact reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Email: | geertv@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This proposal investigates a non-normative solution for the HEVC block artifacts in certain sequences that have been reported and studied during previous JCT-VC meetings. A blockiness metric is integrated in the encoder, which analyzes the large block artifacts in each picture and determines appropriate deblocking offset parameters that are signalled to the decoder. Experimental results indicate that the large block artifacts are significantly reduced. Source code is provided.

# Technical description

The blockiness metric in the encoder analyzes the vertical and horizontal edges of the 32x32 block grid in the picture. The following condition is checked at each line i perpendicular to the edge:

dpi=p2i−2p1i+p0idqi=q0i−2q1i+q2idi=dpi+dqi

2<2di<β4

If the condition is true for line i, then the following step size is computed for line i and accumulated for the entire vertical/horizontal edge that is being analyzed:

p0i−q0i

The sum for the current horizontal or vertical grid edge is multiplied by 1024 and normalized by the number of horizontal edges and picture width, or the number of vertical edges and the picture height, respectively. Subsequently, both sums are averaged and this average is compared with a threshold value, for example 2048 in the provided source code. If the average (avg) is greater than the threshold, then tc and beta offset values are computed to control the deblocking filter as follows:

Tc_offset_div2 = Clip3(2,6,avg>>9)

Beta_offset_div2 = Clip3(2,6,avg>>9)

These offsets are signalled to the decoder per slice (=picture).

# Simulation results

The blockiness metric is integrated into HM9.1. The following results are obtained for the CTC conditions (restricted to 100 frames) with the configuration parameter DeblockingFilterControlPresent set equal to 1. The execution timings are obtained in a cluster computing environment and are approximate.

|  | All Intra Main |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  |
| Class A | 0.1% | 0.1% |  | 0.1% |  |
| Class B | 0.0% | 0.0% |  | 0.0% |  |
| Class C | 0.0% | 0.0% |  | 0.0% |  |
| Class D | 0.1% | 0.1% |  | 0.1% |  |
| Class E | 0.0% | 0.0% |  | 0.0% |  |
| Class F | 0.0% | 0.0% |  | 0.0% |  |
| Overall | 0.0% | 0.0% |  | 0.0% |  |
|  | 0.1% | 0.0% |  | 0.1% |  |
| Enc Time[%] | 102% |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |
|  | Random Access Main |  |  |  |  |
|  | Y | U |  | V |  |
| Class A | 0.1% | 0.1% |  | 0.1% |  |
| Class B | 0.0% | 0.0% |  | 0.0% |  |
| Class C | 0.0% | 0.0% |  | 0.0% |  |
| Class D | 0.0% | 0.0% |  | 0.1% |  |
| Class E |  |  |  |  |  |
| Class F | 0.0% | 0.0% |  | 0.0% |  |
| Overall | 0.0% | 0.0% |  | 0.0% |  |
|  | 0.0% | 0.0% |  | 0.1% |  |
| Enc Time[%] | 101% |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  |
|  | Low delay B Main |  |  |  |  |
|  | Y | U |  | V |  |
| Class A |  |  |  |  |  |
| Class B | 0.0% | 0.0% |  | 0.0% |  |
| Class C | 0.0% | 0.0% |  | 0.0% |  |
| Class D | 0.0% | 0.1% |  | 0.0% |  |
| Class E | 0.0% | 0.0% |  | 0.0% |  |
| Class F | 0.0% | 0.0% |  | 0.0% |  |
| Overall | 0.0% | 0.0% |  | 0.0% |  |
|  | 0.0% | 0.0% |  | 0.0% |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |
|  | Low delay P Main |  |  |  |  |
|  | Y | U |  | V |  |
| Class A |  |  |  |  |  |
| Class B | 0.0% | 0.0% |  | 0.0% |  |
| Class C | 0.0% | 0.0% |  | 0.0% |  |
| Class D | 0.0% | 0.0% |  | -0.1% |  |
| Class E | 0.0% | 0.0% |  | 0.0% |  |
| Class F | 0.1% | 0.1% |  | 0.1% |  |
| Overall | 0.0% | 0.0% |  | 0.0% |  |
|  | 0.0% | 0.0% |  | 0.0% |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |

There is no significant impact on the objective quality of the CTC sequences. The encoding and decoding times are similar to the anchor.

According to [1], two types of sequences are to be studied in subjective tests:

- Type 1 sequences: These sequences are used to demonstrate improvements in visual quality: "Riverbed", "West Winds Easy", "Ducks Take Off @ 25fps", "China Speed", "Red Kayak".

  - See included file “JCTVC-L0386_results_Type1_HM91_vs_DBFM.xlsx” for bit rates corresponding with encodings: QPs 27, 32, 37, 41 and LB-Main, RA-Main configurations

- Type 2 sequences: These sequences are used to demonstrate that there is no visual degradation on common test conditions: "Kimono", "Cactus", "BasketballDrillText", "BQMall", and "Johnny".

  - See included file “JCTVC-L0386_results_Type2_HM91_vs_DBFM.xlsx” for bit rates corresponding with encodings: QPs 27, 32, 37, 41 and LB-Main, RA-Main configurations

Our observation is that the encoder blockiness metric is able to substantially reduce block artifacts in Type 1 sequences without degrading the quality of the Type 2 sequences.

# Conclusion

Proposed to adopt the blockiness metric into the HM10 encoder software.

# References

- A. Norkin, T. Suzuki, M. Budagavi, G. Van der Auwera, “Description of Core Experiment 1: Deblocking Filter,” 10th JCT-VC Meeting, Stockholm, Sweden, July 2012, Doc. JCTVC-J1101

# Patent rights declaration

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).