| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0390 |
| --- | --- |

| Title: | On layer id and dimension id mapping in VPS extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is observed that it may not be uncommon for scalable HEVC bitstreams to have only one type of scalability. In such case, the mapping between layer identification and scalability identification in VPS extension would be redundant layer id and dimension id are the same so that there is no need for signalling the mapping. This contribution proposes modification to avoid signalling the mapping between layer id and dimension id when a bitstream uses only one type of scalability.

Apology

The authors sincerely apologize for registering and submitting this contribution late and would greatly appreciate its consideration.

# Introduction and problem description

Currently, layer / dimension structure of scalable extension of HEVC (SHEVC) is signalled in the following mechanism, as described in JCTVC-K1007 [1]:

- In NAL unit header, there are 6 bits are allocated to indicate layer_id of each NAL units as shown in the below table. This layer_id does not indicate the layer / dimension types (e.g., spatial scalability / quality scalability / multiview, depth map, etc) of the data in the NAL unit payload.

- In Video Parameter Set Extension (VPS Extension), a mapping mechanism to associate the nuh_layer_id with each of layer / dimension type and their dimension_id is given as shown in the below table.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| scalability_mask | u(16) |
| for( i = 0; i <NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| … |  |
| } |  |

The mapping between layer_id in NAL unit header and detail dimension_id is designed by anticipating that SHEVC bitstreams may contain many types of scalability at once. For example, an encoder may encode a bitstream with multiple layers of spatial scalability and multiple layers of quality scalability in each spatial scalability layers.

However, it is also a common case when there is only one type of scalability used to encode an SHEVC bitstream. In such case, it may not be necessary to signal the mapping of the layer_id and dimension_id. Such signalling would be redundant since the dimension_id can simply be inferred as the same as layer_id.

In the next section, we present two options to modify the current signalling method to avoid signalling the mapping when there is only one type of scalability.

# Proposal

We propose that when the number of scalability type / dimension type used in the coded bitstream is only one, there is no need to signal the mapping and dimension identification can be simply inferred to be the same as layer identification.

To accommodate the proposed concept, the required update to the current signalling can be there are some modifications to the current signalling of the mapping between layer identification and dimension identification in VPS extension. We propose two options for updating current signalling method to accommodate the proposed concept. Out of the two options, the authors preferred option 1 since it requires less modification compared to option 2.

Option 1 – Add a conditional check before signalling the mapping information

In this option, scalability mask is always signalled in order to know how many scalability types exist. Then a condition check is applied to decide whether or not the information for the mapping is needed.

When there is only one type of scalability, a new syntax element called ‘dimension_type’ coded as u(4) is signalled whereas dimension_id_len_minus1, vps_nuh_layer_id_present_flag, layer_id_in_nuh [i], and dimension_id [i][j] are not signalled and can be simply inferred.

The modification to the syntax table and semantic changes are as follow:

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| scalability_mask | u(16) |
| if (NumScalabilityTypes == 1) |  |
| dimension_type | u(4) |
| else { |  |
| for( i = 0; i <NumScalabilityTypes; i++ ) |  |
| dimension_id_len_minus1[ i ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

dimension_type specifies the dimenstion / scalability type of layers of current bitstream. The scalability / dimension type is the following table:

Mapping of ScalabiltyId to scalability dimensions

| dimension_type | Scalability dimension | ScalabiltyId mapping |
| --- | --- | --- |
| 0 | spatial | DependencyId |
| 1 | quality | QualityId |
| 2 | depth | DepthFlag |
| 3 | multiview | ViewId |
| 4 - 15 | Reserved | Reserved |

When NumScalabilityTypes equals 1, the syntax element dimension_id_len_minus1[ i ], vps_nuh_layer_id_present_flag, and dimension_id[ i ][ j ] are not signaled but are inferred as follow:

dimension_id_len_minus1[ 0 ] = 5

vps_nuh_layer_id_present_flag = 0

for (int i = 0; i <= vps_max_layers_minus1; i++)

dimension_id[ i ][ 0 ] = nuh_layer_id

NOTE: semantic of other syntax elements is the same as the semantics in the current specification.

Option 2 – Add a flag to indicate that the bitstream has only one dimension

In this option, scalability mask is not signalled, instead, a new flag called “single_dimension_flag is added. If the flag is set, the number of scalability is set to 1.

When there is only one type of scalability, a new syntax element called ‘dimension_type’ coded as u(4) is signalled whereas dimension_id_len_minus1, vps_nuh_layer_id_present_flag, layer_id_in_nuh [i], and dimension_id [i][j] are not signalled and can be simply inferred.

The modification to the syntax table and semantic changes are as follow:

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| single_dimension_flag | u(1) |
| if (single_dimension_flag) |  |
| dimension_type | u(4) |
| else { |  |
| scalability_mask | u(16) |
| for( i = 0; i <NumScalabilityTypes; i++ ) |  |
| dimension_id_len_minus1[ i ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

single_dimension_flag equals 1 indicates that there is only one type of dimension / scalability in the bitstream.

dimension_type specifies the dimenstion / scalability type of layers of current bitstream. The scalability / dimension type is the following table:

Mapping of ScalabiltyId to scalability dimensions

| dimension_type | Scalability dimension | ScalabiltyId mapping |
| --- | --- | --- |
| 0 | spatial | DependencyId |
| 1 | quality | QualityId |
| 2 | depth | DepthFlag |
| 3 | multiview | ViewId |
| 4 - 15 | Reserved | Reserved |

When NumScalabilityTypes equals 1, the syntax element dimension_id_len_minus1[ i ], vps_nuh_layer_id_present_flag, and dimension_id[ i ][ j ] are not signaled but are inferred as follow:

for (int i = 0; i <16; i++)

scalability_mask_flag [i] = 0

scalability_mask_flag [dimension_type] = 1

dimension_id_len_minus1[ 0 ] = 5

vps_nuh_layer_id_present_flag = 0

for (int i = 0; i <= vps_max_layers_minus1; i++)

dimension_id[ i ][ 0 ] = nuh_layer_id

NOTE: semantic of other syntax elements is the same as the semantics in the current specification.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).