| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0396 |
| --- | --- |

| Title: | TEA1: Cross check for Test 4.4 on chroma up-sampling filter (JCTVC-L0083) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi | Email: | madhukar@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Introduction

This contribution reports cross-check results for JCTVC-L0083 TEA1 4.4 chroma upsampling filter without accurate chroma phase alignment. To support 2x and 1.5x scalability, only the following three phases are used in TEA1 4.4: 1/3, 1/2 and 2/3. This results in use of 3 upsampling filters instead of the 8 used in SMuC. It is reported that there is a match to the BD-Rate results generated by the proponents. Summary of BD-Rate results are provided below.

JCTVC-L0083 TEA1 4.4 v/s SMuC

|  | Y | U | V |
| --- | --- | --- | --- |
| AI HEVC 2x | 0.1% | 0.2% | 0.2% |
| AI HEVC 1.5x | 0.0% | -0.1% | -0.1% |
| RA HEVC 2x | 0.0% | 0.1% | 0.2% |
| RA HEVC 1.5x | 0.0% | -0.1% | -0.1% |
| LD-P HEVC 2x | 0.0% | 0.1% | 0.3% |
| LD-P HEVC 1.5x | 0.0% | 0.0% | -0.1% |

JCTVC-L0083 TEA1 4.4 v/s Simulcast

|  | Y | U | V |
| --- | --- | --- | --- |
| AI HEVC 2x | -22.7% | -22.8% | -22.9% |
| AI HEVC 1.5x | -32.6% | -33.0% | -33.1% |
| RA HEVC 2x | -16.3% | -8.7% | -8.2% |
| RA HEVC 1.5x | -26.1% | -20.3% | -18.8% |
| LD-P HEVC 2x | -12.2% | -5.4% | -4.4% |
| LD-P HEVC 1.5x | -21.9% | -17.0% | -15.3% |