| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0404 |
| --- | --- |

| Title: | AhG6: Cross-check for deblocking filter process and parameter modifications suggested in JCTVC-L0232 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_a.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This is verification of proposed in JCTVC-L0232 normative and non-normative changes for block artifact removal in HM9.1.

# Introduction

If HM9.1 encoder is used then visual quality at very low bit-rates sometimes is very poor, especially for videos with fast and stochastic motion. HEVC supports de-blocking filter parameters signalling in PPS or slice header, but by default this option is disabled in HM s/w. Was to find optimal “non-trivial” settings for de-blocking parameters. This is one of encoder modifications suggested in JCTVC-L0232. These modification follows adopted for HM s/w in Shanghai meeting JCTVC-K0189.

Another modification suggested for blocking artefact reduction is restriction of large Intra CUs in Inter-Slices. This was achieved with only encoder modification.

The third part of verified contribution is normative change of strong de-blocking filter and decision about its application. They are very similar to what was proposed in Stockholm and later in Shanghai. However, the thresholds are rather similar to the current HEVC de-blocking filter, so the difference to  current filter with the zero deblocking parameters are less than at the filter proposed before.

# Test results for verified algorithm.

S/w provided by proponent based on HM9.1 and was tested in all 8 test cases: all-intra, random access, low delay-B and low delay-P for main and HE-10 configurations.

Results for encoder only de-blocking parameter setting according to the picture depth in hierarchical coding structure are reported in Table 1.

Results for modified de-blocking filter parameters with normative modifications for strong filter and decision are summarized in Table 2.

Results for additional penalty for large Intra CUs in Inter slices are summarized in Table 3.

Table 1. Encoder modification for de-blocking parameters only.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class C | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class F | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 90% |  |  | 90% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,3% | 0,2% | -0,1% | 0,2% | 0,4% |
| Class B | -0,1% | 0,2% | 0,3% | -0,1% | 0,3% | 0,3% |
| Class C | 0,0% | 0,4% | 0,5% | -0,1% | 0,4% | 0,5% |
| Class D | 0,1% | 0,3% | 0,6% | 0,1% | 0,6% | 0,5% |
| Class E |  |  |  |  |  |  |
| Overall | 0,0% | 0,3% | 0,4% | 0,0% | 0,4% | 0,4% |
|  | 0,0% | 0,3% | 0,4% | 0,0% | 0,4% | 0,4% |
| Class F | 0,3% | 0,3% | 0,4% | 0,3% | 0,3% | 0,4% |
| Enc Time[%] | 81% |  |  | 81% |  |  |
| Dec Time[%] | 81% |  |  | 82% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,0% | 0,1% | -1,2% | -0,4% | -0,2% |
| Class C | 0,0% | 0,3% | 0,2% | 0,0% | 0,2% | 0,3% |
| Class D | 0,2% | 0,1% | -0,4% | 0,2% | 0,0% | -0,3% |
| Class E | 0,1% | -0,1% | -0,5% | 0,2% | 0,3% | 0,1% |
| Overall | 0,0% | 0,1% | -0,1% | -0,3% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% |
| Class F | 0,5% | 0,0% | 1,0% | 0,5% | 0,2% | 0,0% |
| Enc Time[%] | 82% |  |  | 79% |  |  |
| Dec Time[%] | 83% |  |  | 82% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,3% | 0,0% | 0,1% | -0,4% | 0,1% | 0,1% |
| Class C | -0,1% | 0,2% | 0,3% | -0,2% | 0,3% | 0,2% |
| Class D | 0,0% | 0,1% | 0,4% | 0,0% | 0,6% | 0,5% |
| Class E | -0,2% | 0,0% | -0,5% | -0,1% | 0,3% | 0,4% |
| Overall |  |  |  |  |  |  |
| Class F | 0,3% | 0,1% | 0,4% | 0,4% | 0,6% | 1,9% |
| Enc Time[%] | 86% |  |  | 86% |  |  |
| Dec Time[%] | 85% |  |  | 86% |  |  |

Table 2. Additionally de-blocking filter is modified (normative change).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class C | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | -0,1% | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class F | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 89% |  |  | 90% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,4% | 2,6% | 3,3% | 0,3% | 2,6% | 3,7% |
| Class B | -0,1% | 0,3% | 0,3% | 0,3% | 0,9% | 0,9% |
| Class C | 0,0% | 0,4% | 0,5% | 0,0% | 0,4% | 0,4% |
| Class D | 0,1% | 0,3% | 0,7% | 0,1% | 0,6% | 0,5% |
| Class E |  |  |  |  |  |  |
| Overall | 0,1% | 0,8% | 1,1% | 0,2% | 1,1% | 1,3% |
|  | 0,1% | 1,0% | 1,5% | 0,3% | 1,2% | 1,7% |
| Class F | 0,2% | 0,2% | 0,3% | 0,2% | 0,3% | 0,3% |
| Enc Time[%] | 80% |  |  | 77% |  |  |
| Dec Time[%] | 78% |  |  | 77% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1,1% | -0,5% | 0,1% | -1,7% | -1,4% | -1,1% |
| Class C | 0,1% | 0,2% | 0,0% | 0,4% | 0,6% | 0,4% |
| Class D | 0,1% | 0,2% | 0,2% | 0,3% | 0,2% | 0,0% |
| Class E | 0,5% | 0,1% | -0,4% | 1,8% | 1,4% | 1,3% |
| Overall | -0,2% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | -0,1% | 0,0% | 0,1% | 0,1% | 0,1% |
| Class F | 0,5% | 0,4% | 1,1% | 0,8% | 0,1% | 0,8% |
| Enc Time[%] | 79% |  |  | 76% |  |  |
| Dec Time[%] | 80% |  |  | 78% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,5% | 0,1% | 0,5% | -1,6% | -0,1% | -0,3% |
| Class C | -0,1% | 0,3% | 0,3% | 0,0% | 0,3% | 0,4% |
| Class D | 0,0% | 0,0% | 0,2% | 0,0% | 0,4% | 0,2% |
| Class E | -0,2% | -0,4% | -0,2% | 0,0% | 0,5% | 0,6% |
| Overall |  |  |  |  |  |  |
| Class F | 0,3% | 0,5% | 0,7% | 0,4% | 0,5% | 1,6% |
| Enc Time[%] | 86% |  |  | 83% |  |  |
| Dec Time[%] | 83% |  |  | 82% |  |  |

Table 3. Additionally large CUs are restricted in Inter slices.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class C | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | -0,1% | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class F | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 89% |  |  | 90% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1,2% | 5,6% | 6,4% | 1,2% | 5,7% | 6,2% |
| Class B | 0,2% | 1,0% | 1,0% | -0,6% | 0,4% | 0,8% |
| Class C | 0,1% | 0,5% | 0,5% | 0,1% | 0,6% | 0,5% |
| Class D | 0,1% | 0,4% | 0,7% | 0,2% | 0,7% | 0,6% |
| Class E |  |  |  |  |  |  |
| Overall | 0,4% | 1,8% | 2,1% | 0,2% | 1,8% | 2,0% |
|  | 0,4% | 1,9% | 2,4% | 0,3% | 1,9% | 2,3% |
| Class F | 0,4% | 0,7% | 0,8% | 0,5% | 0,6% | 0,7% |
| Enc Time[%] | 80% |  |  | 78% |  |  |
| Dec Time[%] | 78% |  |  | 76% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1,9% | -1,2% | -0,8% | -1,1% | -0,1% | 0,2% |
| Class C | 0,2% | 0,3% | 0,3% | 0,4% | 0,5% | 0,5% |
| Class D | 0,2% | -0,2% | 0,1% | 0,3% | 0,4% | 0,4% |
| Class E | 0,6% | 0,6% | 0,2% | 1,8% | 1,4% | 1,3% |
| Overall | -0,4% | -0,2% | -0,1% | 0,2% | 0,5% | 0,5% |
|  | -0,2% | -0,2% | 0,0% | 0,3% | 0,5% | 0,6% |
| Class F | 0,7% | 0,5% | 1,4% | 1,0% | 0,4% | 1,1% |
| Enc Time[%] | 77% |  |  | 74% |  |  |
| Dec Time[%] | 76% |  |  | 75% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1,2% | 0,5% | 1,1% | -2,4% | -0,6% | 0,1% |
| Class C | 0,0% | 0,4% | 0,5% | 0,1% | 0,4% | 0,6% |
| Class D | 0,0% | 0,6% | 0,3% | 0,0% | 0,4% | 0,2% |
| Class E | -0,1% | 0,7% | 0,6% | 0,0% | 0,9% | 0,6% |
| Overall |  |  |  |  |  |  |
| Class F | 0,5% | 1,0% | 1,2% | 0,9% | 1,6% | 3,0% |
| Enc Time[%] | 84% |  |  | 81% |  |  |
| Dec Time[%] | 80% |  |  | 79% |  |  |

Time measurement is not reliable: different version of compiler were used for anchor and tests.

Tests 1-3 also were performed for additional video sequences previously tested in CE1. Performance of additional tests us included into excel tables attached to this report.

Due to the lack of time visual quality was evaluated for only one test sequence WestWindEasy_p1920x1080_30fps_420pl_300fr.yuv in Random Access test, QP=37.

Fragments of video encoded with different variants of HM s/w and suggested modification from JCTVC-L0232 are show below. In following order (from left to right):

- HM9.1

- encode modification for de-blocking parameters only,

- encode modification for de-blocking parameters and Intra CUs restrictions (non-normative),

- Additionally strong de-blocking filter and decisions are modified.

+Non-normative 1 11

+Non-normative 2

+Normative

HM9.1

POC=2

POC=4

POC=7

# Conclusion

Objective performance of proposed in JCTVC-L0232 normative and non-normative changes for block artifact removal in HM9.1 is verified here.