| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0405 |
| --- | --- |

| Title: | Non-TEC5: Cross-check for Inter-layer Intra mode prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Tammy Lee JeongHoon Park 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, tammy.lee, jeonghoon}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This is verification of non-TEC5 contribution from LG and MediaTek (JCTVC-L0260). In verified contribution both 3 predictors for Intra Direction on enhancement layer are borrowed from base layer. In average 0.3% Luma BD-rate gain is verified in all-intra tests.  If parsing dependency is resolved by disabling mode-dependent scan in IntraBL blocks then an average gain is 0.2%.

# Introduction

The first part of TEC5 is devoted to inter-layer prediction for Intra direction. Extreme case when both 3MPMs are borrowed from base layer was investigated in LG&Mediatek’s non-TEC5 proposal. For central position in a block collocated BL block is derived. Intra prediction direction in this block is first MPM (uiIntraDirPred[0] ). Second and third are derived as follows:

uiIntraDirPred[1] = ((uiIntraDirPred[0] + 29) % 32) + 2;

uiIntraDirPred[2] = ((uiIntraDirPred[0] - 1 ) % 32) + 2;

Additionally to this modification MDCS was disabled on EL in order to resolve parsing dependency issue.

# Test results for verified algorithm.

Verified algorithm was tested vs SMuC 0.1.1 s/w in All Intra (x2, x1.5) tests only. Simulation results for original proposal and  w/o parsing dependency are summarised in Tables 1-2 below.

Table 1. Summation results for 3 MPMs from base layer.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0,3% | -0,1% | 0,0% |  |  |  |
| Class B | -0,4% | -0,2% | -0,2% | -0,2% | 0,1% | 0,1% |
| Overall (EL+BL) | -0,4% | -0,2% | -0,2% | -0,2% | 0,1% | 0,1% |
| Overall (EL) | -0,7% | -0,2% | -0,2% | -0,4% | 0,3% | 0,2% |
| Enc Time[%] | 103,7% |  |  | 103,1% |  |  |
| Dec Time[%] | 100,6% |  |  | 99,6% |  |  |
| Enc Mem[%] | 100,0% |  |  | 99,7% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 2. Summation results for 3 MPMs from base layer (w/o MDSC).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0,2% | 0,0% | 0,1% |  |  |  |
| Class B | -0,3% | 0,0% | 0,1% | -0,1% | 0,1% | 0,1% |
| Overall (EL+BL) | -0,3% | 0,0% | 0,1% | -0,1% | 0,1% | 0,1% |
| Overall (EL) | -0,5% | 0,0% | 0,2% | -0,4% | 0,3% | 0,4% |
| Enc Time[%] | 103,5% |  |  | 102,9% |  |  |
| Dec Time[%] | 100,7% |  |  | 100,1% |  |  |
| Enc Mem[%] | 100,1% |  |  | 100,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusion

The performance of inter-layer prediction for Intra direction is verified in this report. If both 3 MPMs are borrowed from base layer then Luma BD-rate gain is 0.3% (in all-intra tests). If additionally MDSC is disabled for enhancement layer then average Luma BD-rate gain is 0.2%.