| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0407 |
| --- | --- |

| Title: | Cross-check of Non-CE1: Square transform blocks for 4:2:2 (L0351) by Qualcomm and the BBC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of Qualcomm and BBC proposal JCTVC-L0351 for square transform blocks for 4:2:2, as a non-CE1 contribution.  The source code was studied and found to match the description in the contribution document.  The simulation result for all configurations were reported by Qualcomm and the BBC and tested by Canon.  The cross check result confirms the simulation result reported by Qualcomm and the BBC.

# Cross Check Procedure

This is a cross-check of the contribution relating to ‘Test 3’ of Core Experiment 1 on transforms for Range Extensions [1].

The source code for JCTVC-L0351 [2] was provided by Qualcomm and the BBC.  Canon compiled the source code using GCC 3.4.6 and the simulation was performed on a Linux cluster under Range Extensions common test conditions [3]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

Method 1 only:

- Adjustment of the intra-prediction direction for rectangular blocks is removed.

Method 1 and 2:

- QP adaptation for rectangular transforms was removed.

- The rectangular chroma blocks of 4:2:2 are processed using two square transforms. Separate coded block flags are residual coding are used for each square transform.

- Intra-prediction is performed separately for each square block in chroma, with neighbouring samples determined separately for each square block.

- Each 4x4 chroma block resulting from a 4x8 region split may have independent values of transform skip flag.

Method 2 only:

- The absolute value of the intra-prediction angle is clamped at +/- 32 and when clamping occurs, the ‘invAngle’ is clamped at 256.

This is consistent with the description in [2].

# Coding Performance Measurement

Table 1 and Table 2 provide the cross-check result for methods 1 and 2 of JCTVC-L0351, comparing this case with the CE1 Test 1 result as reference.  The simulation result matches the result provided by Qualcomm and the BBC.

Table 1. JCTVC-L0351 Method 1 simulation result compared with CE1 Test 1 simulation result.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.8% | 3.6% | 5.0% | 0.6% | 1.9% | 2.3% |
| Overall | 0.3% | 1.1% | 1.6% | 0.2% | 0.6% | 0.7% |
|  | 0.3% | 1.1% | 1.6% | 0.2% | 0.7% | 0.8% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.4% | 4.5% | 5.6% | 0.3% | 2.4% | 3.6% |
| Overall | 0.1% | 1.4% | 1.8% | 0.1% | 0.8% | 1.2% |
|  | 0.1% | 1.4% | 1.8% | 0.1% | 0.6% | 1.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.2% | 4.0% | 5.0% | 0.0% | 2.3% | 3.4% |
| Overall | 0.0% | 1.3% | 1.6% | 0.0% | 0.7% | 1.1% |
|  | 0.0% | 1.3% | 1.6% | 0.0% | 0.5% | 1.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 2. JCTVC-L0351 Method 2 simulation result compared with CE1 Test 1 simulation result (RA and LD results included in r1).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.2% | 2.9% | 4.3% | 0.1% | 1.5% | 1.9% |
| Overall | 0.1% | 0.9% | 1.4% | 0.0% | 0.5% | 0.6% |
|  | 0.1% | 0.9% | 1.4% | 0.0% | 0.5% | 0.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.2% | 3.8% | 5.0% | 0.0% | 2.1% | 3.3% |
| Overall | 0.0% | 1.2% | 1.6% | 0.0% | 0.7% | 1.1% |
|  | 0.0% | 1.2% | 1.6% | 0.0% | 0.5% | 1.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 3.5% | 4.6% | -0.1% | 2.2% | 3.2% |
| Overall | 0.0% | 1.1% | 1.5% | 0.0% | 0.7% | 1.0% |
|  | 0.0% | 1.1% | 1.4% | 0.0% | 0.4% | 1.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Conclusion

This cross-check includes results for JCTVC-L0351 source code provided by Qualcomm and the BBC. The source code implements the design as described in the contribution and the cross-check result confirms the simulation result reported by Qualcomm and the BBC.

# References

- J. Sole, “HEVC Range Extensions Core Experiment 1: Square and non-square transforms for 4:2:2 chroma format coding”, JCTVC-K1121, 11th JCT-VC Meeting, Shanghai, China, Oct 2012.

- J. Sole, R. Joshi, M. Karczewicz, A. Gabriellini, M. Mrak, “Non-CE1: Square transform blocks for 4:2:2”, JCTVC-L0351, 12th JCT-VC Meeting, Geneva, CH, Jan 2013.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-K1006, 11th JCT-VC Meeting, Shanghai, China, Oct 2012.