| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0408 |
| --- | --- |

| Title: | Cross-check of JCTVC-L0220 on Inter-layer Intra MPM derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Edouard François Canon Research France | Tel: Email: | Edouard.francois@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This document provides the cross-check results of proposal JCTVC-L0220 related to Inter-layer Intra MPM derivation for EL in SHVC. All the BD-rate results are reported to match exactly those provided by the proponent.

# Proposed approach

The solution proposes a modified MPM derivation process using the mode from the BL. If the left/top CU is IntraBL and the co-located BL CU is intra, the intra mode from the co-located BL CU replaces the left/top CU mode for the MPM derivation. Otherwise the DC mode is used.

# Cross-checking description

Two tests have been made, with or without MDCS. The correspionding software settings are:

MDCS Off

#define SONY_BL_NEIGHBOR          1

#define SONY_EL_MDCS_OFF          1

MDCS On

#define SONY_BL_NEIGHBOR          1

#define SONY_EL_MDCS_OFF          0

The source code was compiled successfully using windows32, VC2008 and using Linux 2.6.18-194.el5, gcc version 4.1.2 20080704 (Red Hat 4.1.2-48) 64 bits.

The full cross-check results are in the attached xls files.

# Conclusion

The results of JCTVC-L0220 presented by Sony are confirmed by the cross-checker.