| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0411 |
| --- | --- |

| Title: | Non-TE5: Cross-check of Inter-layer motion data inheritance by Intel(JCTVC-L0291) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liwei Guo Xiang Li 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution verified the methods and the BD-rate results of JCTVC-L0291 which is proposed by Intel.

# Introduction

JCTVC-L0291 describes inter-layer motion data inheritance method, where a CU can be divided into 4 NxN units. For each NxN unit, the motion information from the corresponding positions at base layer is used for its inter-prediction.  Method 1 allows a 8x8 CU to be divided into 4 4x4 units and method 2 disabled this case.

# Experimental results

The JCTVC-L0291 software provided by the proponents was used to generate the results and compared with anchor results generated with SMuC-0.1.1 software. Test coditions defined in TE5 [2] were used. Encoding time and decoding time were obtained using homogeneous cluster. It was confirmed that for both Method1 and Method2, the BD-rate results exactly match to the results provided by the proponents.

Table 1 Results of Method 1

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -2.5% | -5.4% | -5.2% |  |  |  |
| Class B | -2.0% | -3.5% | -3.6% | -2.2% | -3.9% | -4.1% |
| Overall (EL+BL) | -2.1% | -4.0% | -4.1% | -2.2% | -3.9% | -4.1% |
| Overall (EL) | -4.1% | -7.6% | -7.6% | -5.8% | -9.8% | -10.4% |
| Enc Time[%] | 97.6% |  |  | 96.0% |  |  |
| Dec Time[%] | 98.3% |  |  | 98.2% |  |  |
| Enc Mem[%] | 100.3% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -1.6% | -3.2% | -3.1% |  |  |  |
| Class B | -1.1% | -2.0% | -2.0% | -1.2% | -2.1% | -2.2% |
| Overall (EL+BL) | -1.2% | -2.4% | -2.3% | -1.2% | -2.1% | -2.2% |
| Overall (EL) | -2.2% | -4.3% | -4.2% | -2.7% | -4.7% | -4.8% |
| Enc Time[%] | 99.3% |  |  | 97.2% |  |  |
| Dec Time[%] | 98.4% |  |  | 98.9% |  |  |
| Enc Mem[%] | 100.3% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 2 Results of Method 2

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -2.5% | -5.4% | -5.2% |  |  |  |
| Class B | -2.0% | -3.5% | -3.6% | -2.1% | -3.6% | -3.9% |
| Overall (EL+BL) | -2.1% | -4.0% | -4.1% | -2.1% | -3.6% | -3.9% |
| Overall (EL) | -4.1% | -7.6% | -7.6% | -5.5% | -9.4% | -9.9% |
| Enc Time[%] | 96.3% |  |  | 95.1% |  |  |
| Dec Time[%] | 97.4% |  |  | 98.3% |  |  |
| Enc Mem[%] | 100.3% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -1.6% | -3.2% | -3.1% |  |  |  |
| Class B | -1.1% | -2.0% | -2.0% | -1.1% | -1.9% | -2.0% |
| Overall (EL+BL) | -1.2% | -2.4% | -2.3% | -1.1% | -1.9% | -2.0% |
| Overall (EL) | -2.2% | -4.3% | -4.2% | -2.5% | -4.2% | -4.4% |
| Enc Time[%] | 97.3% |  |  | 95.8% |  |  |
| Dec Time[%] | 97.6% |  |  | 99.0% |  |  |
| Enc Mem[%] | 100.3% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# References

- Z. Deng, W. Zhang, L. Xu, Y. Han, X. Cai, Y. Chiu (Intel), “Non-TE C5: Inter-layer motion data inheritance”, JCTVC-L0291, Geneva, CH, Jan., 2013.

- V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, and H. Laksman, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Shanghai, China, Oct. 2012.