| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0421 |
| --- | --- |

| Title: | Crosscheck of JCTVC-L202 on Sign Data Hiding for Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Félix Henry Gordon Clare Orange Labs 4 rue Clos Courtel 35510 Cesson Sevigne, France | Tel: Email: | +33 2 99 12 45 58 felix.henry@orange.com gordon.clare@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This contribution reports the crosschecking of contribution JCTVC-L0202 that proposes to have independent syntax elements to activate Sign Data Hiding on luma and chroma. The software has been examined and matches the description of the proponents. The software has been run and its performance perfectly matches the results provided by the proponents.

# Description

HEVC currently uses a technique called Sign Data Hiding (SDH) [1][2]. This consists in inferring the sign bit of the first non-zero coefficient of each coefficient group from the parity of the sum of the coefficient. Sign Data Hiding is activated by a syntax element called sign_data_hiding_flag at PPS level. The proponents suggest that this syntax element is replaced by two syntax elements that activate Sign Data Hiding independently for luma and chroma, on the grounds that depending on the encoder choice (RDOQ on/off), there are always compression gains on luma for SDH, but not always on chroma [3].

# Software examination

The software provided by the proponents has been examined and appears to match the description in [3].

# Compression performance

The software has been run using Common Testing Conditions [4]. The anchor is HEVC with SDH active for all components, and results are provided for SDH off in luma and on in chroma (Table 1), and SDH on in luma and off in chroma (Table 2). The results match exactly those reported by the proponents. Encoding and decoding runtime are not reported.

Table 1: Measured performance with luma SDH off and chroma SDH on

Table 1: Measured performance with luma SDH on and chroma SDH off

# Bibliography

[1] JCTVC-H0224, CE11: Sign Data Hiding, Gordon Clare, Felix Henry, Joel Jung, Stephane Pateux

[2] JCTVC-H0481, Multiple Sign Bits Hiding, X. Yu, J. Wang, D. He, G. Martin-Cocher, S. Campbell

[3] JCTVC-L0202, Sign data hiding flag for chroma, J. Sole, M. Karczewicz

[4] JCTVC-K1100, Common HM test conditions and software reference configurations, F. Bossen