| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0428 WG11 Number: m28239 |
| --- | --- |

| Title: | AHG8: Impact of transform skip on new screen content material |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Anthony Vetro Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

At the previous meeting, JCTVC-K0246 showed that the BD-Rate performance of transform skip, when applied to the 4:4:4 sequences used for testing HEVC range extensions, was 0.0%. That document recommended that JCT-VC obtain additional professional-quality content that contains realistic screen-content/graphics material. It was also shown in JCTVC-K0368 that the BD-Rate performance of transform skip on screen-content sequences proposed in JCTVC-K0207 ranged from 23.9% to 35.1%. At the current meeting, new screen-content material has been proposed in JCTVC-L0301 and JCTVC-L0317. The purpose of this document is to report the BD-Rate performance of transform skip on these new sequences. For the set of new 8-bit YCbCr 4:4:4 sequences coded using the common test conditions and software reference for the HEVC range extensions, the BD-Rate change for disabling transform skip for all-intra conditions ranged from 5.5% to 46.1%, with an average of 25.9%. BD-Rate changes for random access and low-delay B averaged to 23.5% and 20.2% respectively.

# Introduction

At the previous meeting JCTVC-K0246 [1] showed that the BD-Rate performance of transform skip, when applied to the 4:4:4 sequences used for testing HEVC range extensions, was 0.0%. That document recommended that JCT-VC obtain additional professional-quality content that contains realistic screen-content/graphics material. It was also shown in JCTVC-K0368 [2] that the BD-Rate performance of transform skip on screen-content sequences proposed in JCTVC-K0207 [3] ranged from 23.9% to 35.1%. At the current meeting, new screen-content material has been proposed in JCTVC-L0301 [4] and JCTVC-L0317 [5]. The purpose of this document is to show whether the transform skip tool is exercised by these new sequences.

# Experimental conditions

Six sequences from JCTVC-L0317 and three sequences from JCTVC-L0301 were tested. All sequences were in the 8-bit YCbCr 4:4:4 format, having a duration of 10 seconds:

sc_cad_waveform_1920x1080_20_200_8bit_444.yuv

sc_pcb_layout_1920x1080_20_200_8bit_444.yuv

sc_ppt_doc_xls_1920x1080_20_200_8bit_444.yuv

sc_video_conferencing_doc_sharing_1280x720_30_300_8bit_444.yuv

sc_web_browsing_1280x720_30_300_8bit_444.yuv

sc_cg_twist_tunnel_1280x720_30_8bit_444.yuv

sc_wordEditing_1280x720_60_8bit_444.yuv

sc_map_1280x720_60_8bit_444.yuv

sc_programming_1280x720_60_8bit_444.yuv

For the reference, the common test conditions and software reference configurations for HEVC range extensions were used [6]. The software version was revision 3055 of the common range-extension software. The QP ranges were 22, 27, 32, and 37, as specified in the High-efficiency main-tier (HEMT) test conditions. The tested configuration used the same parameters, except that transform skip was disabled (TransformSkip=0 and TransformSkipFast=0 in the encoder).

# Experimental results

BD-Rate changes for disabling transform skip are shown in the tables below:

|  | AI-HEMT |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| sc_cad_waveform | 34.8% | 37.0% | 39.7% |
| sc_pcb_layout | 46.1% | 44.4% | 45.7% |
| sc_ppt_doc_xls | 33.3% | 43.6% | 35.4% |
| sc_video_conf. | 29.9% | 30.9% | 33.0% |
| sc_web_browsing | 12.2% | 23.7% | 19.1% |
| sc_cg_twist_tunnel | 39.0% | 35.8% | 37.0% |
| sc_wordEditing | 14.2% | 12.4% | 13.3% |
| sc_map | 5.5% | 7.3% | 11.2% |
| sc_programming | 18.2% | 13.8% | 14.0% |
| Overall | 25.9% | 27.6% | 27.6% |

|  | RA-HEMT |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| sc_cad_waveform | 33.9% | 36.9% | 38.9% |
| sc_pcb_layout | 46.6% | 47.8% | 46.1% |
| sc_ppt_doc_xls | 30.7% | 42.3% | 39.9% |
| sc_video_conf. | 29.4% | 33.3% | 32.8% |
| sc_web_browsing | 11.0% | 22.1% | 20.8% |
| sc_cg_twist_tunnel | 34.6% | 27.5% | 28.9% |
| sc_wordEditing | 9.7% | 9.8% | 10.8% |
| sc_map | 4.7% | 6.4% | 10.4% |
| sc_programming | 10.8% | 8.6% | 10.0% |
| Overall | 23.5% | 26.1% | 26.5% |

|  | LB-HEMT |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| sc_cad_waveform | 31.1% | 33.0% | 33.4% |
| sc_pcb_layout | 36.9% | 38.1% | 35.3% |
| sc_ppt_doc_xls | 24.4% | 36.6% | 33.2% |
| sc_video_conf. | 23.0% | 27.1% | 28.9% |
| sc_web_browsing | 8.0% | 15.9% | 15.4% |
| sc_cg_twist_tunnel | 35.1% | 30.3% | 32.9% |
| sc_wordEditing | 10.5% | 8.4% | 10.1% |
| sc_map | 4.1% | 5.9% | 10.7% |
| sc_programming | 8.3% | 6.2% | 7.0% |
| Overall | 20.2% | 22.4% | 23.0% |

# Discussion and conclusions

For these 9 new screen-content sequences, the effects of transform skip on BD-Rate ranged from 5.5% to 46.1%, with an average of 25.9% for all-intra conditions. BD-Rate changes for random access and low-delay B averaged to 23.5% and 20.2% respectively. This set of sequences would therefore be a good complement to the existing set of material being used for range-extension testing.

# References

- R. Cohen and A. Vetro, “AHG7: The need for screen-content/graphics test material for extended chroma formats,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K0246, Shanghai, CN, October 2012.

- R. Cohen and T. Lin, “Impact of transform skip on screen content sequences from JCTVC-K0207,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K0368, Shanghai, CN, October 2012.

- S. Wang, T. Lin, and K. Zhou, “AHG7: Update on full-chroma (YUV444) screen content test sequences of JCTVC-H0294,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K0207, Shanghai, CN, October 2012.

- H. Yu (Huawei), R. Cohen (MERL), W. Gao, Y. Cao, J. Ye, X. Wang, A. Vetro, and H. Sun, “AHG8: New 4:4:4 screen-content sequences for HEVC extension development,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-L0301, Geneva, CH, January 2013.

- S. Wang, M. Jiao, T. Lin, K. Zhou, “AHG8: YUV444 and RGB screen content test sequences,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-L0317, Geneva, CH, January 2013.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K1006, Shanghai, CN, October 2012.