| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0430 |
| --- | --- |

| Title: | BoG report on subjective viewing test for deblocking filter proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Kenneth Andersson | Email: | andrey.norkin@ericsson.com kenneth.r.andersson@ericsson.com |
| Source: | BoG on subjective viewing |  |  |

_____________________________

# Abstract

This contribution is a report on informal subjective viewing for the deblocking filtering that was held during the Geneva meeting on January 15, 2013. The goal of informal subjective viewing test was to determine if the proposal in AHG6 help reducing block artifacts on problematic sequences. In total, five different combinations were evaluated.

# Test procedure

Five combinations were tested. The proposals were based on HM9.1. The proposals in each test session were evaluated by a group of 3 experts (two experts participated in session 2). In total, seven viewing sessions have been held. The identities of the proposals and the anchor were hidden (the test subjects were shown labels A or B instead).

The following six sequences have been used in the test.

- Riverbed, Qp=32, RA.

- Riverbed, Qp=37, RA

- WestWindEasy, Qp=37, LDB

- DucksTakeOff, Qp=37, LDB

- ChinaSpeed, Qp=37, LDB

- RedKayak, Qp=37, RA, first 10 seconds.

Anchor: HM9.0 in common test conditions.

ABAB test was used. The same sequence and the anchor were shown one after another. The order of proposals was randomized for every test and sequence and identities of the proposals are hidden. Test subjects were asked to rate each proposal on a scale from -2 to 2. The results were later calculated as well as 95-percent confidence intervals for every proposal and test sequence.

The length of one session was about 25 minutes. In total, 20 test subjects participated in scoring. The test sessions were conducted by Andrey Norkin and Kenneth Andersson. Processing of score sheets and calculations of the results were done by Andrey Norkin and Kenneth Andersson (Ericsson). Processing of score sheets was cross-verified by Geert Van Der Auwera (Qualcomm) and Matthias Narroschke (Panasonic).

# Test equipment

Equipment provided by meeting organizers was used. Dell monitor with 30'' diagonal was used in the subjective tests.

# Proposals

JCTVC-L0386 (Qualcomm)

Beta and tc offsets are estimated in the encoder for each frame based on the blockiness which analyzes the vertical and horizontal edges of the 32x32 block grid in the picture. The following condition is checked at each line i perpendicular to the edge:

dpi=p2i−2p1i+p0idqi=q0i−2q1i+q2idi=dpi+dqi

2<2di<β4

If the condition is true for line i, then the following step size is computed for line i and accumulated for the entire vertical/horizontal edge that is being analyzed:

p0i−q0i

The sum for the current horizontal or vertical grid edge is multiplied by 1024 and normalized by the number of horizontal edges and picture width, or the number of vertical edges and the picture height, respectively. Subsequently, both sums are averaged and this average is compared with a threshold value, for example 2048 in the provided source code. If the average (avg) is greater than the threshold, then tc and beta offset values are computed to control the deblocking filter as follows:

Tc_offset_div2 = Clip3(2,6,avg>>9)

Beta_offset_div2 = Clip3(2,6,avg>>9)

These offsets are signalled to the decoder per slice (=picture).

JCTVC-L0386 (Qualcomm)

1. Signaling tc_offsets depending on the picture position in the GOP (config file modification)

The proposed deblocking parameters are listed below. First for low delay configurations:

#     Type POC QPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2

Frame1:  B    1   3        0.4624   5            0

Frame2:  B    2   2        0.4624   3            0

Frame3:  B    3   3        0.4624   5            0

Frame4:  B    4   1        0.578    1            0

and for random access:

#      Type POC QPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2

Frame1:  B    8   1        0.442    1              0

Frame2:  B    4   2        0.3536   3              0

Frame3:  B    2   3        0.3536   4              0

Frame4:  B    1   4        0.68     6              0

Frame5:  B    3   4        0.68     6              0

Frame6:  B    6   3        0.3536   4              0

Frame7:  B    5   4        0.68     6              0

Frame8:  B    7   4        0.68     6              0

2. Modification of strong filtering decisions and the strong filter

2*dpqi < (β>>2)			          (4)    Same as in HM9.0

(|p3i - p0i| + |q0i – q3i|) < (β>>3)         (5)

dp0 = ((| p3,i − 2*p2,i + p1,i | + | q3,i − 2*q2,i + q1,i |) <<1) < (β>>2)  	(5)

// remove a condition that 4 samples from the block boundary have the same level

// instead, add a condition that 4 pixels //from the block boundary form a straight line (flat or inclined),

|p0i – q0i| < ((5*tC + 1)>>1)  		      (6) // same as in HM9.0

The proposed decisions are explained in Figure 1. The proposed decisions enable applying strong filtering also when both sides of the block boundary have a form of ramp (similar to the normal filtering mode).

| (a) | (b) |
| --- | --- |

Figure 1 (a) Proposed decisions for strong filtering. Two checks are included: that pixel values q0, q1, q2 and q3 form a line (the same for p0, p1, p2 and p3) and that | p0 – q0 | is below threshold. (b) Block artifact from Riverbed. Filter decision matches the artifact.

The effect of applying HEVC strong filter and the proposed filter to a ramp is shown in Figure 2. One can see from the figure that HEVC filter significantly disports the ramp when applied to it, while the proposed filter modified the ramp much less.

| - Effect of HEVC strong filter applied to a ramp | - Effect of proposed strong filter applied to a ramp |
| --- | --- |

Figure 2 Comparison of modification of ramp by HEVC filer and the proposed filter. The original signal is shown in blue, the signal after modification by a filter is shown in red.  The HEVC filter (a) modifies the ramp stronger than the proposed filter (b).

3. Rate distortion penalty on the 32x32 intra predicted TU inter-slices

A cost penalty is introduced to 32x32 intra TU in inter slices.

The following combinations have been tested in the subjective viewing test:

HM9.1 + tc_offsets  (L0232_tc)

HM9.1 + tc_offsets + strong filter (L0232_tc+filt)

HM9.1 + tc_offsets + RD_penalty on Intra 32x32 TU in inter slices (L0232_tc+RDpen)

HM9.1 + tc_offsets + RD_penalty on Intra 32x32 TU in inter slices + strong filter (L0232_tc+RDpen+filt)

# Bitrates of the tested sequences

| Proposal | Riverbed, Qp=32, RA | Riverbed, Qp=37, RA | WestWindEasy,Qp37, LDB | DucksTakeOff, Qp=37, LDB | ChinaSpeed, Qp=37, LDB | RedKayak, Qp=37, RA |
| --- | --- | --- | --- | --- | --- | --- |
| HM9.0 | 4778.61 | 2359.65 | 1673.29 | 5115.48 | 613.65 | 1721.03 |
| L0386 | 4775.75 | 2357.84 | 1680.40 | 5102.39 | 614.20 | 1721.14 |
| L0232_tc | 4774.92 | 2359.13 | 1674.83 | 5103.77 | 614.73 | 1720.67 |
| L0232_tc+filt | 4775.86 | 2357.02 | 1673.29 | 5102.36 | 613.65 | 1720.91 |
| L0232_tc+RDpen | 4940.95 | 2440.12 | 1690.53 | 5111.22 | 618.51 | 1733.91 |
| L0232_tc+RDpen+filt | 4940.92 | 2437.36 | 1685.22 | 5108.21 | 618.06 | 1732.84 |

# Test results

The graphs below show the test results. Values with overlapping CI values cannot be rated as distinguishable.

# References

- T. Yamakage, A. Norkin, "JCT-VC AHG report: HEVC in-loop filtering (AHG6)", JCTVC-L0006, Geneva, Jan. 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-K1100, Shanghai, Oct. 2012.