| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0434 |
| --- | --- |

| Title: | Non-TE4: Crosscheck of supplementary data in JCTVC-L0195 on Switchable De-ringing Filter for Inter-layer Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | PoLin Lai, and Shan Liu MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA | Email: | polin.lai@mediatek.com shan.liu@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document provides cross-check results for the supplementary data in Non-TE4 JCTVC-L0195 on switchable inter-layer de-ringing filter (SDRF) for inter-layer texture prediction. The cross-checked supplementary data is produced by testing the combined effect of SDRF together with the “skip techniques” proposed in JCTVC-0230. The results match with the proponents data.

# Introduction

JCTVC-L0195 [1] proposes switchable inter-layer de-ringing filtering techniques (SDRF) for SHVC. Besides providing results of the technique alone, it also includes supplementary data in which the proposed SDRF is used together with the “skip techniques” proposed in JCTVC-0230 [2]. In this document, the supplementary data is cross-checked.

# Experimental Results

The cross-check was conducted using Linux cluster, with roughly homogenous configurations across different machines. The cross-check results exactly match to those provided by the proponents. Runtime also has very similar trend as reported in JCTVC-0195.

Study findings:

- The proposed inter-layer de-ringing filter is basically a bi-lateral filtering approach [3].

- For an Intra-BL coded block, a flag is utilized to indicate whether the upsampled BL has to undergo de-ringing filter or not, thus achieving “switchable DRF”.

- The supplementary test is a straightforward combination of the proposed SDRF and the “skip techniques” proposed in JCTVC-0230.

# Reference

[1] Z. Ma and F. Fernandes, “Non-TE4: Switchable de-ringing filter for inter-layer prediction,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0195, Jan. 2013.

[2] D.-K. Kwon, M. Budagavi, and M. Zhou, “Non-TE2: Improved inter-layer texture prediction signaling,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0230, Jan. 2013.

[3] C. Tomasi and R. Manduchi, “Bilateral filtering for gray and color images,” In proc. IEEE International Conference on Computer Vision, pp.839-846, 1998.