| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0437 |
| --- | --- |

| Title: | BoG Report on Test Model Configurations of Intra_BL Method and Ref_idx Method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | Liwei Guo | Email: | liweig@qti.qualcomm.com |
| Source: | BoG on TE2 |  |  |

_____________________________

# Abstract

TE2 inter-layer texture prediction signaling includes 3 subsets: Intra-BL based approach, RefIdx based approach and skipped enhancement layer slice. Different methods were tested for each subtest. The purpose of this BoG is to recommend the test model configurations for Intra-BL approach and the test model configurations for RefIdx approach.

# List of Input Document

Subset 1: Intra-BL approach

| Test | Proposal No. | Proposal Title | Cross-checker |
| --- | --- | --- | --- |
| 3.1.1 (Qualcomm) | JCTVC-L0284 | TE2: Results of Test 3.1.1 on Inter-layer Texture Prediction Signaling | ETRI (JCTVC-L0061) Huawei (JCTVC-L0295) |
| 3.1.3 (Huawei) | JCTVC-L0165 | TE2: Results of test 3.1.3 on CU based IntraBL signaling | KDDI(JCTVC-L0388) |
| 3.1.4 (Huawei) | JCTVC-L0164 | TE2: Results of test 3.1.4 on CU based IntraBL signaling with skip mode | InterDigital (JCTVC-L0214) |
| 3.1.5 (Intel) | JCTVC-L0285 | TE A2: Signalling of Inter-layer Skip and Direct | Sharp (JCTVC-L0285) |
| 3.1.7 (MediaTek&LG) | JCTVC-L0069 | TE2-3.1.7: Inter-layer intra prediction signalling with residual skip flag | KDDI(JCTVC-L0387) |
| 3.1.8 (Nokia) | JCTVC-L0035 | TE2: Results of test 3.1.8 on PU based Intra BL signaling | LG (JCTVC-L0281) |
| 3.1.9 (Samsung) | JCTVC-L0110 | TE2 Subtest 1: Inter-layer Texture Prediction Signalling (test 3.1.9) | Nokia (JCTVC-L0130) MediaTek (JCTVC-L0341) |

Subset 2: RefIdx approach

| Test | Proposal No. | Proposal Title | Cross-checker |
| --- | --- | --- | --- |
| 3.2.1 (InterDigital) | JCTVC-L0051 | TE2: Results of test 3.2.1 on inter-layer reference picture placement | Qualcomm (JCTVC-L0261) ETRI (JCTVC-L0062) Samsung (JCTVC-L0090) |
| 3.2.2 (ETRI) | JCTVC-L0060 | TE2 : Results of test 3.2.2 on inter-layer reference picture placement | Intel (JCTVC-L0306) and InterDigital (JCTVC-L0201) |
| 3.2.3 (Sharp) | JCTVC-L0251 | TE2-3.2.3 Infer Zero motion vector when referencing co-located base layer picture | TI (JCTVC-L0353) Canon (JCTVC-L0382) |

Subset 3: Enhancement layer slice skip

| Test | Proposal No. | Proposal Title | Cross-checker |
| --- | --- | --- | --- |
| 3.3.1 (InterDigital) | JCTVC-L0055 | TE2: Results of test 3.3.1 Enhancement layer skipped slice | Huawei (JCTVC-L0296) |

# Discussion

## Intra-BL approach

Technical summary of TE proposals

| Proponent | Signaling level | IntraBL Skip | IntraBL 4x4 Trans. | Deblocking BS | Cbf root coding |
| --- | --- | --- | --- | --- | --- |
| QualcommA | CU | Yes | DCT | 1 (IntraBL), 0 (IntraBLSkip) | No |
| QualcommB | CU | Yes | DST | 1 (IntraBL), 0 (IntraBLSkip) | No |
| Huawei | CU | No | DCT | 2 | Yes |
| Huawei | CU | Yes | DCT | 2 | Yes |
| Intel | CU | Yes | Fwd DCT/Bwd DST | 2 | No |
| MTK&LG | CU | No | DST | 1 | Yes |
| Nokia | CU for Intra slice; Second PU will not be signaled if first one is IntraBL | Yes(LCU in Intra Slice) | DST(I Slice)/DCT (P/B Slice) | 2 ( ISlice), 1/0 (P,B slice) | No(I Slice)/Yes (P,B Slice) For Merge, no no_residue_flag for non_skip 2Nx2N; |
| SamsungA | CU | Yes | DCT | 2 | Yes |
| SamsungB | CU | No | DCT | 2 | Yes |

Common features identified in TE contributions:

- DCT vs. DST – transform used for 4x4 unit when prediction is from reconstructed BL image

Two non-TE2 contributions suggest DST (L0204 and L0067).

Comment that Intra-BL prediction is like Inter so DCT is suggested.

Comment that Intra-BL is neither Intra nor Inter, it is an independent mode.

Comment that DST will not introduce new transform for All Intra coding.

Pro of DST:

- Continue to use existing HEVC all Intra pipeline for 4x4

-  0.1% gain in AI-1.5X, no loss.

Pro of DCT:

-the same transform type for all block sizes for Intra-BL

Recommend DST for Intra-BL 4x4 transform – Agreed.

- CU based vs. PU based signalling

Suggestion to verify technical details – code checking of test 3.1.8 showed that the software reflects TE description, but something is missing in the TE proposal.

Comment that most contributions are combos of tools, and a direct comparison is not easy.

Comment that the gain from PU based signaling contribution is not a purely comparison of PU and CU signaling.

Suggestion to start from something simple –CU level, which may be beneficial for next meeting.

Multiple experts supported the above suggestion.

Suggestion to investigate PU based signaling during next meeting cycle.

Recommend CU level signaling – Agreed (keep the current SMuC Intra-BL scheme signaling)

- Intra_BL skip

Up to 0.1% gain

Recommend not to use Intra-BL Skip - Agreed

- Deblocking BS for IntraBL

Only MTK&LG (L0069) showed results with and without Deblocking BS change.

According to Samsung’s report, BS change in L0069 has luma gain 0.1% for AI 2X, 0.2% AI 1.5X, 0.0% for all other cases, also some gain for chroma (0.1% - 0.2%).

The BS setting in L0069 is 1 for luma and 2 for Chroma–Chroma Deblocking is turned on.

Comment:  lack of visual check to verify visually meaningful gain.

Cross-check stated that no visual difference from BS change in QP 37.

Comment that more study needed, in particular for special video sequences.

Comment that BS should be the same as Intra (BS = 2).

Comment that in previous SVC standard, IntraBL is treated as Inter for Deblocking (BS setting)

Comment that stronger Deblocking is safer because large blocks were introduced in HEVC.

Recommend:  to keep BS as 2, and investigate change of BS during next meeting cycle - Agreed

- cbf_root

Comment that not using cbf_root to keep a simple starting point.

A non-proponent supported cbf_root as it is simple.

Decoding time reduction is observed consistently among all proponents with cbf_root (15% for AI)

Comment - 4 companies used this cbf_root in their TE2 proposals.

Suggestion to investigate cbf_root during next meeting cycle.

A non-proponent reported that 0.1% difference (gain) by using cbf_root.

Comment - a 0.5% gain (LDP) reported for applying cbf_root_flag for Intra_BL in Intra Slice and Skip flag (for IntraBL) in Inter Slice.

Comment there is some loss in chroma when using cbf_root_flag for Intra_BL.

Comment that the loss in chroma is a natural trend when introducing new syntax. And the decoding time reduction is not trivial.

Comment that cbf_root_flag is not a new syntax element; just reuse this existing syntax with a small condition change.

Multiple experts supported using cbf_root_flag.

Recommendation- use cbf_root_flag (after cross-checkers verify SMuC0.1.1 based software (with DST transform for IntraBL4x4)) from MediaTek-Agreed (Jan.15)

Jan.17, Samsung reported that the cross-checking results of the above software: Average results over 6 cases (2X+1.5X) x (AI+RA+LDP) are: 0.2% luma gain, 0.4% chroma gain, 12% decoding time reduction.

After seeing the cross-checking results from Samsung, it was agreed to recommend using cbf_root_flag for Intra-BL coding- Agreed

Action: The software SMuC 0.1.1 with the above recommended settings: DST for IntraBL-4x4 +cbf_root_flag (provided by MediaTek) will be included in the uploaded BoG report.

- Software received in the afternoon of Jul17 and included in the BoG report uploading package.

## Jan. 15: There was an interest in TI’s (cbf_root + Skip flag) in L0230 (Non-TE, Track A).  TI will show some results (will be discussed at the end of this BoG).

Jan. 17: At the end of this BoG, results of TI’s (cbf_root+Skipflag) were shown by cross-checker Samsung: luma gain 0.1%-0.2% for AI, 0.5% for LDP 1.5X, in other cases are 0.0%-0.3%. -16% decoding time reduction for AI, 10% decoding time reduction for other cases.

Anchor: SMuC 0.1.1 with bugfix.

## RefIdx approach

Technical summary of TE proposals

| Test |  |
| --- | --- |
| 3.2.1 (InterDigital) | 5 different tests: - A: also called setting1 – RefIdx approach in SMuC 0.1.1 - B: also called setting2 – for temporal layer 0, ILRP inserted at last positions in both L0 and L1; for other temporal layers, ILRP is only inserted in L0 - C: B + forced uni- prediction for ILRP (encoder only constraint) - D: B + forced zero-MV prediction for ILRP (encoder only constraint) - E: B + forced zero-MV uni-prediction for ILRP (encoder only constraint) |
| 3.2.2 (ETRI) | - RefIdx assignment for ILRP: 1 for non-random access pictures and 0 for random access pictures - Reference List assignment for ILRP: L0 and L1 for AI and RA, only L0 for LD-P |
| 3.2.3 (Sharp) | - Inferred zero-MV for ILRP (normative) |

Common features identified in TE contributions:

- Adaptive placement of ILRP

Proponents stated that it is best to keep the current ILRP placement, but suggested the bugfix proposed in L0167 be integrated in the reference software. No objections from other experts.

Per experts’ request, proponents of L0167 presented this bug-fix proposal.

Proponents of L0167 stated that this bug does not affect HM in CTC but affects SMuC+ RefIdx as the same picture is inserted into both lists. Suggest reporting this bug in HM bug-tracker.

Recommend to keep the current ILRP placement; integrate the bugfix proposed in L0167 into reference software (proponents will do the integration) – Agreed.

- Use zero-MV for ILRP; the zero-MV may be signaled or inferred.

Comment that RefIdx can enable low-level change (e.g., inferred zero-MV in 3.2.3), which is not HLS.

Proponent of 3.2.3 supported integrating encoder-only change zero MV (guarantee bit-stream conformance) first.

Comment that normative zero-mv (3.2.3) has a better performance and thus suggested integration of this normative zero-MV.

Request for the proponents to present results of RefIdx using the same number of reference frames (since ILRP are extra frames added to the reference lists)

Multiple experts requested the results of 3 reference frames for RefIdx.

Request to see results of 3 reference frames within the CTC.

Recommendation: integrate encoder-only zero MV into reference software –Agreed.

Action: The software SMuC 0.1.1 with the above recommended settings: encoder-only zero MV (will be provided by InterDigital ) and L0167 bug-fix will be included in the uploaded BoG report.

- Software received in the afternoon on Jul17 and included in the BoG report uploading package.

## Skipped enhancement slice

Technical summary of TE proposals

| Test |  |
| --- | --- |
| 3.3.1 (InterDigital) | Skip entire enhancement layer slice, reconstructed slice is up-sampled base layer image - Test A: with SMuC Intra-BL flag scheme - Test B: with SMuC RefIdx scheme |

Conclusion: already discussed in Track A.