| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0438 |
| --- | --- |

| Title: | BoG report on subjective viewing test comparing normative and non-normative deblocking filter proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Kenneth Andersson | Email: | andrey.norkin@ericsson.com kenneth.r.andersson@ericsson.com |
| Source: | BoG on subjective viewing |  |  |

_____________________________

# Abstract

This contribution is a report on informal subjective viewing for the deblocking filtering that was held during the Geneva meeting on January 17, 2013. The goal of this informal subjective viewing test was to determine if the normative modification from L0232 on top of non-normative modifications from L0232 helps to additionally improve the subjective quality.

# Test procedure

Based on the results of the subjective test for deblocking filter proposals [2], it has been requested to additionally test the best performing approach (that includes normative changes versus) the best performing non-normative approach, by best performing it means in terms of how many times it performed better than the HM9.0 anchor.

The proposals were based on HM9.1. The proposals in each test session were evaluated by a group of 2 experts. In total, twelve viewing sessions have been held. The identities of the proposals and the anchor were hidden (the test subjects were shown labels A or B instead).

The following six Type 1 sequences have been used in the test. Type 1 sequences are used in order to show that there are improvements of the subjective quality.

- Riverbed, Qp=32, RA

- Riverbed, Qp=37, RA

- WestWindEasy, Qp=37, LDB

- DucksTakeOff, Qp=37, LDB

- ChinaSpeed, Qp=37, LDB

- RedKayak, Qp=37, RA, first 10 seconds.

Five Type 2 sequences have been used in the test. Type 2 sequences were used in order to show that there are no degradations of the subjective quality on the common test condition sequences.

- Kimono, Qp=32, LD

- Cactus, Qp=32, LD

- BasketballDrillText, Qp=32, LD

- BQMall, Qp=32, LD

- Johnny, Qp=32, LD

ABAB test was used. The sequence and the anchor were shown one after another. The order of the test and the anchor was randomized for every sequence and every viewing session and identities of the proposals are hidden. Test subjects were asked to rate each proposal on a scale from -2 to 2. The results were later calculated as well as 95-percent confidence intervals for every proposal and test sequence.

The length of one session was about 10 minutes. In total, 24 test subjects participated in scoring. The test sessions, processing of score sheets and calculations of the results were conducted by Andrey Norkin and Kenneth Andersson (Andrey Norkin). Processing of score sheets and calculation of results was cross-verified by Tomoo Yamakage (Toshiba).

# Test equipment

Equipment provided by meeting organizers was used. Dell 3008 LDC monitor was used in the subjective tests. The monitor is 30 inches in diagonal and has resolution 2560x1600. The actual size of the sequences on the screen was smaller since all the sequences were shown in screens native resolution. The software, provided by David Flynn was used in the testing.

# Proposals

JCTVC-L0232 (Ericsson)

1. Signaling tc_offsets depending on the picture position in the GOP (config file modification)

The proposed deblocking parameters are listed below. First for low delay configurations:

#     Type POC QPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2

Frame1:  B    1   3        0.4624   5            0

Frame2:  B    2   2        0.4624   3            0

Frame3:  B    3   3        0.4624   5            0

Frame4:  B    4   1        0.578    1            0

and for random access:

#      Type POC QPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2

Frame1:  B    8   1        0.442    1              0

Frame2:  B    4   2        0.3536   3              0

Frame3:  B    2   3        0.3536   4              0

Frame4:  B    1   4        0.68     6              0

Frame5:  B    3   4        0.68     6              0

Frame6:  B    6   3        0.3536   4              0

Frame7:  B    5   4        0.68     6              0

Frame8:  B    7   4        0.68     6              0

2. Modification of strong filtering decisions and the strong filter

2*dpqi < (β>>2)			          (4)    Same as in HM9.0

(|p3i - p0i| + |q0i – q3i|) < (β>>3)         (5)

dp0 = ((| p3,i − 2*p2,i + p1,i | + | q3,i − 2*q2,i + q1,i |) <<1) < (β>>2)  	(5)

// remove a condition that 4 samples from the block boundary have the same level

// instead, add a condition that 4 pixels //from the block boundary form a straight line (flat or inclined),

|p0i – q0i| < ((5*tC + 1)>>1)  		      (6) // same as in HM9.0

The proposed decisions are explained in Figure 1. The proposed decisions enable applying strong filtering also when both sides of the block boundary have a form of ramp (similar to the normal filtering mode).

| (a) | (b) |
| --- | --- |

Figure 1 (a) Proposed decisions for strong filtering. Two checks are included: that pixel values q0, q1, q2 and q3 form a line (the same for p0, p1, p2 and p3) and that | p0 – q0 | is below threshold. (b) Block artifact from Riverbed. Filter decision matches the artifact.

The effect of applying HEVC strong filter and the proposed filter to a ramp is shown in Figure 2. One can see from the figure that HEVC filter significantly disports the ramp when applied to it, while the proposed filter modified the ramp much less.

| - Effect of HEVC strong filter applied to a ramp | - Effect of proposed strong filter applied to a ramp |
| --- | --- |

Figure 2 Comparison of modification of ramp by HEVC filer and the proposed filter. The original signal is shown in blue, the signal after modification by a filter is shown in red.  The HEVC filter (a) modifies the ramp stronger than the proposed filter (b).

3. Rate distortion penalty on the 32x32 intra predicted TU in inter-slices

A cost penalty is introduced to 32x32 intra TU in inter slices.

# Tested configurations

The following combinations of tools have been tested in the subjective viewing test:

Anchor:

HM9.1 + tc_offsets + RD_penalty on Intra 32x32 TU in inter slices (L0232_tc+RDpen)

Test:

HM9.1 + tc_offsets + RD_penalty on Intra 32x32 TU in inter slices + strong filter (L0232_tc+RDpen+filt)

# Notes on the subjective viewing

During the second session it was noticed that the monitor was placed at a larger distance from the viewers than during the first subjective deblocking test [2]. After that, the monitor was places on the same position as during the first test and the test continued.

# Bitrates of the tested sequences

Type 1 sequences

| Proposal | Riverbed, Qp=32, RA | Riverbed, Qp=37, RA | WestWindEasy,Qp37, LDB | DucksTakeOff, Qp=37, LDB | ChinaSpeed, Qp=37, LDB | RedKayak, Qp=37, RA |
| --- | --- | --- | --- | --- | --- | --- |
| L0232_tc+RDpen | 4940.95 | 2440.12 | 1690.53 | 5111.22 | 618.51 | 1733.91 |
| L0232_tc+RDpen+filt | 4940.92 | 2437.36 | 1685.22 | 5108.21 | 618.06 | 1732.84 |

Type 2 sequences

| Proposal | Kimono, Qp=32, LDB | Cactus, Qp=32, LDB | BQMall,Qp32, LDB | Johnny, Qp=32, LDB | BasketballDrillText, Qp=32, LDB |
| --- | --- | --- | --- | --- | --- |
| L0232_tc+RDpen | 1150,35 | 2575,49 | 902,22 | 185,25 | 880,64 |
| L0232_tc+RDpen+filt | 1148,23 | 2576,90 | 903,34 | 186,52 | 881,89 |

# Test results

The graphs below show the test results. Values with overlapping CI values cannot be rated as distinguishable.

The results for Type 1 sequences are shown in the following plot.

The results for Type 2 sequences are shown in the following plot.

# References

- T. Yamakage, A. Norkin, "JCT-VC AHG report: HEVC in-loop filtering (AHG6)", JCTVC-L0006, Geneva, Jan. 2012.

- A. Norkin, K. Andersson, "BoG report on subjective viewing test for deblocking filter proposals", JCTVC-L0430, Geneva, Jan. 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-K1100, Shanghai, Oct. 2012.