| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0442 |
| --- | --- |

| Title: | Crosscheck of JCTVC-L0187 on HEVC Software modifications for field based coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Félix Henry Gordon Clare Orange Labs 4 rue Clos Courtel 35510 Cesson Sevigne, France | Tel: Email: | +33 2 99 12 45 58 felix.henry@orange.com gordon.clare@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This contribution reports the crosschecking of informative contribution JCTVC-L0187 that proposes modification of the reference software in order to handle interlace material. The modifications include field separation at input, field interlacing at output, and a new GOP structure. The crosschecker ran the software on non-CTC interlace videos and obtained a perfect match with the performance reported by contribution JCTVC-L0187.

# Description

JCTVC-L0187 [1] proposes a number of non-normative changes for implementation in the reference software. These changes aim at improving the compression of interlace video content, and include the following aspects:

- The input video sequence is interlace, the fields are separated before compression

- The fields are re-interlaced after reconstruction

- A new GOP structure is provided, compared to the GOP structure proposed in JCTVC-K0216 [2]

- Changes had to be made to the reference software to accommodate this GOP structure

- These changes are activated by --FieldCoding command line argument (both at encoder and decoder since field SEI massage are not yet implemented in the reference software)

- A additional commandline argument --TopFieldFirst can be used to accommodate top or bottom field first interlace material.

# Software simulation

The software provided by the proponents has been run on the non-CTC sequence provided and the results perfectly match those described in [1]. Table 1 shows the performance of the previous field GOP structure proposed in JCTVC-K0216 [2], and Table 2 shows the performance of the new field GOP structure proposed in JCTVC-L0187 [1].

Table 1: Performance with the previous GOP structure

Table 2: Performance with the proposed GOP structure

# Bibliography

[1] JCTVC-L0187, HEVC Software modifications for field based coding, Zineb AGYO, Jean-Marc THIESSE, Jérôme VIÉRON

[2] JCTVC-K0216, HEVC interlaced coding assessment and Chroma issue consideration, J.-M. Thiesse, J. Viéron, P. Larbier, A. Depreux